| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power saving enhancement in 16m |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Bin Chen Junxian Mo Ronald Mao Huawei Technologies, Ltd | E-mail: binchen@huawei.com |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document shows suggested changes to P802.16m Draft D7 related to power saving enhancement. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Power Saving Enhancement for 16m

Bin Chen, Junxian Mo, Ronald Mao

Huawei Technologies Co., Ltd

# Overview

Power saving is a very important features in future wireless system, where there will be huge amounts of different types of terminal device. Now 16m system use sleep mechanism to achieve power saving in the terminal, but the disadvantage is that it need pre-configuration and follow a strict set of sleep cycle and listening cycle, which makes it not so flexible for ABS to schedule the AMS and traffics.

In this contribution, we would like to propose a scheme to supplement sleep mechanism for power saving. The BS scheduler can have a in-advanced schedule for a AMS, if it can anticipate that there will be no resource allocation for a MS for some next frames, it can tell the AMS to stop listening the channels, so that save the MS’s power.

# Proposed changes

## Change 1

On page 570, line 38:

-------------------------------  Start of text proposal #1 --------------------------------------------

Table 848—DL Basic Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| …. |  |  |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 2 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for multi-layer encoding |
| ….. |  |  |
| }else{ |  |  |
| Reserved | 5 | Reserved bits |
| } |  |  |
| ….. |  |  |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Next-Alloc-Indicator | 1 | Indicate whether there is resource allocation in the next N frames 0b0: no resource allocation in the next N frames; 0b1: there may be resource allocation in the next N frames |
| } |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

## Change 2

Text Proposal #2 < add below text in blue to the section 16.3.6.5.2.4.1 in the page 570 line 52>

-------------------------------  Start of text proposal #2 -------------------------------------------

Resource Index: Resource Index with starting LRU index and size of a single allocation spanning contiguous LRUs.

Next-Alloc-Indicator: Indicate whether there is resource allocation in the next N frames, where the N is broadcasted in SFH. When the AMS recognizes that there is no resource allocation in the next N frames, it may stop listening the A-MAP and traffic channels, so that to save its power.

Resource Indexing in the DL/UL Basic Assignment A-MAP IE

-------------------------------  End of text proposal #2 --------------------------------------------