| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling of PHY feedback headers and messages in multicarrier (16.2.8) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Xiangying Yang Intel Corporation | xiangying.yang@intel.com |
| Re: | Sponsor ballot comment on P802.16m/D7 Target topic: “IEEE P802.16m/D7, section 16.2.8”. |  |
| Abstract | The contribution proposes a clarification of PHY level controls in multicarrier |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handling of PHY feedback headers and messages in multicarrier (16.2.8)

Xiangying Yang

Intel Corporation

- Introduction

The contribution proposes the text changes of clarification of PHY level controls in multicarrier to be included in the 802.16m amendment. In the current draft [1], PHY level controls (MIMO/PC/FFR/CINR report) should be clarified for multi-carrier operation. UL channel operation and DL CINR report operation for CA (Carrier Aggregation) with partially configured secondary carrier are already defined. However, PHY level controls using MAC control message or MAC signaling header for CA with fully configured carriers are not clear.

We propose to handle some PHY level controls per carrier unlike MAC state, mobility and context of an AMS that are managed and controlled by an ABS through the primary carrier. In addition, bandwidth request related to PHY level controls of each carrier is transmitted using the corresponding carrier.

Details are as following:

- Specify the usage of MAC control signal for PHY level control

- Using carrier index to allow the following feedbacks, while controls each carrier separately, to be sent on any grant regardless which carrier the grant comes from. And if necessary, they can also be sent ONLY on the associated carrier, which would need support on bandwidth request behavior.

  - Uplink Power Status Report header, Correlation Matrix Feedback and MIMO Feedback header

  - AAI_FFR-CMD, AAI_FFR-REP, AAI_SingleBS_MIMO_FBK, AAI_MultiBS_MIMO_FBK, AAI_MULTI_BS_MIMO-REQ, AAI_MULTI_BS_MIMO-RSP, AAI_UL_POWER_ADJ, AAI_UL_PSR_CFG, AAI_ULPC_NI*, AAI_DL-IM *, AAI_SCD*

* Indicates broadcast message

Reference

[1] IEEE P802.16m/D7, July. 2010

- Text change

------------------------------------------ Text Start ---------------------------------------------------

Remedy #1

[Modify the section 16.2.8.2.8 title of page 311 as following:]

16.2.8.2.8 DL CINR report operation PHY level control and UL Feedback Channel allocation

Remedy #2

[Add the following text in line 57 of page 311:]

In multicarrier aggregation, PHY level control (MIMO/power control/FFR/CINR report) should be handled per carrier. MAC signaling header or MAC control message for PHY level control of each carrier can be transmitted on any available grant on any carrier. A 6-bit carrier index is included to indicate the relevant carrier that this control message or signaling header is associated with. The following MAC signaling header and MAC control message are handled in this way:

- MAC signaling header: Uplink Power Status Report, Correlation Matrix Feedback and MIMO Feedback

- MAC control message: AAI_FFR-CMD, AAI_FFR-REP, AAI_UL_POWER_ADJ, AAI_UL_PSR_CFG, AAI_SingleBS_MIMO_FBK, AAI_MultiBS_MIMO_FBK, AAI_MULTI_BS_MIMO-REQ, AAI_MULTI_BS_MIMO-RSP

In multicarrier operation mode, the pathloss reference (i.e. preamble) for uplink power control shall be the same carrier in TDD mode and the paired downlink carrier in FDD mode, respectively.

------------------------------------------------ Text End ---------------------------------------------------