| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text changes for Group resource allocation in 16m |  |
| Date Submitted | 2010-8-17 |  |
| Source(s) | Bin Chen, George Cummings Huawei Hyunkyu Yu Samsung | E-mail: binchen@huawei.com gcummings@huawei.com |
| Re: | Call for SB rec#1 on “ P802.16m/D7”: |  |
| Abstract | This contribution analyses the tech problem of group resource allocation in D7, and then provide some solutions to this problem, and finally propose the comments to revise the text in the D7 specification. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot rec#1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text changes for  Group resource allocation in 16m

Bin Chen, George Cummings

Huawei Technologies

- Introduction

In the last sponsor ballot in July, the contribution about C80216m-10_0772r2.doc was adopted which was to fix a problem of group resource allocation issue in D6 that exposed the all the resource allocation information to all the MSs in a group, but the contribution did not change all the texts that need to be changed. This contribution is to supplement that one to make whole standard text alignment.

- References

[1] IEEE P802.16m/D7

- Proposed text change in the P802.16m/D6

------------------------------------------ Begin Proposed Text Change 1-------------------------------------------

[Editor’s Note 1 : change the text in page 323, starting on line 42]

The ABS configures a HARQ Burst Size Set for each flow within group. Each The HARQ burst size set supports four HARQ burst sizes. The Group Configuration MAC control message signaled to add a flow of an AMS to a group contains the HARQ burst sizes assigned to the flows group. The assigned HARQ burst sizes to a flow in the group shall be chosen from the configured set.

------------------------------------------ End Proposed Text Change 1-------------------------------------------