| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change for frame structure: co-existence with TD-SCDMA in 1/16 CP ratio (16.3.3.7) |  |
| Date Submitted | 2010-8-12 |  |
| Source(s) | Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, Yu-Chuan Fang ITRI | zhengyanxiu@itri.org.tw +886-3-5914715 |
| Re: | Sponsor ballot comment on P802.16m/D7 |  |
| Abstract | The contribution proposes frame structures which co-existing with TD-SCDMA of symmetric and asymmetric DL/UL configuration in 1/16 CP ratio |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text change for frame structure: co-existence with TD-SCDMA in 1/16 CP ratio (16.3.3.7)

Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, Yu-Chuan Fang

ITRI

- Introduction

In P802.16m/D7 [1], the frame structure can support that IEEE 802.16m co-exist with TD-SCDMA in 1/8 CP ratio by puncturing 2 OFDM symbols. Detail analysis can be seen in C802.16m/2124r1 [2]. However, it can not support 1/16 CP ratio case and loses a capacity improvement by 6%. This contribution provides frame configuration for 1/16 case. In uplink, type-2 AAI subframe is used to keep uplink channel performance without the impact of sounding.

- TD-SCDMA frame structure and the associated constraints

TD-SCDMA has two TDD configurations: symmetric and asymmetric DL/UL configurations. Fig. 1 illustrates the symmetric DL/UL configuration. The subframe spans 5ms and is composed of DwPTS (75μs), GP (75μs), UpPTS (125μs), 7 slots (675μs x7=4725μs). Downlink is composed of DwPTS, and downlink slots. Uplink is composed of UpPTS and uplink slots.

Fig. 1: TD-SCDMA frame structure of symmetric DL/UL configuration.

Based on this structure, we have several constraints.

- The TTG in IEEE 802.16 frame structure shall not be smaller than 75μs and could be overlapped with GP in TD-SCDMA

- The downlink in IEEE 802.16m frame structure shall be in the duration of downlink of TD-SCDMA

- The uplink in IEEE 802.16m frame structure shall be in the duration of uplink of TD-SCDMA

Fig. 2 illustrates an example of timing relationship between IEEE 802.16 frame structure and TD-SCDMA frame structure. In IEEE 802.16 frame structure, TTG covers GP, downlink in TD-SCDMA covers that in IEEE 802.16 and uplink in TD-SCDMA covers that in IEEE 802.16.

Fig. 2: An example of timing relationship between IEEE 802.16 and TD-SCDMA.

- Frame structure

Table 1 shows the frame structure for 1/16 CP ratio and this frame structure supports TD-SCDMA with 5:2 and 4:3 configurations. In the 5:2 configuration, downlink spans 3450 us, uplink spans 1475 us and guard period spans 75 us. In the 4:3 configuration, downlink spans 2775us and uplink spans 2150us and guard period spans 75 us.

In the 6:2 configuration shown in Table 1, downlink spans 3400us < 3450us and uplink spans 1360us < 1475 us. The TTG ranges from 3400us to 3579.996us and the GP ranges from 3450 to 3525 us if downlink is aligned. Therefore, this configuration can co-exist with TD-SCDMA asymmetric configuration.

In the 5:3 configuration shown in Table 1, downlink spans 2720 us< 2775 us and uplink spans 2040 us < 2150 us. The TTG ranges from 2720 us to 2899.996 us and the GP ranges from 2775 us to 2820 us if downlink is aligned. Therefore, this configuration can co-exist with TD-SCDMA symmetric configuration.

Table 1: Frame structure for 6:2 and 5:3 D:U ratio for 1/16 CP ratio

| D:U | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 6:2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type2 | UL Type2 | 179.996/60 |
| 5:3 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type2 | UL Type2 | UL Type2 | 179.996/60 |

- Conclusion

This contribution provides frame structure to support TDD co-existence with TD-SCDMA with 1/16 CP ratio. Extra 6% spectrum efficiency can be achieved comparing to 1/8 CP ratio.

- References

[]	IEEE P802.16m/D7, DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks – Part 16: Air interference for fixed and mobile broadband wireless access systems, August 2010.

[]	IEEE C802.16m-09/2124r1, Multi-RAT Coexistence Analysis and Frame Configuration (15.3.3), IEEE 802.16m Session 63, Aug. 2010.

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

[Remedy: Change texts in Section 16.3.3.7 in Page 465 Table 804 as follows:]

<\ins>

Table 804—Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 | 5/10/20 | 1/16 | 5 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type2 | UL Type2 | 179.996/60 |
| 42 | 5/10/20 | 1/16 | 6 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type2 | UL Type2 | UL Type2 | 179.996/60 |

<\ins>

-------------------------------  Text End  ---------------------------------------------------