| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto ABS reselection by AMS (16.4.7.5) and Related message (Table 680) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Ying Li, Youngkyo Baek, Anshuman Nigam, Jung Je Son, Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D7change requests. |  |
| Abstract | This contribution is to propose changes in support femtocells. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femto ABS reselection by AMS (16.4.7.5) and Related message (Table 680)

Ying Li, Youngkyo Baek, Anshuman Nigam, Jung Je Son,

Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi

Samsung Electronics

# Introduction

- In the current draft D7, it has several problems related to the femtocell reselection.

16.4.7.5 FemtoABS reselection by AMS

- It has several problems in it.

- Problem 1: “If the received CSGID(s) from the AMS does not match any of the CSGID(s) of the Femto ABS itself, then the Femto ABS shall determine that the AMS is not a member of the Femto ABS and therefore cannot be granted access.”

- The above logic will hold only if: the received CSGID(s) are ALL the CSGIDs and the BSIDs that the AMS has subscribed.

- If the AMS only sends part of the CSGIDs or BSIDs that the AMS subscribes, the above logic does NOT hold.

- Problem 2: “In case the AMS does not support CSG whitelist capability or does not have any CSGID(s) provisioned in its CSG whitelist, the "Redirection Info" may be provided in the AAI_REG-RSP message.”

- If the AMS does not have any CSGID(s) provisioned in its CSG whitelist, why at the first place the AMS would perform ranging to CSG Femto BS? Since this scenario implies that the AMS does not subscribe any CSG, hence the AMS should not try to range to the CSG Femto BS, so there is no need for “Redirection Info”.

- Remedy:

- In section 16.4.7.5, clarify the use cases for “redirection information”.

- In the AAI_RNG-REQ, we present following indication:

ALL CSGIDs included indicator,

“0” AMS has not included ALL CSGIDs in the AAI_RNG-REQ

“1” AMS has included ALL CSGIDs in the AAI_RNG-REQ

# Proposed Text

Please revise D7 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.7.5 FemtoABS reselection by AMS

[Modify texts as suggested below: Page 810, Line 1-41]

When the AMS is trying to perform initial network entry or network re-entry with an Femto ABS, it first performs initial ranging by sending AAI_RNG-REQ message. AMS may include one or more CSGID(s) as part of the AAI_RNG-REQ message to the Femto ABS, if one or more CSGID(s) is(are) provisioned in the AMS. If the Femto ABS is a CSG Femto ABS, it may have one or more CSGID(s) provisioned in it as well. If it is an OSG Femto ABS, then there shall be no CSGID provisioned for it.

If the Femto ABS is an OSG Femto ABS, then the Femto ABS ignores the CSGID(s) (if sent by the AMS) in the AAI_RNG-REQ and goes ahead with the next steps. If the Femto ABS is a CSG Femto ABS, the Femto ABS receives the AAI_RNG-REQ and if needed it looks at the received CSGID(s) and checks if it matches with at least one of its CSGID(s). If there is match of the CSGID, then the Femto ABS knows that the AMS is a member of the Femto ABS and goes ahead with the next steps.

If the AMS sends all the CSGID(s) or BSIDs it subscribes in AAI_RNG-REQ, iIf the received CSGID(s) from the AMS does not match any of the CSGID(s) or BSID of the Femto ABS itself, then the Femto ABS shall determine that the AMS is not a member of the Femto ABS and therefore cannot be granted access. In this case, the Femto ABS shall send a AAI_RNG-RSP and in the AAI_RNG-RSP it indicates the rejection of access for this AMS. In order to help the AMS to attach to nearby Femto ABSs, the Femto ABS provides "Redirection Info" to the AMS in the AAI_RNG-RSP message. The "Redirection Info" consists of the ABSID, preamble index and center frequency of other nearby cells.

If there are no CSGIDs included in the AAI_RNG-REQ then the normal network entry procedure as in section 16.2.15 applies.

If the Femto ABS has CSGID info of nearby Femto ABSs, then it may filter the "Redirection Info" based on the CSGID(s) provided by the AMS in the AAI_RNG-REQ message and only provide the OSG Femto ABSs as well as CSG Femto ABSs with matching CSGID(s) to the AMS in the "Redirection Info".

The AMS then can use this "Redirection Info" and try to attach to the other candidate ABSs.

In case the AMS does not support CSG whitelist capability or does not have any CSGID(s) provisioned in its CSG whitelist, the "Redirection Info" may be provided in the AAI_REG-RSP message.

If the AMS has whitelist, but no CSGID or BS ID in the whitelist, the AMS should not try to perform ranging to CSG-closed Femto ABS. AMS can range a CSG-Open Femto ABS but if the Femto-ABS does not wants to accept it, it may send re-direction info to it.

In ranging procedure, if the ABS cannot accept the network entry while the security key is shared already, AAI_RNG-RSP may be used to provide “Redirection Info”.

If AMS cannot pass the registration, or the ABS cannot accept the network entry due to reasons such as the ABS is already heavily loaded to accept any AMS or AMS with low priority, the AAI_REG-RSP may include the redirection information.

16.2.3.1 AAI_RNG-REQ

[Modify Table 680]

Table 680—AAI_RNG-REQ message Field Descriptions

| …… |  |  |  |  |
| --- | --- | --- | --- | --- |
| O | AMS Mobility Information | 2 | 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved | It may be included when the AMS is attempting to perform the location update |
| O | csgIdArray [1..15] | variable | List of CSG ID | Present if sent by AMS for quick CSG membership detection or ABS reselection assistance. |
|  | A) CSGID | 8 | CSG identifier. |  |
| O | All subscribed CSGIDs/BSIDs included indicator, | 1 | “0” AMS has not included All its subscribed CSGIDs or BSIDs in the AAI_RNG-REQ “1” AMS has included All its subscribed CSGIDs or BSIDs in the AAI_RNG-REQ | It may be included if CSGID presents. |
| O | fidArray [1..15] |  |  |  |
|  | A) FID | 4 | Flow identifier |  |
| ……. |  |  |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------