| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On How to include Femtocell CSGIDs in MAC messages AAI_RNG/SCN-REQ (Table 680, 693) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Ying Li, Anshuman Nigam, Jung Je Son, Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi Samsung Electronics Muthaiah Venkatachalam Intel | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D7change requests. |  |
| Abstract | This contribution is to propose changes in support femtocells. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On How to include Femtocell CSGIDs in MAC messages AAI_RNG/SCN-REQ

(Table 680, 693)

Ying Li, Anshuman Nigam, Jung Je Son,

Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi

Samsung Electronics

Muthaiah Venkatachalam

Intel

# Introduction

- In the current draft D7, several MAC messages include fields for CSGIDs. However, the format of how to include the CSGIDs is not clear.

# Proposed Text

Please revise D7 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2.3.1 AAI_RNG-REQ

[Modify Table 680]

Table 680—AAI_RNG-REQ message Field Descriptions

| …… |  |  |  |  |
| --- | --- | --- | --- | --- |
| O | AMS Mobility Information | 2 | 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved | It may be included when the AMS is attempting to perform the location update |
| O | csgIdArray [1..15] | variable | List of CSG ID | Present if sent by AMS for quick CSG membership detection or ABS reselection assistance. |
|  | A) CSGID Operator ID of the CSG Femtocell | 8 24 | CSG identifier. The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS Present only once for the CSGIDs which belong to it. |
|  | B) CSGID within the Operator ID | Variable | The CSGID within the Operator ID. | It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |
| O | fidArray [1..15] |  |  |  |
|  | A) FID | 4 | Flow identifier |  |
| ……. |  |  |  |  |

16.2.3.13 AAI_SCN-REQ

Table 693—AAI_SCN-REQ parameters

| O | carIndexArr [ 0..63] | Recommended Carrier index at serving ABS | Variable ( 6 x N) | Recommended physical carrier index of the serving ABS the AMS plans to scan | Present if N_Recommended_Carrier_Index_at_Serving_ABS > 0 |
| --- | --- | --- | --- | --- | --- |
| O | csgIdArr[0..15] | CSG Identifier | Variable (8 x N) | Sent by AMS to aid ABS reselection | Present if CSG identifier is available |
|  |  | A) Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS. Present only once for the CSGIDs which belong to it. |
|  |  | B) CSGID within the Operator ID | Variable | The CSGID within the Operator ID. | It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |

16.2.3.8 AAI_REG-RSP

Table 687 – AAI_REG-RSP message Field Descriptions

| …… |  |  |  |  |
| --- | --- | --- | --- | --- |
| O | Redirection Info[1..8] | Variable | ABSID, preamble index and center frequency for one or more neighbor ABSs of the serving ABS | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry (due to various reasons such as high serving ABS load, non existence of CSG membership etc) |
| O | CSGID Length | 5 | Length of CSGID | Since the CSGID can be of variable length (MAX upto 24 bits). |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------