| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-SON-ADV (16.2.3.7) |  |
| Date Submitted | 2010-9-14 |  |
| Source(s) | Inuk Jung LG Electronics | inuk.jung@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-SON-ADV section 16.2.3.7 |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-SON-ADV (16.2.3.7)

Inuk Jung

LG Electronics

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 106, line 1, replace Table 685 with the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685—Parameters for AAI-SON-ADV message

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Type | 3 | Used to indicate the purpose of this message 0b000: ABS Reconfiguration 0b001: ABS Restart 0b010: ABS Scanning 0b011: ABS Reliability 0b100 : LDM parameter change | mandatory- |
| If (Action Type = 0b000) { |  |  |  |
| New IDcell | 10 | New IDcell that the ABS will use after the reconfiguration process. |  |
| Unavailable Start Tme (UST) |  | Start of unavailable time | mandatory- |
| Unavailable Time Interval (UTI) |  | Interval of unavailable time | mandatory- |
| } else if (Action Type = 0b001) { |  |  |  |
| Unavailable Start Tme (UST) |  | Start of unavailable time | mandatory- |
| Unavailable Time Interval (UTI) |  | Interval of unavailable time | mandatory- |
| } else if (Action Type = 0b010) { |  |  |  |
| Unavailable Start Tme (UST) |  | Start of unavailable time | mandatory |
| Unavailable Time Interval (UTI) |  | Interval of unavailable time | mandatory |
| } else if (Action Type = 0b011) { |  |  |  |
| Unavailable Start Tme (UST) |  | Start of unavailable time | optional |
| Unavailable Time Interval (UTI) |  | Interval of unavailable time | optional |
| Reason | 3 | 0b000: Power down 0b001: Power reduction 0b010: FA change 0b011: Backhaul link down 0b100: Disable subframes | mandatory |
| If (Reason = 0b000) { |  |  |  |
| Time of Power Down |  | Expected time when the Femto ABS will be powered down. | mandatory |
| Expected uptime of BS |  | Expected uptime of BS | optional |
| } else if (Reason = 0b001) { |  |  |  |
| Tx Power Reduction |  | dB value of Tx power reduction | mandatory |
| Expected time of power reduction |  | Expected resource adjustment time |  |
| } else if (Reason = 0b010) { |  |  |  |
| FA index |  | FA index | mandatory |
| Expected downtime of current FA |  | Expected current FA downtime |  |
| Expected uptime of new FA |  | Expected uptime of new FA | optional |
| } else if (Reason = 0b100) { |  |  |  |
| Disabled subframes index | 6 | The status of DL subframes Bit 0: the status of the first DL subframe Bit 1: the status of the 2nd DL subframe Bit 2: the status of the 3rd DL subframe Bit 3: the status of the 4th DL subframe Bit 4: the status of the 5th DL subframe Bit 5: the status of the 6th DL subframe (0 means disabled or no such subframe; 1 means enabled) | mandatory |
| } |  |  |  |
| } else if (Acton Type = 0b100) { |  |  |  |
| LDM Parameter -AI | 4 | The new AI for LDM | mandatory |
| LDM Parameter -UAI | 8 | The new UAI for LDM | mandatory |
| LDM Parameter –SFO | 9 | The new SFO for LDM | mandatory |
| } |  |  |  |
| Recommended BSID list |  | Recommended BSID list | It is optional to help AMS to HO. |

--------------------------------------------------  Text End  -----------------------------------------------------------------