| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Clean up for E-MBS MAP |  |
| Date Submitted | 2010-08-16 |  |
| Source(s) | Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Jaejeong Shim Samsung Electronics | Email : kjosiam@sta.samsung.com ty33.kim@samsung.com seonghyeon.chae@samsung.com wang.hai@samsung.com brian.shim@samsung.com |
| Re: | Contribution in support of a comment on P802.16m/D7currently under SB_16m |  |
| Abstract | This contribution suggests clean-up text for E-MBS MAP |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Clean up for E-MBS MAP (16.3.5.2.3)

Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Jaejeong Shim

Samsung Electronics

Text Proposal

--------------------------------------------------- Proposed Text # 1------------------------------------------------------------

Make the following changes to Section 16.3.5.2.3 beginning Line 1 on Page 541

16.3.5.2.3 Enhanced Multicast Broadcast Service MAP (E-MBS MAP)

The E-MBS MAP carries configuration information for enhanced multicast broadcast service for one E-MBS Zone. The E-MBS MAP is transmitted in the first several RUs of the E-MBS region in the beginning of the MSI. The parameters of the E-MBS region that include the SLRUs reserved for E-MBS are transmitted in AAI_SCD message in Section 16.2.3.30 and the parameters required to decode the E-MBS MAP that includes the burst size of the E-MBS MAP is transmitted in the AAI_E-MBS-CFG MAC control message described in 16.9.3.1. The first AAI subframe, excluded from the RUs of E-MBS, of each superframe is reserved for superframe header to transmit. AAI_E-MBS-CFG Change Indication indicates any change in the parameters of AAI_E-MBS-CFG MAC Control Message at its next transmission instance.

-------------------------------------------------------Proposed Text #1--------------------------------------------------

-------------------------------------------------------Proposed Text #2--------------------------------------------------

Make the following changes to Section 16.3.4.6 beginning Line 34 on Page 506

16.3.4.6 Downlink physical structure for E-MBS support

In a carrier that is not dedicated to E-MBS traffic, it may be required to multiplex E-MBS and unicast traffic. When multiplexed with unicast data, the E-MBS traffic is FDMed with the unicast traffic in the downlink subframes. More specifically, the E-MBS traffic is transmitted using subbands assigned in frequency partition FP0. The first AAI subframe in every superframe transmits SFH and does not contain any subbands. Therefore no E-MBS traffic is scheduled in the first AAI subframe of every superframe. The number of subband logical resource units (SLRUs) used for E-MBS data, denoted by KSB,E-MBS, is indicated in the Zone Allocation BitMAP in AAI-SCD message.

-------------------------------------------------------Proposed Text #2--------------------------------------------------

---------------------------------------------------- Proposed Text #3 ------------------------------------------------

Modify Table 864 beginning line 6 on Page 864 in IEEE P802.16m/D7 as follows.

Table 830—E-MBS DATA Information Elements

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS_DATA_IE () { | - | - |
| No. of E-MBS IDs+FIDs | 8 | Total number of E-MBS streams in the IE |
| For (i=0; i< No. of E-MBS ID +FID’s, i++){ | - |  |
| E-MBS ID+FID | 16 | E-MBS ID + Flow ID of an E-MBS stream |
| Isize-offset | 5 | Depends on supported modes, 32modes assumed as baseline |
| MEF | 1 | E-MBS MIMO Encoding Format 0b0: SFBC 0b1: Vertical Encoding |
| If (MEF == 0b1){ |  | Parameters of Vertical Encoding |
| Mt | 1 | Number of streams for transmission for Nt =2, Nt =4, and Nt =8 (Mt <= Nt) 0b0: 1 stream 0b1: 2 streams |
| } |  |  |
| E-MBS AAI subframe offset | variable | Includes the location index of the AAI subframe where the E-MBS data burst ends MSI == 0b00: 6 bits MSI == 0b01: 7 bits MSI == 0b10: 8 bits MSI == 0b11: 9 bits The AAI subframe offset index begins at the beginning of the MSI. |
| E-MBS Resource Indexing | 7 | Includes the location of the SLRU index where the E-MBS data burst ends. The first E-MBS stream begins in the SLRU immediately after E-MBS MAP in the E-MBS region The E-MBS resource indexing field indexes SLRUs of a subframe in the E-MBS region as indicated in the Zone Allocation bitmap transmitted in the AAI_SCD message described in Section 16.2.3.30. |
| } |  |  |

-----------------------------------------------------End of the Text Proposal 2 ------------------------------------------------