| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On Unicasting Information of Accessible NBR Femto ABSs |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Ying Li, Anshuman Nigam, Jaehyuk Jang, Jung Je Son, Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D7change requests. |  |
| Abstract | This contribution is to propose changes in support femtocells. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

On Unicasting Information of Accessible NBR Femto ABSs

Ying Li, Anshuman Nigam, Jaehyuk Jang, Jung Je Son,

Kaushik Josiam, Sudhir Ramakrishna, Zhouyue Pi

Samsung Electronics

# Introduction

- Problem: In the current draft D7, it has text saying

- The AAI_SCN-REP may contain the neighbor request indication, to which the ABS may unicast an AAI_NBR-ADV that includes a list of femto ABSs which is formed based on the reported FA, A-preamble indexor BSIDs, or the reported measurement

- To send the AMS the unicasted list of NBR accessible FBS is a valid idea. However, there could be problems to use unicasted AAI_NBR-ADV. For example,

- In SCN-REQ/SCN-RSP, there is some field indicating whether it is for the ABS in AAI_NBR-ADV, or not.

- In some other messages, there is some field related to ‘the index of the ABS listed in AAI_NBR-ADV’

- Suppose an AMS/ABS receives/sends a broadcasted AAI_NBR-ADV, AND a unicasted AAI_NBR-ADV only with femtos in the list, which AAI_NBR-ADV shall the AMS/ABS interpret when the AMS/ABS reads “AAI_NBR-ADV” such as in case a) and b), the broadcasted one, or the unicasted one??

- Remedy:

- Option 1: Make a separate new message, to unicast information of accessible NBR Femto ABSs. This option would be adding a lot of work to generate a new message.

- Option 2: Make the scanning response to unicast the information of accessible NBR Femto ABSs. In this proposal, we choose Option 2.

# Proposed Text

Please revise D7 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

[Modify Page 812, Line 12]

After scanning and identifying the existence of any Femto ABSs, the AMS may report FA, A-Preamble index or BS IDs and measurement results according to the Trigger conditions included in the AAI_SCD message by sending AAI_SCN-REP. The AAI_SCN-REP may contain the neighbor request indication, to which the ABS may unicast an AAI_NBR-ADV respond with AAI_SCN-RSP that includes a list of femto ABSs which is formed based on the reported FA, A-preamble indexor BSIDs, or the reported measurement.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------