| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_Global-CFG Message Clean Up (16.2.3.57) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng MediaTek Inc. Chia-Lung Tsai, Ping-Heng Kuo, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting ITRI Hyunjeong Kang, Jaeweon Cho Samsung Soojung Jung ETRI JinSam Kwak LG | IK.Fu@mediatek.com kelvin.chou@mediatek.com pk.liao@mediatek.com ythsieh@itri.org.tw hyunjeong.kang@samsung.com sjjung@etri.re.kr jinsam.kwak@lge.com |
| Re: | P802.16m/D7 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_Global-CFG Message Clean Up

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#3, page#225 of P802.16m/D7]

16.2.3.57 AAI_Global-CFG (global carrier configuration) Message

The AAI_Global-CFG message provides the carrier information for all available carriers in the network. The ABS transmits the AAI_Global-CFG message to an AMS after transmitting the AAI_REG-RSP message.

The ASN.1 code of the AAI_Global-CFG message follows the format depicted in Table 759.

Table 759-AAI_Global-CFG MAC Control Message Format

| Field | Size (bit) | Description | Condition |
| --- | --- | --- | --- |
| MAC Control Message Type. | 8 |  |  |
| Global carrier configuration change count | 3 | Indicates the value of Global carrier configuration change count of the network, will be incremented each time the contents of this message change. Value range is 01-7 | Mandatory |
| Number of Carrier Groups. | 4 | Groups of contiguous carriers whose subcarriers are aligned with a frequency offset In case of FDD, only DL carrier groups are included. Value Range is 1(0x0)~16 (0xF) | Mandatory Note that the Number of Carrier Groups cannot be zero |
| For (i=0; i< Number of Carrier Groups; i++){ |  |  |  |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD | Mandatory |
| Number of Carriers | 43 | Number of component carriers in this configuration. Value Range is 1(0x0)~16 (0xF) | Mandatory |
| If(Number of Carriers ==0x0){ |  |  |  |
| Center Frequency Assignment Index | 818 | Frequency Assignment Index of the carrier Center frequency (in unit of 50 kHz) of the carrier in the i-th carrier group | Shall be present when Number of Carriers equal to one |
| Physical Carrier Index | 46 | Index of the physical carrier | Shall be present when Number of Carriers equal to one |
| If(Duplexing_Mode==1){ |  |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier | Shall be present when Number of Carriers equal to one and Duplexing_Mode equal to one |
| } |  |  |  |
| }else { |  |  |  |
| Configuration Type | 1 | 0b0 : All carriers have same bandwidth in the carrier group 0b1: Multiple bandwidths are mixed for the carrier group | Shall be present when Number of Carriers larger than one |
| If(Configuration Type ==0b0){ |  | Single bandwidth in the carrier group |  |
| Bandwidth | 3 | Bandwidth of the component carrier 0b000 : 5MHz 0b001 : 10MHz 0b010 : 20MHz 0b011 : 7MHz 0b100 : 8.75MHz 0b101~: reserved | Shall be present when Number of Carriers larger than one and Configuration Type equal to zero |
| } |  |  |  |
| Start Frequency | 18 | Center frequency (in unit of 50 kHz) of the carrier located at lowest frequency position in the i-th carrier group | Shall be present when Number of Carriers larger than one |
| Number of Carriers | 3 |  |  |
| For(j=0;j<Number of Carriers; j++){ |  |  |  |
| Physical Carrier Index | 46 | Index of the physical carrier | Shall be present when Number of Carriers larger than one |
| If(Configuration Type ==0b1){ |  |  |  |
| Bandwidth | 26 | Bandwidth of the component carrier 0b00 : 5MHz 0b01 : 10MHz 0b10 : 20MHz 0b11 : reserved | Shall be present when Number of Carriers larger than one and Configuration Type equal to one |
| } |  |  |  |
| If(Duplexing_Mode==1){ |  |  |  |
| Carrier Type | 1 | Indicates the type of this physical carrier. 0b0: Fully configured carrier 0b1: Partially configured carrier | Shall be present when Number of Carriers larger than one and Duplexing_Mode equal to one |
| } |  |  |  |
| Center Frequency Shift Frequency offset | 3 | The value of center frequency shift of the current carrier for sub-carrier alignment (kHz) 0b000: 0 0b001: 1.5625 0b010: 3.125 0b011: 4.6875 0b100: -4.6875 0b101: -3.125 0b110: -1.5625 {0, 1.5625, 3.125, 4.6875, 0, -1.5625, -3.125, -4.6875}kHz |  |
| } |  |  |  |
| } |  |  |  |

---------------------------------------------------------End of the Text-----------------------------------------------------------