| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-REQ & AAI_MC-RSP Messages Clean Up (16.2.3.52) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com kelvin.chou@mediatek.com pk.liao@mediatek.com |
| Re: | P802.16m/D7 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-REQ & AAI_MC-RSP Messages Clean Up

I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Remedy 1]

[Adopt the following text modifications from line#59, page#216 of P802.16m/D7]

16.2.3.52 AAI_MC-REQ (multicarrier Request) Message

The Multi-Carrier Request Message (AAI_MC-REQ) is sent by an AMS to an ABS to request the list of Assigned Carriers. The AMS cannot send the AAI_MC-REQ message until it receives the AAI_Global-CFGConfig message and AAI_MC-ADV message from its serving ABS. According to the multi-carrier configuration supported by serving ABS, which is indicated in AAI_MC-ADV, the AMS shall determine the subset of carriers which it can simultaneously support process under its hardware capability. Then AMS will send AAI_MC-REQ message to the serving ABS and include sets of physical carrier index to inform ABS this information.

The ASN.1 code of AAI_MC-REQ message follows the format depicted in Table 753.

Table 753—AAI_MC-REQ message format

| Field | Size (bits) | Notes | Condition |
| --- | --- | --- | --- |
| Global_Support | 1 | Indicates whether the AMS can simultaneously process all the carriers supported by the serving ABS 0b0: The AMS can only process a subset of the carriers supported by the serving ABS 0b1: The AMS can simultaneously process all carriers supported by the serving ABS | Mandatory |
| If (Global_Support==0){ |  |  |  |
| Number of Candidate Combinations (N) | 2 |  | Shall be present if Global_Support is equal to zero |
| For (i=0, i<N; i++) { |  |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  | Shall be present if Global_Support is equal to zero. Note that N cannot be zero. |
| For (j=0, j<Nc; j++) { |  |  |  |
| Physical Carrier Index | 46 | The carriers AMS can simultaneously process | Shall be present if Global_Support is equal to zero. Note that Nc cannot be zero. |
| } |  |  |  |
| DL/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present if Global_Support is equal to zero |
| } |  |  |  |
| } |  |  |  |
| Support of data transmission over guard sub-carrier inbetween two adjacent carriers that are subcarrier aligned | 1 | 0b0 = not supported 0b1=support | Mandatory |

Table 756—Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the AMS can simultaneously process all the carriers supported by serving ABS 0b0: The AMS can only process a subset of the carriers supported by serving ABS 0b1: The AMS can simultaneously process all carriers supported by serving ABS |  |
| O | Number of candidate combinations | 2 | Indicates the number of carrier combination AMS can simultaneously process | Shall be present when Global Support is zero |
| O | Number of candidate assigned carrier | 3 | Indicates the number of carriers AMS can simultaneously process | Shall be present when the number of candidate combinations is not zero |
| O | Physical carrier index | 4 | Indicates the carriers AMS can simultaneously process | Shall be present when the number of candidate assigned carrier is not zero |
| O | DU/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present when the number of candidate combinations is not zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |

[Remedy 2]

[Adopt the following text modifications from line#46, page#218 of P802.16m/D7]

16.2.3.53 AAI_MC-RSP (multicarrier Response) Message

Based on information provided by the AMS in the AAI_MC-REQ message, the ABS shall respond to the AMS through the AAI_MC-RSP message to provide the AMS with information about its assigned carriers. The following are the parameters shall be included in the AAI_MC-RSP message:

The ASN.1 code of AAI_MC-RSP message follows the format depicted in Table 755.

Table 755—AAI_MC-RSP message format

| Field | Size (bits) | Notes | Condition |
| --- | --- | --- | --- |
| Global_Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS | Mandatory |
| If (Global_Assign == 0) { |  |  |  |
| Number of Assigned Carriers (N) | 3 | Number of Assigned Secondary Carriers. | Shall be presented if Global_Assign equals to zero |
| For (i=0; i<N; i++) { |  |  |  |
| Physical carrier index | 6 | The index refers to a Physical carrier index in AAI_MC-ADV message. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be presented if Global_Assign equals to zero. Note that N cannot be zero. |
| DL/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. | Shall be presented if Global_Assign equals to zero. Note that N cannot be zero. |
| Support of data transmission over guard sub-carrier inbetween two adjacent carriers that are subcarrier aligned | 1 | 0b0 = not supported 0b1 = support | Shall be presented if Global_Assign equals to zero. Note that N cannot be zero. |
| } |  |  |  |
| } |  |  |  |

Table 755—Contents of AAI_MC-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Assign | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers | 3 | Indicates the number of carriers to be assigned to the AMS | Shall be present when the Global Assign is zero |
| O | Physical carrier index | 4 | Indicates the carriers to be assigned to the AMS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be present when the Global Assign is zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |
| O | DU/UL indic | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. | Shall be present when the Global Assign is zero |

---------------------------------------------------------End of the Text-----------------------------------------------------------