| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_MC-ADV Message Clean Up (16.2.3.56) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com kelvin.chou@mediatek.com pk.liao@mediatek.com |
| Re: | P802.16m/D7 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_MC-ADV Message Clean Up

I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#17, page#222 of P802.16m/D7]

16.2.3.56 AAI_MC-ADV (multicarrier Advertisement) Message

The ABS which supports multiple RF carriers shall periodically broadcast AAI_MC-ADV message for the reception by all AMSs.

Table 758—AAI_MC-ADV message format

| Field | Size (bit) | Descrpition |
| --- | --- | --- |
| Multi-carrier configuration change count | 4 | The value is incremented by 1 upon each update |
| Serving BS Carrier Number | 3 | Indicates the number of carriers supported by the serving ABS |
| MAC Protocol version | 8 | Indicates MAC protocol version that the current carrier supports: it is consistent with REV.2 definition with new MAC protocol version 9 defined for 16m |
| For(i=1;i<=Serving BS Carrier Number-1; i++){ |  |  |
| PGID | 16 | Indicate Paging group identifier where the carrier belongs |
| Physical carrier index | 6 | Indicates the carrier index that the ABS carries except current carrier: the physical carrier index refers to AAI_Global-CFG message |
| SA-Preamble Index | 10 | Indicates the SA-Preamble index of the carrier |
| Paging carrier indicator | 1 | Indicates whether this carrier is a paging carrier or not 0: no paging carrier 1: paging carrier |
| SFH encoding format | 2 | 0b00: full SFH subpacket and AAI_SCD information 0b01: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of current carrier which transmits this AAI_MC-ADV message 0b10: delta encoding of SFH_Info (SFH subpacket and AAI_SCD) with reference to the information of preceding carrier 0b11: reserved |
| S-SFH change count | 4 | Change count of S-SFH SP IE(s) |
| Change count for AAI_SCD | 4 | Change configuration count of AAI_SCD |
| If (SFH_encoding_format==0b00) { |  |  |
| SFHSubpkt 1 | Variable | SFH subpacket 1 is included, length depends on FFT size. |
| SFHSubpkt 2 | Variable | SFH subpacket 2 is included, length depends on FFT size. |
| SFHSubpkt 3 | Variable | SFH subpacket 3 is included |
| AAI_SCD_Info | Variable | Information from AAI_SCD is included |
| } |  |  |
| If (SFH_encoding_format==0b01 or 0b10) { |  |  |
| Delta information | Variable | Indicates delta encoding between the reference carrier and the current carrier where this message is transmitted if SFH_encoding_format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format = 0b10 Delta information may contain SFH SP1, SP2, and SP3 attributes as defined in Table 840, Table 841, and table 842, respectively. It may contain the following AAI_SCD attributes: •BS_Restart_Count •SA_PreamblePartitionforBStype •Trigger definitions •NormalizedCINR •OL-Region-Type0-ON •OL-Region-Type2-SLRU-Size •periodicityOfRngChSync •cntlStartCodeOfRngChSync •rangingPreambleCodeSync •UlpcDataChannelSet •gammaIotFp0 •gammaIotFp1 •gammaIotFp2 •gammaIotFp3 •Alpha •Beta •dataSinrMin •dataSinrMax •targetHarqSinr •targetSyncRangingSinr •targetPfbchSinr •targetSfbchBaseSinr •targetSfbchDeltaSinr •targetBwRequestSinr •gammaIotSounding •soundingSinrMin •T_ReTx_Interval •BR_Channel Configuration MIN Access Class of the (i+0)-th frame •BR_Channel Configuration MIN Access Class of the (i+1)-th frame •BR_Channel Configuration MIN Access Class of the (i+2)-th frame •BR_Channel Configuration MIN Access Class of the (i+3)-th frame •SoundingMultiplexingType •decimationValueD •maxCyclicShiftIndexP •shiftValueUForSoundingSymbol •16m_Relay_zone_AMS_allocation_indicator •resourceMetricFP2 •resourceMetricFP3 Each delta information attribute is optional. |
| } |  |  |
| } |  |  |

[Delete the original Table 758 started from line#26, page#222 until line#65, page#224 of P802.16m/D7]

---------------------------------------------------------End of the Text-----------------------------------------------------------