| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Frame Structure for Mixed Deployment over Multiple Carriers (16.3.3.6.1) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com kelvin.chou@mediatek.com pk.liao@mediatek.com |
| Re: | P802.16m/D7 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Frame Structure for Mixed Deployment over Multiple Carriers

I-Kang Fu, Kelvin Chou, Pei-Kai Liao, Paul Cheng

MediaTek Inc.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modifications from line#44, page#459 of P802.16m/D7]

16.3.3.6 Frame structure supporting wider bandwidth

In order to support wider bandwidth, multiple RF carriers can be used for network deployment. The ABS may serve an AMS over one of the carriers by single-carrier operation or serve an AMS over multiple carriers by multicarrier operation.

When the network is deployed over multiple RF carriers, the The same frame structure (16.3.3.1, 16.3.3.2, 16.3.3.3) is used for each carrier regardless of supporting in multicarrier mode operation or not. Each carrier shall have its own superframe header. Some carriers may have only part of superframe header. Figure 486 illustrates the example of the frame structure to support deployment over multiple RF carriers multicarrier operation. For FDD UL, the preamble and superframe headers are replaced with traffic OFDMA symbols.

The multiple carriers involved in multicarrier operation may be in a contiguous or non-contiguous spectrum. When carriers are in the same spectrum and adjacent and when the separation of center frequency between two adjacent carriers is multiples of subcarrier spacing, no guard subcarriers are necessary between adjacent carriers.

Each AMS is controlled through an RF carrier which is the primary carrier. When multicarrier feature is supported, the system may define and utilize additional RF carriers to improve the user experience and QoS or provide services through additional RF carriers configured or optimized for specific services. These additional RF carriers from AMS perspective are defined as the secondary carriers. The detailed description of the multicarrier operation is referred can be found in 16.2.8.

[Adopt the following text modifications from line#46, page#460 of P802.16m/D7]

Figure 486—Example of the frame structure to support deployment over multiple RF carriers multicarrier operation

---------------------------------------------------------End of the Text-----------------------------------------------------------