| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Handover Message Tables (16.2.3.9) |  |
| Date Submitted | 2010-08-16 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D7) |  |
| Abstract | Clarification on the tables for handover MAC messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Handover Message Tables (16.2.3.9)

Kelvin Chou

MediaTek Inc.

Introduction

Current MAC control message table in 16m/D7 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for Handover MAC control messages.

Proposed Text

Remedy 1:

In the P802.16m/D7, page 105, line 3, replace Table 689 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 689—AAI_HO-IND Parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| HO Event Code | 2 | Used to distinguish AAI_HO-IND among different scenarios 0b00: Target ABS selection in case of multiple candidate T-ABSs in HO-CMD. The serving ABS does not meet the "ABS unreachable" triggers. 0b01: All target ABSs in AAI_HO-CMD are unreachable. The AMS includes a new target ABS that was not included in AAI_HO-CMD. The serving ABS does not meet the "ABS unreachable" triggers. 0b10: The serving ABS meets the "ABS unreachable" triggers. The AMS will attempt network re-entry at the indicated T-ABS. 0b11: HO cancel The serving ABS does not meet the "ABS unreachable" triggers. |  |
| If (HO Event Code = 0b00 or 0b01 or 0b10) { |  |  |  |
| Target ABS ID | 48 |  |  |
| TABS Physical Carrier Index | 6 | Physical carrier index of the Target ABS where an AMS will perform network reentry procedure | May be included when the target ABS is a multi-carrier ABS |
| AMS Physical carrier index at the serving ABS | 6 | Recommended carrier index of the AMS which was used in the serving ABS, and to be used for network reentry to the target ABS. All other AMS’s carriers which was used in the serving ABS can be used for data communication with the serving ABS. | May be included when AMS's multicarrier capability = 0b10, HO Reentry Mode = 1 and the target carrier is different from the serving carrier |
| }else if If (HO Event Code = 0b11) { |  |  |  |
| SFH mismatch indication | 1 | 1: the AMS does not have the latest SFH for any of the T-ABSs included in the AAI_HO-CMD message; 0: otherwise |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D7, page 107, line 11, replace Table 690 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 690—AAI_HO-REQ Parameters

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI_NBR-ADV Change count | 8 | AAI_NBR-ADV change count last received from the serving ABS | N.A. |
| for i= 1 to N_New_ABS_Index { |  | N_New_ABS_Index is the number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message. |  |
| Neighbor_ABS_Index | 8 | ABS index corresponds to AAI_NBR-ADV. |  |
| ABS CINR mean | 8 | CINR mean of the new ABS | Either ABS CINR mean for ABS RSSI mean shall be included. |
| ABS RSSI mean | 8 | RSSI mean of the new ABS | Either ABS CINR mean for ABS RSSI mean shall be included. |
| Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when AMS supports MC mode=0b010 or 0b011 or 0b100 |
| Physical_Carrier_Index | 6 | Physical carrier index of the ABS | May be included for each ABS with ABS Index or ABS ID when the ABS is multi-carrier ABS |
| } |  |  |  |
| for i= 1 to N_New_ABS_Full { |  | N_New_ABS_ID is the number of neighboring ABSs to be are identified by full considered for handover, which is 48 bit ABSID |  |
| Neighbor_ABS_ID | 8 | The 48 bits neighbor ABSID |  |
| ABS CINR mean | 8 | CINR mean of the new ABS | Either ABS CINR mean for ABS RSSI mean shall be included. |
| ABS RSSI mean | 8 | RSSI mean of the new ABS | Either ABS CINR mean for ABS RSSI mean shall be included. |
| Carrier Preassignment Indication | 1 | Indicates whether AMS needs pre-assignment of secondary carriers at the Target ABS. | May be included when AMS supports MC mode=0b010 or 0b011 or 0b100 |
| Physical_Carrier_Index | 6 | Physical carrier index of the ABS | May be included for each ABS with ABS Index or ABS ID when the ABS is multi-carrier ABS |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------