| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text changes to the HARQ feedback channel section (16.3.5.3.2.2) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Roshni Srinivasan, Yi Hsuan Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | IEEE 802.16 Working Group Letter Ballot#31 Recirc Target topic: “IEEE P802.16m/D7, section 16.3.5.3.2.2”. |  |
| Abstract | The contribution proposes text changes to clean up the section describing the HARQ feedback channel (section 16.3.5.3.2.2) in P802.16m/D7. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text changes to the HARQ feedback channel section (16.3.5.3.2.2)

Roshni Srinivasan, Yi Hsuan

Intel Corporation

- Introduction

The contribution proposes text changes to clean up the section describing the HARQ feedback channel (section 16.3.5.3.2.2) in the 802.16m amendment [1].

- References

[1] IEEE P802.16m/D7, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- Proposed changes to section 16.3.5.3.2.2. in P802.16m/D7

Remedy-1: Modify the text on page 544 between lines 31 and 66 as follows:

-------------------------------------------------------------Begin Proposed Text Changes----------------------------------------------------------------

Figure 527 shows the construction procedure of HF-A-MAP symbols from HF-A-MAP-IEs for constituting an HF-A-MAP group. An HF-A-MAP region has NHF-A-MAP/4 HF-A-MAP groups and each group of HF-A-MAP consists of 4 HF-A-MAPs numbered as 4c, 4c+1, 4c+2, 4c+3 where NHF-A-MAP is the total maximum number of HF-A-MAP-IEs configured per HF-A-MAP region, and c (0≤ c < NHF-A-MAP/4) is the HF-A-MAP group index. The HF-A-MAP IE resource index is then denoted as 4c+k , where(0≤ k < 4)  .

Each HF-A-MAP IE carries 1 bit information. Firstly, it this bit is repeated NRep, HF-A-MAP times, where NRep, HF-A-MAP is 8. A bitwise XOR operation And is then performed on the repeated HF-A-MAP IE bit with the NRep, HF-A-MAP LSBs of the STID or the RAID for of the associated AMS to which the HF-A-MAP IE is assigned. are applied to the repeated HF-A-MAP IE bits by bitwise XOR operation. Then, Following this operation, 0 is mapped to +1 and 1 is mapped to -1.

After scrambling, power scaling by sqrt(Pi) (0≤ i < NHF-A-MAP) is applied to the sequence transmitted in to the ith ith HF-A-MAP resource where sqrt(Pi) is the value determined by the management entity to satisfy the link performance. Note that if the jth jth HF-A-MAP resource is not allocated to any AMSs, Pj is set to 0.

Each power-scaled sequence is mapped to either real part or imaginary part in the signal constellation and multiplexed with other sequence s, if they exist. As shown in Figure 527, HF-A-MAP IE 4c occupies the real part of the first symbol in each tone pair, and HF-A-MAP IE 4c+1 occupies the imaginary part of the first symbol in each tone pair. In the same way, HF-A-MAP IE 4c+2 occupies the real part of the second symbol in each tone pair, and HF-A-MAP IE 4c+3 occupies the imaginary part of the second symbol in each tone pair.

-------------------------------------------------------------End Proposed Text Changes----------------------------------------------------------------

Remedy-2: Modify the text on page 545 between lines 22 and 56 as follows:

-------------------------------------------------------------Begin Proposed Text Changes----------------------------------------------------------------

Within each HF-A-MAP region, the ABS allocates the an HF-A-MAP resource index corresponding to an HARQ burst allocated to the an AMS using following rules.

For group resource allocation, the HF-A-MAP resource index for the lth AMS in the GRA allocation is (istart+ [l. NHF-A-MAP / NGRA]) mod NHF-A-MAP), where istart is the HFA Offset in the group resource allocation A-MAP IE for UL allocation, NHF-A-MAP is the total maximum number of HF-A-MAP IEs configured per HF-A-MAP region, and NGRA is the User Bitmap Size in the group resource allocation A-MAP IE for UL allocations.

For the BR-ACK A-MAP IE, the HF-A-MAP resource index for the lth AMS grant in the BR-ACK bitmap is (0≤ i < NHF-A-MAP)  , where istart is the HFA start offset in the BR-ACK A-MAP IE, NHF-A-MAP is the total maximum number of HF-A-MAP IEs channels configured per HF-A-MAP region, and NBR-ACK is the number of AMSs with grants in the BR-ACK A-MAP IE.

For resource allocation using the UL Basic Assignment A-MAP IE, UL Subband Assignment A-MAP IE, CDMA Allocation A-MAP IE, Feedback Polling A-MAP IE, and the UL Persistent Allocation A-MAP IE, the HF-A-MAP resource index is (M(j)+n) mod NHF-A-MAP , where j {0,1} is the value of the HF-A-MAP Index Parameter in the Non-user specific A-MAP IE, n is a 3 bit HFA value in each assignment A-MAP IE, and NHF-A-MAP is the total maximum number of HF-A-MAP IEs channels configured per HF-A-MAP region.

In order to determine a unique set of HARQ A-MAP resource indexes in a subframe, the ABS can set j to 0. In this case, M(j) is set to the STID for the UL Basic Assignment A-MAP IE and UL Subband Assignment A-MAP IE and M(j) is set to the RA-ID for CDMA Allocation A-MAP IE when j=0. Alternatively, and a different set of HARQ A-MAP resource indexes can be obtained when M(j) is set to the lowest LRU index of the corresponding UL transmission when j=1. For the UL Persistent Allocation A-MAP IE and Feedback Polling A-MAP IE, M(j) is always set to the STID regardless of the value of j.

-------------------------------------------------------------End Proposed Text Changes---------------------------------------------------------------