| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Coverage Enhancement for MAC management messages and BR Header |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Jon Labs, Wavesat |  |
| Re: | Maintenance Change Request 0044 in IEEE 802.16maint-09/0007r9; IEEE L802.16-10/0070r1, Annex D |  |
| Abstract | Clarifications proposed to the IEEE 802.16 baseline standard on ambiguities for UL Coverage Enhancement for MAC management messages and BR Header |  |
| Purpose | Formally introduce the changes into the IEEE 802.16 standard by way of the P802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Coverage Enhancement for MAC management messages and BR Header

Jon Labs, Wavesat

Problem Statement

[Re: Maintenance Change Request 0044 in IEEE 802.16maint-09/0007r9]

[Re: IEEE L802.16-10/0070r1, Annex D]

In IEEE 802.16-2009, HARQ which brings benefits to extend downlink/uplink coverage can be applied for management messages as well as data. For coverage extension, HARQ is required for RNG-REQ, SBC-REQ and BRH messages.

However, HARQ can be used to transmit the management message only after exchanging the SBC-REQ/RSP messages because HARQ parameters are negotiated through the SBC-REQ/RSP messages.

Moreover, if the MS wants to receive uplink resources using existing HARQ UL-MAP IEs, it requires basic CID. But, during network (re)entry, the MS does not have any CID.

Even further, BS cannot distinguish between HARQ-applied burst and HARQ-non-applied bursts unless it allocates uplink resources using different MAP IE (i.e., using normal UL-MAP IE or HARQ UL-MAP IE).

Proposed Solution

[Insert the following change language text on page 17 of P802.16m/D7 after line 49:]:

[Update Table 38 in section 6.3.2.3, page 79, in IEEE802.16-2009 as follows:]

Table 38— MAC management messages

| Type | Message name | Message description | Connection |
| --- | --- | --- | --- |
| … |  |  |  |
| 4 | RNG-REQ | Ranging request | Initial ranging, or basic or Primary management |
| … |  |  |  |
| 26 | SBC-REQ | SS basic capability request | Basic or Primary management |
| … |  |  |  |

[Insert the following change language text on page 19 of P802.16m/D7 after line 60:]:

[Add following new section 6.3.2.3.23.1 right after section 6.3.2.3.23 in page 132, in IEEE802.16-2009 as follows:]

6.3.2.3.23 SBC-REQ (SS basic capability request) message

….

6.3.2.3.23.1 HARQ support

When the BS has successfully received a ranging code from the Initial/HO Ranging Code set 1 (see Types 219 or 220 in Table 571) from the MS, the BS should use HARQ UL-MAP IE with Basic CID rather than normal UL-MAP IE to allocate uplink bandwidth for the SBC-REQ message.

When the MS sends the SBC-REQ message using uplink resources allocated by HARQ UL-MAP IE, the MS shall set the CID field in the MAC header to the Primary Management CID. If uplink resources allocated by HARQ UL-MAP IE are not sufficient to send the SBC-REQ message, the MS may fragment the SBC-REQ message using Fragmentation Subheader (FSH). If the SBC-REQ message is fragmented, the MS shall start the T18 timer when it transmits the first fragment of the SBC-REQ message. If the BS responds to a SBC-REQ message which is fragmented, the BS shall respond with a SBC-RSP message after it receives all the fragments of the SBC-REQ message.

When the MS transmits SBC-REQ message using the UL allocation specified by HARQ UL-MAP IE, the BS shall use Chase combining HARQ on the UL burst containing the SBC-REQ message. In this case, The BS shall use HARQ capability of the HARQ set 1 defined in the OFDMA parameter set TLV for HARQ operations until completion of the capability negotiation through SBC-REQ/RSP.

[Insert the following change language text on page 23 of P802.16m/D7 after line 24:]:

[Add following new section 6.3.6.5.1 right after section 6.3.6.5 in page 301, in IEEE802.16-2009 as follows:]

6.3.6.5 Contention-based CDMA BRs for WirelessMAN-OFDMA

…

6.3.6.5.1 Contention-based CDMA bandwidth requests using soft combining

When the BS allocates uplink bandwidth in response to receiving a ranging code from BR Ranging Code set 1 (see Type 221 in Table 571), the BS shall perform the allocation by using the CDMA_Allocation_IE. If the MS receives an uplink burst allocation in the CDMA_Allocation_IE, the MS shall transmit a BR header as described in 6.3.6.5.

To request the MS to retransmit the bandwidth request, the BS shall transmit a CDMA_Allocation_IE with the same ranging attributes (frame number index, ranging code, ranging symbol, and ranging subchannel) transmitted in the original CDMA_Allocation_IE. When the MS receives the CDMA_Allocation_IE with the same parameters sent in the original CDMA_Allocation_IE, the MS shall transmit the same BR header corresponding to the ranging attributes in the CDMA allocation IE. The CDMA allocation retransmission process shall be terminated by the frame N+15 where N is the frame number when the MS sends the BR Ranging Code. The BS should perform soft combining on subsequent BRH transmissions in the UL bursts in the allocations made with the same CDMA_Allocation_IE within 16 frames.

[Insert the following change language text on page 23 of P802.16m/D7 after line 24:]:

[Modify section 6.3.10.3 in IEEE802.16-2009 as follows:]

6.3.10.3 OFDMA-based ranging

The WirelessMAN-OFDMA PHY specifies a ranging subchannel and a set of special pseudonoise ranging codes. Subsets of codes shall be allocated in the UCD channel encoding for initial ranging, periodic ranging requests, and BRs, and Initial/HO/BR Ranging Code set 1 (see Types 219, 220 and 221 in Table 571)  so that the BS can determine the purpose of the received code by the subset to which the code belongs. An example of ranging channel in OFDMA frame structure is specified in Figure 228.

[Add following new section 6.3.10.3.1.1 right before section 6.3.10.3.2 in the middle of page 352, in IEEE802.16-2009 as follows:]

6.3.10.3.1 Contention-based initial ranging and automatic adjustments

….

6.3.10.3.1.1 OFDMA- initial ranging with soft combining

An MS may send a ranging code from the Initial Ranging Code set 1 which is defined by Type 219 in Table 571 to perform Initial Network Entry. The MS shall use random backoff with the Ranging Backoff Start and Ranging Backoff End described in the UCD message when it sends the Initial Ranging Code set 1.

When the BS responds to an Initial Ranging Code set 1, and PHY corrections are not needed, the BS shall provide at least 25 bytes of uplink bandwidth by transmitting a CDMA_Allocation_IE for the MS to transmit RNG-REQ message. The MS shall use the bandwidth to send the MAC PDU carrying RNG-REQ message on Ranging CID with MS MAC address TLV and MAC Version TLV, Generic MAC Header, and MAC CRC-32.

To request retransmission of the RNG-REQ, the BS shall send a CDMA Allocation IE with the same ranging attributes (frame number index, ranging code, ranging symbol, and ranging subchannel) which were sent in the original CDMA Allocation_IE. The process of retransmitting the CDMA_Allocation_IE shall be terminated by the frame N+15 where N is the frame number when the MS sends a Ranging Code. When the MS receives the UL bandwidth allocation by the same CDMA_Allocation_IE before T3 expires, the MS shall transmit the same MAC PDU that carries the RNG-REQ corresponding to the ranging attributes in the CDMA_Allocation_IE . In this case, the BS should perform soft combining to achieve combining gain on the UL bursts transmitted by the MS as per the allocation made with the same CDMA_Allocation_IE.

Upon expiration of T3 timer, the MS may either transmit a ranging code to the BS or perform the BS re-selection procedure as described in 6.3.21.2.1.

Upon successful reception of the RNG-REQ message with Ranging CID, the BS  may respond with the RNG-RSP message. The RNG-RSP message shall include Ranging Status TLV with a status either of Success or Abort. If the Ranging Status TLV in the RNG-RSP message is Abort, the MS may perform the cell reselection procedure defined in section 6.3.21.2.1.

Figure XX— Message flows during Initial network entry with ranging code from the Initial Ranging Code set 1

[Modify section 6.3.10.4 in IEEE802.16-2009 as follows:]

6.3.10.4 CDMA HO ranging and automatic adjustment

An MS that wishes to perform HO ranging shall take a process similar to that defined in the initial ranging section with the following modifications.

In CDMA HO ranging process, the random selection is used instead of random backoff and the CDMA HO ranging code is used instead of the initial ranging code. The code is selected from the HO ranging domain as defined in 8.4.7.3. The MS may select a code from HO Ranging Code Set 1 (Type 220 in Table 571) to perform OFDMA HO ranging with soft combining as defined in 6.3.10.4.2.

Alternatively, if the BS is prenotified for the upcoming HO MS, it may provide bandwidth allocation information to the MS using Fast Ranging IE to send a RNG-REQ message.

[Add following new section 6.3.10.4.2 right before section 6.3.11 in page 358, in IEEE802.16-2009 as follows:]

6.3.10.4.2 OFDMA HO ranging with soft combining

An MS may send a ranging code from the HO Ranging Code set 1 (see Type 220 in Table 571) in order to perform network re-entry during HO, network re-entry from idle mode or Location Update.

When the BS responds to a code from HO Ranging Code set 1, and PHY corrections are not needed, the BS shall provide at least 24 bytes of uplink bandwidth by transmitting a CDMA_Allocation_IE for the MS to transmit RNG-REQ message. The MS shall use the bandwidth to send the MAC PDU carrying RNG-REQ message on Ranging CID with MS MAC address TLV, Generic MAC Header, GMSH and MAC CRC-32.

To request retransmission, the BS shall transmit a CDMA_Allocation_IE with the same ranging attributes (frame number index, ranging code, ranging symbol, and ranging subchannel) which were sent in the original CDMA Allocation_IE. The retransmission process shall be terminated by the frame N+15 where N is the frame number when the MS sends a Ranging Code. When the MS receives a CDMA_Allocation_IE with the parameters set to the same values as the original CDMA_Allocation_IE before T3 expires, the MS shall transmit the same MAC PDU. The BS should perform combining processing on the UL bursts allocated by the same CDMA_Allocation_IE which will carry the same RNG-REQ message.

Upon expiration of T3 timer, the MS may either transmit a ranging code to the BS or perform the cell reselection procedure as described in 6.3.21.2.1.

Upon successful reception of the first RNG-REQ message with Ranging CID, the BS may respond with the first RNG-RSP message. The first RNG-RSP message shall include Ranging Status TLV with a status either of Success or Abort. If the Ranging Status TLV in the first RNG-RSP message is Abort, the MS may perform the cell reselection procedure defined in section 6.3.21.2.1.

If the required TLV message elements cannot be accommodated in the uplink bandwidth assigned for the first RNG-REQ message, the MS shall indicate required additional bandwidth using GMSH in the first RNG-REQ message. If the GMSH is included in the first RNG-REQ message and the BS allocates bandwidth for the second RNG-REQ message, the BS shall perform the allocation for the second RNG-REQ message by using HARQ UL-MAP IE with the Basic CID assigned by RNG-RSP message which was sent in response to first RNG-REQ message.

When the MS sends the second RNG-REQ message through uplink resources allocated by HARQ UL-MAP IE, the CID field in the MAC header shall contain the Primary Management CID.

If uplink resources allocated by HARQ UL-MAP IE are not sufficient to send the second RNG-REQ message which shall include at least MAC Version TLV, the MS may fragment the second RNG-REQ message using a Fragmentation Subheader (FSH). If the RNG-REQ message is fragmented, the MS shall start the T3 timer at the transmission of the first fragment of the second RNG-REQ message. If the second RNG-REQ message is fragmented, the BS shall respond with a RNG-RSP message after it receives all fragments of the RNG-REQ message.

If T3 timer expires after transmission of the second RNG-REQ message, the MS shall either retransmit the second RNG-REQ message, or perform the BS re-selection procedure as described in 6.3.21.2.1.

When the BS responds to successful reception of the second RNG-REQ message, the BS shall respond with the second RNG-RSP message which shall include the Ranging Status TLV with a status either of Success or Abort. If the Ranging Status TLV in the second RNG-RSP message is Abort, the MS may perform the cell reselection process defined in section 6.3.21.2.1.

If the MS has transmitted a ranging code from the HO Ranging Code set 1, the HARQ capability of HARQ set 1 defined in the OFDMA parameter set TLV shall be used for HARQ operations for second RNG-REQ until the completion of the capability negotiation through SBC-REQ/RSP process.

Figure XY— Message flows during Network Re-Entry during HO, Network Re-Entry from Idle Mode, or Location Update with a ranging code from the HO Ranging Code set 1

[Insert the following change language text on page 33 of P802.16m/D7 after line 16:]:

[Modify section 8.4.7.3,  page 982, in IEEE802.16-2009 as follows:]

The number of available codes is 256, numbered 0..255. Each BS uses a subgroup of these codes, where the

subgroup is defined by a number S, 0≤ S ≤255. The group of codes shall be between S and

((S + O + N + M + L+K+J+H) mod 256).

- The first N codes produced are for initial ranging. Clock the PRBS generator 144 (S mod 256) times to 144 ((S + N) mod 256) – 1 times.

- The next M codes produced are for periodic ranging. Clock the PRBS generator 144 ((N + S) mod 256) times to 144 ((N + M + S) mod 256) – 1 times.

- The next L codes produced are for BRs. Clock the PRBS generator 144 ((N + M + S) mod 256) times to 144 ((N + M + L + S) mod 256) – 1 times.

- The next O codes produced are for HO ranging. Clock the PRBS generator 144 ((N + M + L + S)

mod 256) times to 144 ((N + M + L + O + S) mod 256) – 1 times.

For  the MS and BS supporting OFDMA- initial ranging with soft combining (see 6.3.10.3.1.1), OFDMA HO ranging with soft combining (see 6.3.10.4.2 and 6.2.2.3.23.1), and Contention-based CDMA bandwidth requests using soft combining (see 6.3.6.5.1), the corresponding codes shall be generated by the following methods.

- The next K codes produced are for initial ranging code for soft combining. Clock the PRBS generator 144 (( N + M + L + O + S) mod 256) times to 144 ((K+ N + M + L + O + S) mod 256) – 1 times.

- The next J codes produced are for BR ranging code for soft combining. Clock the PRBS generator 144 (( K+ N + M + L + O + S) mod 256) times to 144 ((J+ K+ N + M + L + O + S) mod 256) – 1 times.

- The next H codes produced are for HO ranging for soft combining. Clock the PRBS generator 144 (( J+ K+ N + M + L + O + S) mod 256) times to 144 ((H+ J+ K+ N + M + L + O + S) mod 256) – 1 times.

[Modify Table 554 on page 35 of P802.16m/D7 with the following change language:]

Table 554—Parameters and constants (continued)

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| SS, MS | T3 | Ranging response reception timeout following the transmission of a ranging request. | — | OFDMA: 60 msec: RNG-RSP after CDMA ranging or RNG-REQ during initial or periodic ranging 50 msec: RNG-RSP after RNGREQ during HO to negotiated target BS 200 msec: RNG-RSP after RNGREQ during HO to non-negotiated target BS 200 msec: RNG-RSP after RNGREQ during location update or re-entry from idle mode 80 msec: RNG-RSP after CDMA raning using Initial/HO Ranging Code set 1 during initial ranging/ handover/ location update/ re-entry from idle mode 100 msec: RNG-RSP after RNG-REQ on the Primary Management Connection during initial ranging/ handover/ location update/ re-entry from idle mode | 200 ms |
| … |  |  |  |  |  |
| SS | T18 | Wait for SBC-RSP timeout. | — | 50 ms: when SBC-REQ is on the Basic Connection. 100 ms: when SBC-REQ is on the Primary Management Connection. | << T9 |
| … |  |  |  |  |  |

[Insert the following change language text on page 37 of P802.16m/D7 after line 19:]:

11.3 UCD management message encodings

11.3.1 UCD channel encodings

[Modify Table 571, page 1179, in IEEE802.16-2009 as follows:]

Table 571—UCD PHY-specific channel encodings—WirelessMAN-OFDMA

| Name | Type (1 byte) | Length | Value |
| --- | --- | --- | --- |
| … |  |  |  |
| Start of ranging codes group | 155 | 1 | Indicates the starting number, S, of the group of codes used for this UL. If not specified, the default value shall be set to zero. All the ranging codes used on this UL shall be between S and ((S+O+N+M+L+K+J+H) mod 256) where N is the number of initial ranging codes M is the number of periodic ranging codes L is the number of BR codes O is the number of HO ranging codes H is the number of HO ranging codes in HO Ranging Code set 1 J is the number of BR ranging codes in BR Ranging Code set 1 K is the number of Initial ranging codes in Initial Ranging Code set 1 The range of values is 0 ≤ S ≤ 255. |
| … |  |  |  |
| Initial Ranging Code set 1 | 219 | 1 | Number of Initial ranging CDMA codes for soft combining. Possible values are 0–255. Default value is 0 |
| HO Ranging Code set 1 | 220 | 1 | Number of HO ranging CDMA codes for soft combining. Possible values are 0–255. Default value is 0 |
| BR Ranging Code set 1 | 221 | 1 | Number of BR ranging CDMA codes for soft combining Possible values are 0–255. Default value is 0 |