| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for MultiBS MIMO feedback |  |
| Date Submitted | 2010-08-15 |  |
| Source(s) | Shuangfeng Han, Taeyoung Kim, Jaeweon Cho, Heewon Kang Samsung Electronics | E-mail: Shuangf.han@samsung.com ty33.kim@samsung.com |
| Re: | IEEE 802.16m Sponsor Ballot: P802.16m/D7 |  |
| Abstract | The contribution proposes relative amplitude feedback for multiBS MIMO in P802.16m/D7 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for MultiBS MIMO feedback

- Shuangfeng Han, Taeyoung Kim, Jaeweon Cho, Heewon Kang

- Samsung electronics

# Introduction

IEEE 802.16m currently supports interference mitigation techniques based on joint MIMO transmission across multiple ABSs.  The ABS and AMS may optionally support one or both adaptive precoding based multiBS joint processing techniques, e.g. Closed-loop Macro Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission.  CL-MD is used with DL MIMO mode 2, while Co- MIMO is used with DL MIMO mode 4 (CL MU-MIMO).

In draft D7, feedback for Co-MIMO includes CQI, PMI report for serving and neighboring cell, and CPMI for concatenating PMIs. For MultiBS MU-MIMO, each collaborative ABS transmits several streams to several AMS, inter-stream interference needs careful consideration. Take collaborative ABSs as a super ABS, this super ABS receives independent channel information (in the form of PMI) for each involved ABS, thus lacking the composite channel information if the relative channel amplitude is not fed back (although CPMI is also feedback). This lack of composite channel information will directly result in inter-stream interference, because precoder for each AMS can not be designed correctly.

Targeting for a better Co-MIMO performance, we are proposing in this contribution the feedback of relative amplitude as periodic feedback.

# References

[1] IEEE P802.16m/D7

# Proposed Modifications

Section 16.2.3.38

---------------------------------------Start of text for proposed Modification -----------------------------------

[TO add 1 row from line 50 in table 722]

Table 722—AAI_MultiBS_MIMO_FBK message format

| O | tempBsid | 4 | Diversity set member ID refer 16.2.3.34. This ID indicates the ABS with largest channel amplitude | Only for ICT=0b11 |
| --- | --- | --- | --- | --- |

[TO add 1 row (under the last row) in table 722]

Table 722—AAI_MultiBS_MIMO_FBK message format

| O | D) RelativeAmp | 4 | Uniformly quantized relative channel amplitude (Normalized by the largest channel amplitude of collaborative ABS) | Only for ICT=0b11 |
| --- | --- | --- | --- | --- |

[TO add following text under Table 722.]

For Co-MIMO, AMS needs to feedback RelativeAmp of collaborative ABSs (including the serving ABS and the ABSs indicated in AAI_MULTI_BS_MIMO-RSP). In case the DL channel amplitude of the serving ABS is the largest, the i-th RelativeAmp feedback shall correspond to the i-th collaborative ABS in bitmap adjAbsBitmapMultiBSMIMO. In case the DL channel amplitude of the ABS which corresponds to the i-th “1” in

adjAbsBitmapMultiBSMIMO is the largest, the i-th RelativeAmp feedback shall correspond to the serving ABS.

---------------------------------------End of text for proposed Modification ----------------------------------------------

Section 16.5.1.1

[TO add 1 row under the ‘CPMI for neighboring cell’ in table 722]

---------------------------------------Start of text for proposed Modification -----------------------------------

Table 970—Feedback information for DL Multi-BS MIMO supported by codebook based feedback

| RelativeAmp | Relative channel amplitude (Normalized by the the largest channel amplitude of collaborative ABS). |
| --- | --- |

---------------------------------------End of text for proposed Modification ----------------------------------------------