| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Table Format for Idle mode MAC control message (16.2.3) |  |
| Date Submitted | 2010-08-14 |  |
| Source(s) | Hyunjeong Kang, Jaehyuk Jang, Youngbin Chang, Rakesh Taori Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #1 on P802.16m/D7 |  |
| Abstract | The contribution proposes to clean up MAC control message table format for idle mode messages (DREG-REQ/RSP, PAG-ADV, PGID_Info). |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested Table Format for Idle mode MAC control message

Hyunjeong Kang, Jaehyuk Jang, Youngbin Chang, Rakesh Taori

Samsung Electronics

- Introduction

Current MAC control message table in 16m/D7 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for idle mode MAC control messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Replace the Table 704 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| De-registration_Request_Code | 3 | Used to indicate the purpose of this message 0x00: AMS deregistration request from ABS and network 0x01: request for AMS deregistration from serving ABS and initiation of AMS idle mode. 0x02: response for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS. 0x03: reject for the unsolicited AAI_DREG-RSP message with action code 0x05 by the ABS. This code is applicable only when an AMS has a pending UL data to transmit. 0x04: request for AMS deregistration from serving ABS to enter DCR mode 0x05: response for the unsolicited AAI_DREG-RSP message with action code 0x00, 0x01, 0x02 or 0x03 0x06-0x07: reserved |  |
| If (De-registration_Request_Code == 0x01) { |  |  |  |
| Paging cycle request | 4 | Used to indicate Paging cycle recommended by MS 0b0000 - 0b1111: reserved |  |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain MS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain MS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 includes configuration of all Service Flows in the AMS as set by successful AAI_DSA and AAI_DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information) |  |
| Mobility information | 2 | Used to indicate MS's mobility level 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved | May be present if a system supports Mobility information |
| } |  |  |  |
| If (De-registration_Request_Code == 0x04) { |  |  |  |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain AMS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain AMS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain AMS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain AMS service and operational information associated with network address. Bit 4: Retain AMS state information. The information retained by setting bit 4 includes configuration of all Service Flows in the AMS as set by successful AAI_DSA and AAI_DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information) |  |
| } |  |  |  |

[Remedy 2: Replace the Table 705 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 4 | Used to indicate the purpose of this message 0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS 0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-RSP message with action code 0x02 or 0x03 is received. 0x02: AMS shall listen to the current ABS but only transmit on the control connection. 0x03: AMS shall return to normal operation and may transmit on any of its active connections. 0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS. 0x05: AMS shall begin idle mode initiation: a) to signal AMS to begin idle mode in unsolicited manner or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x07: This option is valid in response to a AAI_DREG-REQ message with De-registration-request-code= 0x01 to allow AMS-initiated idle mode request. 0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS's connection information 0x09: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS's connection information. 0x10-0x15: reserved |  |
| If (Action code == 0x05) { |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Paging offset | 12 | Used to indicate Paging offset for the AMS. Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. |  |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS's idle mode information 0..248-1 |  |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..216-1 |  |
| Deregistration ID | 12 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..212-1 | Present when the Network Configuration indicates ABS is attached to the advanced network |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain AMS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain AMS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain AMS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain AMS service and operational information associated with network address. Bit 4: Retain AMS state information. The information retained by setting bit 4 includes configuration of all Service Flows in the AMS as set by successful AAI_DSA and AAI_DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information) |  |
| REQ-Duration | 8 | Used to indicate waiting value for the AAI_DREG-REQ message with De-registration_Request_Code=0x01 0..28-1: measured in frames | May be present |
| } |  |  |  |
| If (Action code == 0x07) { |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Paging offset | 12 | Used to indicate Paging offset for the AMS. Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. |  |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS's idle mode information 0..248-1 |  |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..216-1 |  |
| Deregistration ID | 12 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..212-1 | Present when the Network Configuration indicates ABS is attached to the advanced network |
| Idle Mode Retain Information element | 5 | Provided as part of this message indicative only. Network reentry from idle mode process requirements may change at time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry control messages shall not be retained and managed; a value of 1 indicates the information for the associated reentry control message shall be retained and managed. Bit 0: Retain MS service and operational information associated with AAI_SBC-REQ/RSP messages. Bit 1: Retain MS service and operational information associated with AAI_PKM-REQ/RSP messages. Bit 2: Retain MS service and operational information associated with AAI_REG-REQ/RSP messages. Bit 3: Retain MS service and operational information associated with network address. Bit 4: Retain MS state information. The information retained by setting bit 4 includes configuration of all Service Flows in the AMS as set by successful AAI_DSA and AAI_DSC transactions. In particular it includes FIDs and related description (QoS descriptors and CS classifier information) |  |
| } |  |  |  |

[Remedy 3: Replace the Table 706 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI_PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID_Info message. 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | L equals the Num_PGIDs in PGID_Info message |
| For (i=0; i<M; i++) { |  |  | M equals the number of bits in Paging_Group_IDs bitmap whose bit is set to 1. |
| Num_AMSs | 5 | Indicate the number of paged AMSs in a corresponding paging group 1..32 |  |
| For (j=0; j<Num_AMSs; j++) { |  |  |  |
| Deregistration Identifier | 12 | Used to indicate Deregistration ID for the MS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..212-1 | Present if the Network Configuration indicates ABS is attached to the advanced network |
| MAC Address Hash | 24 | used to identify the MS to be paged | Present if the Network Configuration indicates ABS is attached to the legacy network |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Action code | 1 | Used to indicate the purpose of the AAI_PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI_PAG-ADV message exists 0b0: this is the last fragment of the AAI_PAG-ADV message 0b1: this is not the last fragment of the AAI_PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: there is no emergency information 0b1: there is emergency information |  |

[Remedy 4: Replace the Table 707 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Num_PGIDs | 2 | Indicate the number of PGID included in PGID_Info message 1..4 |  |
| PG_uniformity flag | 1 | 0: Otherwise 1: All PGs have same paging indication bitmap | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups |
| For(i=0; i<Num_PGIDs; i++) { |  |  |  |
| PGID | 16 | Indicate Paging group identifier where the ABS belongs 0..216-1 |  |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame number of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation Npaging frame = AMS's deregistration identifier mod m to determine a predefined paging frame for the AMS 1..4 |  |
| Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap (L) equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. The paging carriers indicated in the PGID_Info are chosen among the carriers on the same frequency band where the PGID_Info message is received. | Present only for when an ABS supports multiple carrier operation and when PG_uniformity flag=0 |
| } |  |  |  |
| Paging carrier indication bitmap | L | This bitmap is used to specify if carriers are a paging carrier in the ABS or not. The size of paging carrier indication bitmap(L) equals to the number of carriers in the AAI_Global_Config message. The physical carrier index of each bit in bitmap corresponds to the ascending order of physical carrier index of the carriers in the AAI_Global_Config message. The value N used to determine the paging carrier index is equal to the number of bits set to 1. The paging carriers indicated in this PGID_Info are chosen among the carriers on the same frequency band where the PGID_Info message is received. | Present only when an ABS supports multiple carrier operation and belongs to multiple paging groups with PG_uniformity flag=1 |

-------------------------------  Text End  ---------------------------------------------------