| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Table Format for SingleBS_MIMO_FBK MAC Control Message (16.2.3.37) |  |
| Date Submitted | 2010-08-14 |  |
| Source(s) | Taeyoung Kim, Hyunjeong Kang, Jaehyuk Jang Samsung Electronics | ty33.kim@samsung.com |
| Re: | SB recirc #1 on 16m/D7 |  |
| Abstract | Suggested Table Format for 802.16m MAC Control Message |  |
| Purpose | For TGm discussion and adoption into P802.16m/D8 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested Table Format for SingleBS_MIMO_FBK MAC Control Message

Taeyoung Kim, Hyunjeong Kang, Jaehyuk Jang

Samsung Electronics

- Introduction

This contribution suggests alternative table format for AAI_SingleBS_MIMO_FBK MAC control message.

- Text change

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Remedy 1: Replace the Table 721 with the followings:]

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| If ( ((MFM==3) or (MFM==4) or (MFM==6) or (MFM==7)) and (q>0) ){ |  | MFM and long period q are indicated in Feedback Polling A-MAP IEs relevant to currently assigned feedback processes. |  |
| For (i=1; i <= Nt; i++){ |  |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in section 16.3.6.2.5.5.1 |  |
| For (j=i+1; j <= Nt; j++){ |  |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in section 16.3.6.2.5.5.1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MFM_bitmap | 8 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 1 LSB #2: MFM 2 LSB #3: MFM 3 LSB #4: MFM 4 LSB #5: MFM 5 LSB #6: MFM 6 LSB #7: MFM 7 |  |
| If (LSB #0 in MFM_bitmap == 1){ |  | MFM 0 |  |
| STC_Rate | 1-2 | Wideband STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits | Shall be omitted if MaxMt == 1 |
| Wideband CQI | 4 | Wideband CQI |  |
| } |  |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | MFM 1 |  |
| Wideband CQI | 4 | Wideband CQI |  |
| } |  |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | MFM 2 |  |
| Best_subbands_index | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| STC_Rate | 1-3 | STC rate If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt == 1. |
| For (m=0; m<Num_best_subbands; m ++){ |  | The subbands CQIs are sorted in order of increasing logical index |  |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |  |
| } |  |  |  |
| } |  |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | MFM 3 |  |
| Best_subbands_index | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| STC_Rate | 1-3 | STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits | Shall be omitted if MaxMt == 1 |
| For (m=0; m<Num_best_subbands; m ++){ |  | The subbands CQIs are sorted in order of increasing logical index |  |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |  |
| Subband PMI | 3-6 | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits The subbands PMIs are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. |  |
| } |  |  |  |
| } |  |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | MFM 4 |  |
| STC_Rate | 1-2 | Wideband STC rate MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits | Shall be omitted if MaxMt == 1 |
| Wideband CQI | 4 | Wideband CQI |  |
| Wideband PMI | 3-6 | Wideband PMI Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits | Presents when q == 0 |
| } |  |  |  |
| If (LSB #5 in MFM_bitmap == 1){ |  | MFM 5 |  |
| Best_subbands_index | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| For (m=0; m<Num_best_subbands; m ++){ |  | The subbands CQIs are sorted in order of increasing logical index |  |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |  |
| Stream index | 1-2 | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit The subbands Best stream indexes are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. |  |
| } |  |  |  |
| } |  |  |  |
| If (LSB #6 in MFM_bitmap == 1){ |  | MFM 6 |  |
| Best_subbands_index | 0-19 | Best subband index 512 FFT(fiveM): 2 bit 1024 FFT(tenM): 4 bit for best 1, 8 bit for best 6 2048 FFT(twentyM): 5 bit for best 1, 16 bit for best 6, 19 bit for best 12 | This field shall be omitted for full feedback (full) |
| For (m=0; m<Num_best_subbands; m ++){ |  | The subbands CQIs are sorted in order of increasing logical index |  |
| Subband CQI | 4 | MCS of m-th subband indicated by Best_subbands_index |  |
| Subband PMI | 3-6 | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits The subbands PMIs are sorted in order of increasing logical index. PMI of subbands indicated by BestSubbands. |  |
| } |  |  |  |
| } |  |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | MFM 7 |  |
| Wideband CQI | 4 | Wideband CQI |  |
| Wideband PMI | 3-6 | Wideband PMI Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits | Presents when q == 0 |
| } |  |  |  |

---------------------------------------------------------End of the Text-----------------------------------------------------------