| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Group Management Enhancement in 16m |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Bin Chen Guanghui Fan Junxian Mo Yanhong Wang Huawei Technologies | E-mail: binchen@huawei.com |
| Re: | Call for SB on “ P802.16m/D7”: |  |
| Abstract | This contribution analyses the tech issues of group management in D7 specification, and then propose the comments to revise the text in the D6 specification to fix these issues. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Group Management in 16m

Bin Chen, Guanghui Fan, Junxian Mo, Yanhong Wang

Huawei Technologies

- Introduction

In the specification of D7 [1], there are 2 main technical issues in the MAC message of AAI_GRP-CFG.

- It can just add or delete one FID by one message. This will need several MAC message when it is needed to add more than one FIDs to a group or to several groups, especially when need to change a FID to another group (one to delete and another one to add), or add a FID to UL group and another FID to DL group, 2 AAI_GRP-CFG messages are needed.

- A FID can be added to just one group. This is a limitation to the function of group resource allocation, especially when the service is PoC or Gaming whose traffics are small and not so regular.

- References

[1] IEEE P802.16m/D7

- Proposed text change in the P802.16m/D7

------------------------------------------ Begin Proposed Text Change 1-------------------------------------------

[Editor’s Note 1 : change the text in page 214, from line 34 to 55, Table 750—AAI_GRP-CFG]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Message type | 8 |  |  |
| M | Number of FID to be handled (N) | 3 | Number of FID will be handled in this message. |  |
| M | for (j=0; j<N; j++) { |  |  |  |
| M | Deletion Flag | 1 | Flag to signal whether this message includes addition or deletion information. 0: Flow is added to a group 1: Flow is explicitly deleted from a group |  |
| M | dlUlIndicato | 1 | 0: DL allocation 1: UL allocation |  |
| M | FID | 4 | jth FID that is added to or deleted from GRA group |  |
| … | …. | … |  |  |
| M | Group ID | 12 | ID of the group to which the flow is to be added or deleted |  |
| O | GRA Info for added flow |  |  |  |
| O | B) Long TTI Indicator | 1 | Defines number of AAI subframes spanned by the allocated resource. 0: 1 AAI subframe (default) 1: 4 DL/UL AAI subframes for FDD or all DL/UL AAI subframe for TDD | Present if Deletion Flag == 0 |
| …. | …. | …. |  |  |
|  | } |  |  |  |

------------------------------------------ End Proposed Text Change 1-------------------------------------------