| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A security issue in 16m |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Bin Chen George Cummings Huawei Technologies, Ltd | E-mail: binchen@huawei.com gcummings@huawei.com |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document shows suggested changes to P802.16m Draft D7 related to fix a security issue in 16m D7. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A Security Issue in 16m D7

Bin Chen, George Cummings

Huawei Technologies Co., Ltd

# Overview

Current 16m radio resource allocation mechanism includes physical resource permutation to logical resource unit, and A-MAP resource allocation. The permutation is defined in the standard text and known by all the terminals, while A-MAP is broadcast in a all known place with pre-defined MCS. Every AMS in the cell can detect and decode all the A-MAP, no matter whether it is the intended one. It just uses a STID to mask the CRC to check whether the A-MAPs are for itself.

The security problem is that a cracker can decode all the A-MAPs, and try all the STID to calculate the CRC. Since the 16 bits CRC is masked with 12 bits STID plus 4 fixed bits, the error detection rate will be very low for current 16 bits CRC algorithm. So that the cracker can receive all MAC frames and send them to background high quality calculating system to save and decrypt.

In this contribution, we would like to provide a simple method to make the A-MAP save with no additional overhead. The main idea is to set a mask ID to mask the resource index in the A-MAP IE before CRC calculating.

# Proposed changes

## Change 1

On page 100, line 37, subclause 16.2.3.8:

-------------------------------  Start of text proposal #1 --------------------------------------------

Table 687—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bit) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | STID | 12 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. | Present if needed |
| M | MAPMaskID | 11 | It is used in ABS to mask the resource index in the A-MAP before CRC calculating, and used in the AMS to re-mask the resource index in the A-MAP after the CRC calculating. |  |
| O | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
|  | …. |  |  |  |
| O | AMS initiated aGP Service Adaptation Capability: | 1 | AMS initiated aGP Service Adaptation Capability: | Present if neede |

-------------------------------  End of text proposal #1 --------------------------------------------

## Change 2

Text Proposal #2 < add below text in blue to the section 16.3.6.5.2.4.1 in the page 571 line 1>

-------------------------------  Start of text proposal #2 -------------------------------------------

The IE “Resource Index” shall be masked with the parameter “MAPMaskID” in the ABS before A-MAP’s CRC calculating, and shall be re-masked with the parameter “MAPMaskID” in the AMS after A-MAP’s CRC calculating. The mask processing in the ABS is: Sending IE “Resource Index” = Real Resource Index XOR MAPMaskID; the mask processing in the AMS is: Real Resource Index = Received IE “Resource Index” XOR MAPMaskID. The parameter MAPMaskID is configured by the ABS to the AMS via MAC signaling AAI_REG-RSP

-------------------------------  End of text proposal #2 --------------------------------------------