| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Table Format for AAI_CM-CMD/IND MAC control message (16.2.3) |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Hyunjeong Kang, Jaeweon Cho Samsung Electronics I-Kang Fu, Kelvin Chou MediaTek Inc. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #1 on P802.16m/D7 |  |
| Abstract | The contribution proposes to clean up MAC control message table format for AAI_CM-CMD and AAI_CM-IND messages. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Suggested Table Format for AAI_CM-CMD/IND MAC control message

Hyunjeong Kang, Jaeweon Cho

Samsung Electronics

I-Kang Fu, Kelvin Chou

MediaTek Inc.

- Introduction

Current MAC control message table in 16m/D7 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for AAI_CM-CMD and AAI_CM-IND MAC control messages.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Replace the Table 756 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 1 | Used to indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | Mandatory |
| If (Action code == 0b0) { |  |  |  |
| Indication Type | 2 | Indicate the corresponding secondary carrier is activated or deactivated 00: deactivation only 01: activation only 10: both activation and deactivation 11: reserved | Shall be present when Action code is set to 0b0 |
| If (Indication Type == 00){ |  |  |  |
| Num of target carrier | 3 | Indicate the number of secondary carrier(s) to be deactivated 1..8 | Shall be present when Indication Type is set to 00 |
| For(i=0;i<Num of target carrier; i++) { |  |  |  |
| Target carrier index | 6 | Indicate the secondary carrier index to be deactivated 0 ... 63 | Shall be present when Indication Type is set to 00 |
| Deactivation of DL/UL | 1 | Indicate the deactivation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are deactivated 0b1: UL of the secondary carrier is deactivated but DL of the secondary carrier is kept active | Shall be present when Indication Type is set to 00 |
| } |  |  |  |
| } |  |  |  |
| If (Indication Type == 01){ |  |  |  |
| Activation Deadline | 6 | LSB bits of frame number after the AAI_CM-CMD is sent for the AMS to confirm the activation of secondary carrier by sending the AAI_CM-IND message | Shall be present when Indication Type is set to 01 |
| Num of target carrier | 3 | Indicate the number of secondary carrier(s) to be activated 1..8 | Shall be present when Indication Type is set to 01 |
| For(i=0;i<Num of target carrier; i++) { |  |  |  |
| Target carrier index | 6 | Indicate the secondary carrier index to be activated 0 ... 63 | Shall be present when Indication Type is set to 01 |
| Activation of DL/UL | 1 | Indicate the activation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are activated 0b1: DL of the secondary carrier is activated but UL of the secondary carrier is not activated | Shall be present when Indication Type is set to 01 |
| Ranging indicator | 1 | Indicate the periodic ranging is required for the carrier 0b0: no initial or periodic ranging is required for the carrier 0b1: periodic ranging is required for the carrier | Shall be present when Indication Type is set to 01 |
| } |  |  |  |
| } |  |  |  |
| If (Indication Type == 10){ |  |  |  |
| Activation Deadline | 6 | LSB bits of frame number after the AAI_CM-CMD is sent for the AMS to confirm the activation of secondary carrier by sending the AAI_CM-IND message | Shall be present when Indication Type is set to 10 |
| Num of activated target carrier | 3 | Indicate the number of secondary carrier(s) to be activated 1..8 | Shall be present when Indication Type is set to 10 |
| Num of deactivated target carrier | 3 | Indicate the number of secondary carrier(s) to be deactivated 1..8 | Shall be present when Indication Type is set to 10 |
| For(i=0;i<Num of activated target carrier; i++) { |  |  |  |
| Target carrier index | 6 | Indicate the secondary carrier index to be activated 0 ... 63 | Shall be present when Indication Type is set to 10 and Num of activated target carrier larger than zero |
| Activation of DL/UL | 1 | Indicate the activation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are activated 0b1: DL of the secondary carrier is activated but UL of the secondary carrier is not activated | Shall be present when Indication Type is set to 10 and Num of activated target carrier larger than zero |
| Ranging indicator | 1 | Indicate the periodic ranging is required for the carrier 0b0: no initial or periodic ranging is required for the carrier 0b1: periodic ranging is required for the carrier | Shall be present when Indication Type is set to 10 and Num of activated target carrier larger than zero |
| } |  |  |  |
| For(i=0;i<Num of deactivated target carrier; i++) { |  |  |  |
| Target carrier index | 6 | Indicate the secondary carrier index to be deactivated 0 ... 63 | Shall be present when Indication Type is set to 10 and Num of deactivated target carrier larger than zero |
| Deactivation of DL/UL | 1 | Indicate the deactivation of DL or UL in the corresponding secondary carrier 0b0: Both DL/UL of the secondary carrier are deactivated 0b1: UL of the secondary carrier is deactivated but DL of the secondary carrier is kept active | Shall be present when Indication Type is set to 10 and Num of deactivated target carrier larger than zero |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| If (Action code == 0b1) { |  |  |  |
| Physical carrier index of Target carrier | 6 | Indicate the physical carrier index of a target carrier for primary carrier change. If the AMS supports multicarrier operation, the carrier shall be one of the assigned carriers 0..63 | Shall be presented if Action code equal to 0b1 |
| Ranging indicator | 1 | Indicate the periodic ranging is required for the carrier 0b0: no initial or periodic ranging is required for the carrier 0b1: periodic ranging is required for the carrier | Shall be presented if Action code equal to 0b1 |
| Action Time | 3 | LSB bits of frame number at the time to switch to the target carrier The value shall be set to the value more than the Message ACK timer for AAI_CM-CMD message 0..7: LSB of frame | Shall be presented if Action code equal to 0b1 |
| Next state of serving primary carrier | 1 | Indicate the deactivation of serving primary carrier after primary carrier change. 0b0: serving primary carrier will be deactivated after primary carrier change. If the AMS does not support carrier aggregation, this field shall be always set to 0b0. 0b1: serving primary carrier is kept active after primary carrier change | Shall be presented if Action code equal to 0b1 |
| } |  |  |  |

[Remedy 2: Replace the Table 757 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 1 | Used to indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | Mandatory |

-------------------------------  Text End  ---------------------------------------------------