| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DID Clean-up (16.2.3.22) |  |
| Date Submitted | 2010-08-18 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yung-Han Chen, Yu-Min Chiu ITRI | Voice : + 886 3-5914629 E-mail: mhtao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.3 (MAC Control messages) |  |
| Abstract | The contribution cleans-up the DID system. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DID Clean-Up (16.2.3.22)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

This contribution adds one bit in the AAI_PAG-ADV message to indicate which identification system (DID or MAC address hash) is used in the AAI_PAG-ADV to identify the idle mode AMS.

- Proposed Text for IEEE P802.16m/D7

[Remedy 1: Modify Table 706 from line 53 of page 142 as follows.]

Table 706—AAI_PAG-ADV message

| M/O | Attributes / Array of Attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Paging Identifier Indication |  | 1 | 0b0: DID and paging cycle are used to identify the paged AMS. 0b1: MAC Address Hash is used to identify the paged AMS. | N.A. |
| M | Paging_Group_IDs bitmap |  | - | Indicate that the paging information for the corresponding paging group is included in AAI_PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID_Info message. 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | N.A. |
| O | Num_AMSs |  | 5 | Indicate the number of paged MSs in a corresponding paging group 1..32 | Shall be present when Paging_Group_IDs bitmap of a corresponding PGID is set to 1 |
| O | Paged MS Info[1..32] | Deregistration Identifier | 12 | Used to indicate Deregistration ID for the MS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0…212-1 | Shall be present if the Network Configuration indicates ABS is attached to the advanced network Paging Identifier Indication is set to 0 and unless Num_AMSs is 0 |
| O | Paged MS Info[1..32] | MAC Address Hash | 24 | used to identify the MS to be paged. | Shall be present if the Network Configuration indicates ABS is attached to the legacy network Paging Identifier Indication is set to 1 and unless Num_AMSs is 0 |
| MO | Paged MS Info[1..32] | Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved | Shall be present if Paging Identifier Indication is set to 0 and unless Num_AMSs is 0 |
| M | Paged MS Info[1..32] | Action Code | 1 | Used to indicate the purpose of the AAI_PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update | Shall be present unless Num_AMSs is 0 |
| M | Extension Flag |  | 1 | Used to indicate the remaining part of the AAI_PAG-ADV message exists 0b0: this is the last fragment of the AAI_PAG-ADV message 0b1: this is not the last fragment of the AAI_PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames. | N.A. |
| M | Emergency Alert Indication |  | 1 | Used to indicate the presence of emergency information 0b0: there is no emergency information 0b1: there is emergency information | N.A. |

============================== End of Proposed Text ====================================

- References

- IEEE P802.16m/D7. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, July 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.