| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 code of MAC Control Messages |  |
| Date Submitted | 2010-09-10 |  |
| Source(s) | Joey Chou Intel Alessandro Triglia, Bancroft Scott OSS Nakalva Scott Probasco, Nokia | E-mail: joey.chou@intel.com sandro@oss.com baos@oss.com scott.probasco@nokia.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to relocate ASN.1 code of MAC Control Messages |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- ASN.1 code of MAC Control Messages

- Joey Chou,

- Intel

- Alessandro Triglia,

- Bancroft Scott

- OSS Nakalva

- Scott Probasco,

- Nokia

- Introduction

Section 16.2.3 provides detailed descriptions and tables of MAC control messages. But, the ASN.1 representation of these messages is listed in Annex R.2 that makes it very difficult to review, and to align both tables and ASN.1 code.

Therefore, this contribution proposes:

- Add a new subsection 16.2.3.x to group MAC control messages into functional areas that are defined in Table 678. MAC control messages are then located in subsections 16.2.3.x.1, …

- Move MAC control message structure that maps to MAC messages in Table 678 to section 16.2.3.

- Move common type definitions to be used by all MAC control messages to section 16.2.3

- Move type definitions specific to each functional area to section 16.2.3.x

- Move ASN.1 code of each message to section 16.2.3.x.n right after the message tables.

When all ASN.1 code in Annex R.2 is moved to section 16.2.3, 802.16m can release a separate ASN.1 ASCII file along with the 802.16m draft that can be used for MAC message implementation.

This contribution relocates ASN.1 code of MAC control messages.

- Proposed text

II.1 Proposed text 1

16.2.3 MAC Control messages

------------------------------------------------- Start of proposed text I --------------------------------------------------

[Editor Note: at Page 82, Line 56, add text as shown in the following, right after Table 678:]

The following lists the ASN.1 representation of MAC control messages as defined in Table 678.

MAC-Control-Message ::= SEQUENCE {

message               MAC-Control-Msg-Type,

...

}

MAC-Control-Msg-Type ::= CHOICE {

-- System information

aaiSCD                AAI-SCD,

aaiSIIAdv             AAI-SII-ADV,

aaiULPCNi             AAI-ULPC-NI,

-- Network entry / re-entry

aaiRngReq             AAI-RNG-REQ,

aaiRngRsp             AAI-RNG-RSP,

aaiRngAck             AAI-RNG-ACK,

aaiRngCfm             AAI-RNG-CFM,

aaiRegReq             AAI-REG-REQ,

aaiRegRsp             AAI-REG-RSP,

aaiSbcReq             AAI-SBC-REQ,

aaiSbcRsp             AAI-SBC-RSP,

-- Network exit

aaiDregReq            AAI-DREG-REQ,

aaiDregRsp            AAI-DREG-RSP,

-- Connection management

aaiDsaReq             AAI-DSA-REQ,

aaiDsaRsp             AAI-DSA-RSP,

aaiDsaAck             AAI-DSA-ACK,

aaiDscReq             AAI-DSC-REQ,

aaiDscRsp             AAI-DSC-RSP,

aaiDscAck             AAI-DSC-ACK,

aaiDsdReq             AAI-DSD-REQ,

aaiDsdRsp             AAI-DSD-RSP,

aaiGrpCfg             AAI-GRP-CFG,

-- Security

aaiPkmReq             AAI-PKM-REQ,

aaiPkmRsp             AAI-PKM-RSP,

-- ARQ

aaiArqFbk             AAI-ARQ-FBK,

aaiArqDsc             AAI-ARQ-DSC,

aaiArqRst             AAI-ARQ-RST,

-- Sleep mode

aaiSlpReq             AAI-SLP-REQ,

aaiSlpRsp             AAI-SLP-RSP,

aaiTrfInd             AAI-TRF-IND,

aaiTrfIndReq          AAI-TRF-IND-REQ,

aaiTrfIndRsp          AAI-TRF-IND-RSP,

-- Handover

aaiHoReq              AAI-HO-REQ,

aaiHoCmd              AAI-HO-CMD,

aaiHoInd              AAI-HO-IND,

aaiNbrAdv             AAI-NBR-ADV,

aaiNbrReq             AAI-NBR-REQ,

aaiScnReq             AAI-SCN-REQ,

aaiScnRsp             AAI-SCN-RSP,

aaiScnRep             AAI-SCN-REP,

-- Idle mode

aaiPagAdv             AAI-PAG-ADV,

aaiPgidInfo           AAI-PGID-INFO,

-- Multicarrier

aaiMcAdv              AAI-MC-ADV,

aaiMcReq              AAI-MC-REQ,

aaiMcRsp              AAI-MC-RSP,

aaiCmCmd              AAI-CM-CMD,

aaiCmInd              AAI-CM-IND,

aaiGlobalConfig       AAI-GLOBAL-CFG,

-- Power Control

aaiUlPowerAdj         AAI-UL-POWER-ADJ,

aaiUlPsrConfig        AAI-UL-PSR-CFG,

-- Collocated Coexistence

aaiClcReq             AAI-CLC-REQ,

aaiClcRsp             AAI-CLC-RSP,

-- MIMO

aaiSbsMimoFbk         AAI-SBS-MIMO-FBK,

aaiMbsMimoFbk         AAI-MBS-MIMO-FBK,

aaiDlIm               AAI-DL-IM,

aaiMbsMimoReq         AAI-MBS-MIMO-REQ,

aaiMbsMimoRsp         AAI-MBS-MIMO-RSP,

aaiMbsMimoSbp         AAI-MBS-MIMO-SBP,

aaiMbsSoundingCal     AAI-MBS-SOUNDING-CAL,

-- FFR

aaiFfrCmd             AAI-FFR-CMD,

aaiFfrRep             AAI-FFR-REP,

-- SON

aaiSonAdv             AAI-SON-ADV,

-- Relay

aaiARSCfgCmd          AAI-ARS-CFG-CMD,

-- EMBS

aaiEmbsCfg            AAI-EMBS-CFG,

-- LBS

aaiLbsAdv             AAI-LBS-ADV,

aaiLbsInd             AAI-LBS-IND,

-- Misc

aaiL2Xfer             AAI-L2-XFER,

aaiMsgAck             AAI-MSG-ACK,

aaiResCmd             AAI-RES-CMD}

The follwing lists the common type definitions that are used by MAC control messages.

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Common type definitions

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

PhyCarrierIndex    ::= INTEGER (0..63)

LogCarrierIndex    ::= INTEGER (0..7)

PreambleIndex      ::= INTEGER (0..1023)

PreambleIndex2     ::= INTEGER (0..63)

PreambleIndex3     ::= INTEGER (0..127)

PCID               ::= BIT STRING (SIZE(48))

PGID               ::= BIT STRING (SIZE(16))

DREGID             ::= BIT STRING (SIZE(12))

PgCycle            ::= INTEGER (0..15)

PgOffset           ::= INTEGER (0..63)

PgOffset1          ::= INTEGER (0..4095)

FidChangeCount     ::= INTEGER (0..15)

AbsIndex           ::= INTEGER (0..255)

MBSZoneID          ::= INTEGER (0..127)

EMBSZoneID         ::= MBSZoneID

MacProtocolVersion ::= INTEGER (0..255)

BSID               ::= BIT STRING (SIZE(48))

STID               ::= BIT STRING (SIZE(12))

CID                ::= BIT STRING (SIZE(16))

FID                ::= INTEGER (0..15)

ReportMetric ::= BIT STRING {

absCINRMean (0),

absRSSIMean (1),

relativeDelay (2),

absRTD (3)} (SIZE(4))

RedirectionInfo ::= SEQUENCE {

absid                          BSID,

preambleIndex                  PreambleIndex,

centerFrequency                INTEGER(0..4294967295)}

------------------------------------------------- End of proposed text I --------------------------------------------------