| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-ARQ-Feedback, AAI-ARQ-Discard, AAI-ARQ-Reset (16.2.3) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Anil Agiwal, Youngbin Chang Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Modification of the tables for ARQ MAC messages |  |
| Purpose | To be discussed and adopted in D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-ARQ-Feedback, AAI-ARQ-Discard, AAI-ARQ-Reset

Anil Agiwal, Youngbin Chang

Samsung Electronics

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for ARQ MAC control messages.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 202, replace Table 734 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 734—AAI-ARQ-Feedback message format

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ARQ feedback IE { | variable | One or more ARQ feedback IEs may be present in the AAI-ARQ-Feedback message | Maximum number of ARQ feedback IEs is equal to maximum number of transport FIDs |
| FID | 4 | The FID of the connection corresponding to this ARQ feedback IE |  |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |  |
| SN | 10 | Indicates the sequence number of an ARQ block. FLAG = 0, indicates ARQ blocks up to and including the sequence number in the SN field have been received successfully. FLAG = 1, indicates ARQ blocks less than the sequence number in the SN field have been received successfully. It also indicates the sequence number of ARQ block whose ACK or NAK information is indicated by the MSB of Selective ACK MAP. |  |
| if(FLAG==1){ |  |  |  |
| NSI | 1 | NACK Suspended Indicator 0 = Bit marked '0' in the following Selective ACK MAP represents a ARQ block NACK 1 = Bit marked '0' in the following Selective ACK MAP represents that ARQ block NACK decision is suspended (i.e., ARQ_ERROR_DETECTION_TIMER is running for corresponding ARQ block). |  |
| EXT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |  |
| Selective ACK MAP | variable | Each bit represents ACK or NACK or NACK Suspended of corresponding ARQ block. '0' is NACK if NSI equals zero '0' is NACK suspended if NSI equals one '1' is ACK. MSB of the First Selective ACK MAP represents ACK or NAK information of an ARQ block identified by the sequence numbers in the SN field. Contiguous bits after MSB of the First Selective ACK MAP represents ACK or NACK information of contiguous ARQ blocks following the ARQ block identified by the sequence number in the SN field. | Maximum size of Selective ACK MAP is equal to size of ARQ_WINODW_SIZE |
| if(EXT==1){ |  |  |  |
| SEM | L | Sub-block-Existence Map Sub-block-Existence Map The numbers of bits in Sub-block-Existence Map is equal to the number of bits that indicates the NACKed ARQ blocks in the Selective ACK MAPs. Each bit represents the existence of partially NACKed ARQ blocks or completely NACKed ARQ blocks. The most significant bit corresponds to the first NACKed ARQ block in the Selective ACK MAP and the least significant bit corresponds to the last NACKed ARQ block in the Selective ACK MAP. 1 = Partially NACKed ARQ block (the ARQ block has some sub-blocks which have received correctly) 0 = Completely NACKed ARQ blocks( the ARQ block has no sub-blocks which have received correctly) | L is equal to the number of bits that indicates the NACKed ARQ blocks in the Selective ACK MAPs |
| For(i=0;i<Num_SN; i++){ |  |  | Num_SN is the number of bits equal to ‘1’ in SEM field |
| For(k=0;k<Num_Sub_Block_Info;ki++){ |  |  |  |
| SubBlockACKInfo { | variable | For each ‘i’, One or more SubBlockACKInfo can be present. If NSI bit set to '0', all ARQ sub-blocks that are not listed in the Start_SSN and Num_SSN in all the SubBlockACKInfos are considered as NACK. If NSI bit set to '1', for all the ARQ sub-blocks that are not listed in the Start_SSN and Num_SSN in all the SubBlockACKInfos, it is considered that NACK decision is suspended. | The Maximum number of SubBlockACKInfo is equal to 1024 |
| START_SSN | 11 | Start of ARQ sub-block SN which was received correctly. |  |
| NUM_SSN | 11 | Number of consecutive ARQ sub-blocks which were received correctly from START_SSN onwards. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 203, replace Table 735 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 735—AAI-ARQ-Discard message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| FID | 4 | Corresponding connection ID to perform ARQ block discard. |  |
| SN | 10 | ARQ block SN of the last block in the transmission window that the transmitter wants to discard. |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D8, page 203, replace Table 736 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 736—AAI-ARQ-Reset message format

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| DL/UL indicator | 1 | 0b0 = Downlink 0b1 = Uplink |  |
| FID | 4 | Corresponding connection ID to perform ARQ reset procedure. |  |
| Type | 2 | 0b00 = Original message from Initiator 0b01 = Acknowledgement from Responder 0b10 = Confirmation from Initiator 0b11 = Reserved |  |
| If ( type == 0b00 ¦¦ type == 0b01) { |  |  |  |
| SN | 10 | ARQ transmitter sets this field to (ARQ_TX_WINDOW_START_SN + ARQ_WINDOW_SIZE) mod (ARQ_SN_MODULUS). ( see section 16.2.13.5.4 for details) | This field shall be included when the ARQ transmitter sends an ARQ reset message with type 0b00 or 0b01. This field is not included when the ARQ receiver sends an ARQ reset message. |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------