| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ASN.1 code for AAI-ARQ-RST |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Anil Agiwal, Youngbin Chang Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Corrected ASN.1 code for AAI-ARQ-RST |  |
| Purpose | To be discussed and adopted in D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ASN.1 Code for AAI-ARQ-RST

Anil Agiwal, Youngbin Chang

Samsung Electronics

Introduction

This contribution proposes correction to ASN.1 code for AAI-ARQ-RST message.

Proposed Text

[Replace the existing code lines 2-13 on page 952 with following]

AAI-ARQ-RST :: = SEQUENCE {

directionIndicator 		DirIndicator,

fid 						FID,

type 					ENUMERATED {

originalMessageFromInitiator,

acknowledgementFromResponder,

confirmationFromInitiator},

sequenceNumber 			ArqBlockSn  OPTIONAL,

-- ARQ transmitter sets this field to (ARQ_TX_WINDOW_START_SN + ARQ_WINDOW_SIZE) mod

-- (ARQ_SN_MODULUS).See section 16.2.13.5.4 for details

...

}