| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on Concatenating Precoders for DL Multi-BS Joint MIMO Processing (16.5.1) |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: pinghengkuo@itri.org.tw |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D8) |  |
| Abstract | This contribution proposes some clarifications on concatenating precoders for multi-BS downlink joint processing. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on Concatenating Precoders for

DL Multi-BS Joint MIMO Processing (16.5.1)

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

- For multi-BS downlink joint processing in the current draft [1], the AMS may select and feedback additional phase information to form a concatenating precoder for each neighboring ABS. Such mechanism can prevent the signal destructions at the receiver. However, the construction of concatenating precoder is not clear. Therefore, this contribution suggests adding text as well as an equation to delineate the operation in more details.

- Text proposal for inclusion in the 802.16m Draft 8

[Modify the following text in Section 16.5.1.3.1]

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.1.3.1 Operation procedure

With adaptive precoding, the precoder matrix W is derived from the feedback of the AMS, with codebook-based feedback or sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For codebook-based feedback, the AMS(s) choose the PMIs for the serving cell and the neighboring cells based on the respective estimated channel state information. The serving ABS should also instruct the AMS(s) to feedback a 3-bit uniformly quantized phase information for each neighboring cell, such that ABS can form a concatenating PMI based on the phase information for the by applying phase adjustments to PMIs for neighboring cells to further improve the system performance. The equation for phase quantization adjustment is, where b corresponds to a 3-bit CPMI value which is defined in Table 973:

Table 973—Quantization parameters for b

| CPMI Value | b |
| --- | --- |
| 0b000 | 0 |
| 0b001 | 1/8 |
| 0b010 | 2/8 |
| 0b011 | 3/8 |
| 0b100 | 4/8 |
| 0b101 | 5/8 |
| 0b110 | 6/8 |
| 0b111 | 7/8 |

The concatenating precoder for the i-th neighboring cell is constructed as

,                              (xxx)

where Wi is the precoding matrix corresponds to the PMI chosen from base codebook for the i-th neighboring cell, and bi is the CPMI value selected from Table 973.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, Aug 2010.