| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification on Secondary Advanced Preamble (16.3.5.1.2) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Chung-Pao Chen, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting ITRI Hsin-Piao Lin NTUT | Voice: E-mail: ythsieh@itri.org.tw E-mail : hplin@ntut.edu.tw |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes one modification for the secondary advanced preamble in Section 16.3.5.1.2 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification for Amendment Working Document on Secondary Advanced Preamble (16.3.5.1.2)

Tsung-Pao Chen, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting

- ITRI

- Hsin-Piao Lin

- NTUT

- Introduction

The text description for Figure 516 is ambiguous. In the text from Line 36 Page 559 of the AWD [1], the “block” is defined as 8 consecutive sub-blocks {E, F, G, H, A, B, C, D} as shown in Figure A.

For the case of 512-FFT in Figure 516, however, it seems that the “block” should be defined as 8 consecutive sub-blocks {A,B,C,D,E,F,G,H} instead of {E, F, G, H, A, B, C, D}. For clarification, the beginning of the block for 512-FFT size should be at the right hand side of the DC and then cyclic shift to the left hand side as shown in Figure B.

- Text Proposal

Black text: text in the subclause 16.3.5.1.2 of P802.16m/D8

Blue text: new text

---------------------------------------------------------Start of the Text--------------------------------------------------------

[Note to Editor – Modify the text in section 16.3.5.1.2 Secondary advanced preamble from line 36 of page 559 as follows]

Let “block” denote 8 consecutive sub-blocks {E, F, G, H, A, B, C, D}. For 512-FFT size, the beginning of the block should be at the right hand side of the DC and then cyclic shift to the left hand side. The algorithm to assign the preamble blocks to multiple transmit antennas where the tone dropping is not applied and the number of antennas is power of 2 can be described as follows.

---------------------------------------------------------End of the Text---------------------------------------------------------

- References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, Aug. 2010.