| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up for AAI-Global-CFG message (16.2.3) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: chickout2@itri.org.tw |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes renew text for relay support. |  |
| Purpose | Discussion and adoption by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up for AAI-Global-CFG messages (16.2.3)

Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting

ITRI

Introduction

Some texts are not consistent with AAI-Global-CFG messages after session 68.5. In this contribution, we provide text modifications to correct the errors.

Proposed text

Black text: text in the subclause 16.2.3 of P802.16m/D8

Red Text: delete text

Blue text: new text

---------------------------------------------------------Start of the Text #1--------------------------------------------------------

[Note to Editor – Modify the text from line #26 page 334 in section 16.2.8.1 Multicarrier Types and Operational Modes as follows]

In addition to the information about the primary carrier, an ABS can also provide AMSs with some configuration information about its available carriers through such primary carrier by AAI-Global-Config CFG and AAI-MC-ADV messages.

---------------------------------------------------------End of the Text #1---------------------------------------------------------

---------------------------------------------------------Start of the Text #2--------------------------------------------------------

[Note to Editor – Modify the text from line #1 page 421 in section 16.2.15.6 Registration as follows]

ABS shall unicast the AAI-Global-Config CFG message to the AMS. Also, if AMS's Global carrier configuration information is expired, the AMS shall not perform any procedure that refers to the AAI-Global-Config CFG message until the valid AAI-Global-Config CFG message is sent by the ABS.

If the AAI-Global-Config CFG message parameters are to be updated, the ABS shall unicast the updated AAI-Global-Config CFG message to each AMS's served by the ABS after the AMS and ABS complete switching between TSTID and STID. The ABS shall set Polling bit to 1 in MCEH of the AAI-Global-Config CFG message.

---------------------------------------------------------End of the Text #2---------------------------------------------------------

---------------------------------------------------------Start of the Text #3--------------------------------------------------------

[Note to Editor – Modify the text from line #25 page 495 in section 16.3.3.6.2 Subcarrier alignment for multicarrier operation as follows]

In Table 803, a lookup table is defined to indicate the configurations for a group of contiguous carriers. If the network supports multiple groups of contiguous carriers, the AAI-Global-Config CFG message will separately indicate each group of the contiguous carriers by referring to the index of this table. For example, the multicarrier configuration {5, 10} indicates two contiguous carriers are supported. The first one is a 5MHz carrier and another one is a 10MHz carrier, where the order in this configuration is sorted from lower frequency to higher frequency.

---------------------------------------------------------End of the Text #3---------------------------------------------------------

---------------------------------------------------------Start of the Text #4--------------------------------------------------------

[Note to Editor – Modify the text from line #43 page 81 in section 16.2.3 MAC Control messages as follows]

Table 678-MAC Control Messages

| … | … | … | … | … | … |
| --- | --- | --- | --- | --- | --- |
| 46 | Multicarrier | AAI-CM-IND | Carrier Management Indication | Encrypted/ICV | Unicast |
| 47 | Multicarrier | AAI-Global-Config CFG | Global Carrier Configuration | Encrypted/ICV | Unicast |
| 48 | Power Control | AAI-UL_POWER_ADJ | Uplink TX power adjustment | Null | Unicast |
| … | … | … | … | … | … |

---------------------------------------------------------End of the Text #4---------------------------------------------------------

---------------------------------------------------------Start of the Text #5--------------------------------------------------------

[Note to Editor – Modify the text from line #16 page 126 in section 16.2.3.13 AAI-NBR-ADV as follows]

Within each cell type, if S-ABS chooses to broadcast configuration information for each individual ABS instead of specifying SA-Preamble Index range and Physical carrier range, a list of ABSs are provided and the following parameters are carried for each ABS

•48-bit BS-ID

•ABS SA-Preamble Index

•indication whether full system information or partial information is carried for this ABS, which includes

—SFH information

—Physical carrier index (6 bits, refer to the "physical carrier index" defined in AAI-Global-Config CFG)

—MAC protocol versions (8 bits)

—Paging carrier indication (1 bit, refer to specify if a carrier is a paging carrier or not)

---------------------------------------------------------End of the Text #5---------------------------------------------------------

---------------------------------------------------------Start of the Text #6--------------------------------------------------------

[Note to Editor – Modify the text from line #57 page 127 in section 16.2.3.13 AAI-NBR-ADV as follows]

Table 692- AAI-NBR-ADV parameters

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| M | D.2) A-PREAMBLE transmit power | 8 | - |  |
| M | D.3) Physical carrier index | 6 | Refer to the physical carrier index in AAI-Global-Config CFG message |  |
| O | D.4) Paging carrier indication | 1 | Indicates whether the carrier is a paging carrier in ABS or not 0: no paging carrier 1: paging carrier | May be present when multiple carrier operation is applied. |
| … | … | … | … | … |

---------------------------------------------------------End of the Text #6---------------------------------------------------------

---------------------------------------------------------Start of the Text #7--------------------------------------------------------

[Note to Editor – Modify the text from line #20 page 129 in section 16.2.3.13 AAI-NBR-ADV as follows]

Table 692- AAI-NBR-ADV parameters

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| M | C) PHY mode ID | 16 | Specifies the PHY parameters of each neighbor R1 BS, including channel bandwidth, FFT size, cyclic prefix, and frame duration, defined in 11. 18. 2 |  |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI-Global-Config CFG message | For ABS type whose system info are not included in AAI-NBR-ADV |
| O | IDCell range start | 10 | For ABS type whose system info are not included in AAI-NBR-ADV |  |
| … | … | … | … | … |

---------------------------------------------------------End of the Text #7---------------------------------------------------------

---------------------------------------------------------Start of the Text #8--------------------------------------------------------

[Note to Editor – Modify the text from line #26 page 137 in section 16.2.3.15 AAI-SCN-RSP as follows]

Table 694- AAI-SCN-RSP parameters

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| O | C) carrierIndexArr[0..63] |  |  |  |
| O | C.1) Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-Config CFG message | Present if N_Recommended_Carrier_Index > 0 |
| O | AAI-NBR-ADV Segment Index | 4 | Indicates current segment index of AAI-NBR-ADV in the specific cell type used by the AMS | Present if N_Recommended_ABS_Index |
| … | … | … | … | … |

---------------------------------------------------------End of the Text #8---------------------------------------------------------

---------------------------------------------------------Start of the Text #9--------------------------------------------------------

[Note to Editor – Modify the text from line 40 page 138 in section 16.2.3.15 AAI-SCN-RSP as follows]

Table 694- AAI-SCN-RSP parameters

| … | … | … | … | … |
| --- | --- | --- | --- | --- |
| O | C) carrierIndexArr[0..63] |  |  |  |
| O | C.1) Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI-Global-Config CFG message | Present if N_Recommended_Carrier_Index > 0 |
| O | C.2) SA Preamble Index | Variable ( 10 x N) | SA Preamble Index to be scanned | Present if N_Recommended_Carr |
| … | … | … | … | … |

---------------------------------------------------------End of the Text #9---------------------------------------------------------

References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, Aug. 2010.