| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Modification for ranging of relay support (16.6.3) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Chia-Lung Tsai, Ping-Heng Kuo, Rong-Terng Juang, Yu-Tao Hsieh, Pang-An Ting ITRI Hsin-Piao Lin NTUT | E-mail: chickout2@itri.org.tw hplin@ntut.edu.tw |
| Re: | Comments on IEEE P802.16m/D8 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes renew text for relay support. |  |
| Purpose | Discussion and adoption by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Modification for ranging of relay support (16.6.3)

Chia-Lung Tsai, Ping-Heng Kuo, Rong-Terng Juang,

Yu-Tao Hsieh, Pang-An Ting

ITRI

Hsin-Piao Lin

NTUT

Introduction

Some concerns have been identified in the section 16.6.3.2.1. If the ranging channel is transmitted in the first UL AAI subframe, the UL frame structure of relay in the FDD system in [1] may be malfunctioned, as R-RTI is not zero. In this contribution, we suggest some modifications to solve the problem. In particular, we shift the location of R-RTI in the frame structure.

Proposed text

Black text: text in the subclause 16.6.3.2 of P802.16m/D8

Red Text: delete text

Blue text: new text

---------------------------------------------------------Start of the Text #1--------------------------------------------------------

[Note to Editor – Modify the text in section 16.6.3.2.1 FDD frame structure in page 879 as follows]

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between AAI Access zone in the previous UL ARS frame and AAI Relay zone and a R-TTI between AAI Relay zone and AAI Access zone. The location of the R-RTI is the first last OFDM symbol of the first last AAI subframe of AAI Relay zone Access zone and the location of R-TTI is the last OFDM symbol of the last AAI subframe of AAI Relay zone. The duration of R-RTI and R-TTI in UL for each ARS shall be calculated by following equations:

(338)

(339)

where the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in <<Table 802>>

Figure 602-illustrates an example frame structure with ARS support for FDD mode, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. All transition intervals R-TTI and R-RTI in the figure example are equal to the duration of one OFDMA symbol.

Figure 602-Example of ARS FDD frame structure with G = 1/8 in 5/10/20MHz

---------------------------------------------------------End of the Text #1---------------------------------------------------------

References

[1] IEEE P802.16m/D8, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, Aug. 2010.