| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-LBS-ADV and AAI-LBS-IND (16.2.3) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Lei Zhou, Xufeng Zheng，Hai Wang China Samsung Telecom R&D Center Sangheon Kim, Hyunjeong Kang, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Sponsor ballot recirc #2 on P802.16m/D8 |  |
| Abstract | The contribution proposes to clean up MAC control message table format for LBS messages (LBS-ADV, LBS-IND). |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-LBS-ADV and AAI-LBS-IND (16.2.3)

Lei Zhou, Xufeng Zheng, Hai wang

China Samsung Telecom R&D Center

Sangheon Kim, Hyunjeong Kang, Jaeweon Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for LBS MAC control messages. And this contribution only modifies bit size of change count for AAI-NBR-ADV in AAI-LBS-ADV in order to be consistent with message AAI-NBR-ADV in 16m/D8.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Replace the Table 757 from line# 39 Page 250 in P802.16m/D8 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Format Indication for the transmitting BS’s coordinates | 1 | Represent absolute position whether using long format or short format. The value indicates: 0x0: long format is used 0x1: short format is used |  |
| If (Format Indication for the transmitting BS’s coordinates == 0x0) { |  |  |  |
| Longitude | 40 | Bit 0-5: longitude resolution 1-34 - # of valid bits in fixed-point value of longitude value 35 – LBS not supported Other – Reserved Bit 6-14: longitude integer Bit 15-39: longitude fraction |  |
| Latitude | 40 | Bit 0-5: latitude resolution 1-34 - # of valid bits in fixed-point value of latitude value 35 – LBS not supported Other – Reserved Bit 6-14: latitude integer Bit 15-39: latitude fraction |  |
| Altitude | 40 | Bit 0-3: altitude type 1 – meter 2 – floor Other – Reserved Bit 4-9: altitude resolution 1-30 - # of valid bits in fixed-point value of altitude value 31 – LBS not supported Other – Reserved Bit 10-31: altitude integer Bit 32-39: altitude fraction |  |
| } |  |  |  |
| If (Format Indication for the transmitting BS’s coordinates == 0x1) { |  |  |  |
| Type indication of short format | 1 | 0x0: short format including altitude is used. 0x1: short format excluding altitude is used |  |
| Longitude | 24 | Longitude expressed in 2-15 parts of a degree |  |
| Latitude | 24 | Latitude expressed in 2-16 parts of a degree |  |
| If (Type indication of short format == 0x0) { |  |  |  |
| Altitude | 16 | Bit 0-15: altitude in meters above sea level |  |
| } |  |  |  |
| } |  |  |  |
| GPS time in units of frame duration | 22 | Refer to 11.21.4 |  |
| GPS frame transmission time offset | 10 |  |  |
| GPS time accuracy | 5 |  |  |
| Frequency Accuracy | 8 | Information about the frequency accuracy. It is included once in the message. Refer to 11.21.5 |  |
| Number of ABS | 8 | Number of neighbor BSs included in this message that are identified using the BSID. |  |
| For (i=0; i< Number of ABS; i ++){ |  |  |  |
| BSID | 16 | 16-bit LSB of unique 48-bit identifier of the Neighbor ABS for number of BS. |  |
| Type indication of relative position format | 1 | 0x0: Relative position format including altitude is used. 0x1: Relative position format excluding altitude is used |  |
| Longitude | 16 | Distance east of reference point in meters |  |
| Latitude | 16 | Distance north of reference point in meters |  |
| If (Type indication of relative position format == 0x0) { |  |  |  |
| Altitude | 16 | Bit 0-15: altitude in meters above sea level |  |
| } |  |  |  |
| } |  |  |  |
| Number of ABS Index | 8 | Number of neighbor ABSs included in this message that are identified using an index to their position in the AAI-NBR-ADV message |  |
| if( Number of ABS Index != 0 ){ |  |  |  |
| Configuration change count for AAI-NBR-ADV | 8 3 |  |  |
| } |  |  |  |
| For (i=0; i< Number of ABS Index; i ++){ |  |  |  |
| Type indication of relative position format | 1 | 0x0: Relative position format including altitude is used. 0x1: Relative position format excluding altitude is used |  |
| Neighbor ABS Index | 8 | Index that corresponds to the position of the ABS in the AAI-NBR-ADV message |  |
| Longitude | 16 | Distance east of reference point in meters |  |
| Latitude | 16 | Distance north of reference point in meters |  |
| If (Type indication of relative position format == 0x0) { |  |  |  |
| Altitude | 16 | Bit 0-15: altitude in meters above sea level |  |
| } |  |  |  |
| } |  |  |  |

[Remedy 2: Replace the Table 758 from line# 4 Page 253 in P802.16m/D8 with the followings:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 1 | 0x0: Special U-TDOA 0x1: General U-TDOA |  |
| Frame identifier | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe |  |
| Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |  |
| Ranging opportunity index for the serving ABS | 3 | Indicates the index of the allocated ranging opportunity. |  |
| Subframe Index for the serving ABS | 2 | Indicates the subframe index of the allocated ranging opportunity. |  |
| If (Action code == 0x1) { |  |  |  |
| Neighbor_ABS_Info_Array | 8 | List of neighbor ABS Info |  |
| For (i=0; i< Neighbor_ABS_Info_Array; i ++){ |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to AAI-NBR-ADV message |  |
| Action time | 8 | Action Time included in this message is the absolute frame number at the serving ABS. |  |
| Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code |  |
| Ranging opportunity index of neighbor ABS | 3 | Indicates the index of the allocated ranging opportunity |  |
| Subframe Index of neighbor ABS | 2 | Indicates the subframe index of the allocated ranging opportunity |  |
| } |  |  |  |
| } |  |  |  |

-------------------------------  Text End  ---------------------------------------------------