| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for cleanup multiple carriers for LBS(16.8.3) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Hai Wang China Samsung Telecom R&D Center Sangheon Kim, Ji-yun Seol, Hyunjeong Kang, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D8 |  |
| Abstract | This contribution proposes to remove multiple carriers for LBS |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for cleanup multiple carriers for LBS(16.8.3)

Lei Zhou, Xufeng Zheng, Hai Wang

China Samsung Telecom R&D Center

Sangheon Kim, Ji-yun Seol, Hyunjeong Kang, Jaeweon Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes to remove the function of multiple carriers for LBS because actually, multiple carriers for LBS can’t enhance performance of LBS. At the same time, relative delay can’t be used for handover. So the contribution proposes to remove parameter relative delay for multiple carriers in AAI-SCN-REP and description sentence in E-LBS zone.

The Proposed Text in AWD

------------------------------- Text Proposal Start ---------------------------------------------------

[Remedy 1: Modify the text on page 893 between lines 11 and 21 in P802.16m/D8 with the followings:]

-------------------------------------------------------Begin Proposed Text Changes----------------------------------------------------

16.8.3 Enhanced LBS Support

The Enhanced LBS (ELBS) is an optional LBS capability involving the following functions:

- ABS ability to transmit special ELBS waveforms to be used for location specific measurements to enable more

accurate location determination.

- ABS’s ability to signal to the AMS the location of the special ELBS waveforms.

- AMS’s ability to detect, measure and report relevant measurements to the ABS.

- LBS measurement may be performed on multiple carriers.

-----------------------------------------------------End Proposed Text Changes-------------------------------------------------------------

[Remedy 2: Modify Table 695 beginning line 46 on Page 142 in P802.16m/D8 with the followings:]

-------------------------------------------------------------Begin Proposed Text Changes----------------------------------------------------------------

Table 695—AAI-SCN-REP parameters

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| ... | ... |  |  | ... | ... | ... |
| O |  |  | D) Relative delay | 8 | This parameter indicates the delay of neighbor DL signals relative to the S-ABS, as measured by the MS for this carrier. The value shall be interpreted as a signed integer in units of samples. | Present if AMS is asked to report relative delay |
| O | Neighbor_request_indication |  |  | 1 | Request indication for system information and the list of the neighbor Femto ABSs |  |
| ... | ... |  |  | ... | ... | ... |

-------------------------------------------------------------End Proposed Text Changes----------------------------------------------------------------

------------------------------- Text Proposal End ---------------------------------------------------