| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to Femto IM and AAI-SON-ADV message (16.2.3.6) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.3 (MAC Control messages) |  |
| Abstract | This contribution proposes changes to the Section 16.2.3.6 of IEEE P802.16m/D8 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to Femto IM and AAI-SON-ADV Message (16.2.3.6)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

In current D8, to mitigate the interference between Femto ABSs, it is said that “Femto ABS may disable some of its subframes and announce the disabled subframes via AAI_SON-ADV”. However, there is no parameter in the AAI_SON-ADV message enabling this function. In this contribution, we propose a new parameter, “Disabled subframe index”, in the AAI_SON-ADV message to enable this function.

- Proposed Text for IEEE P802.16m/D8

[Accept the following modifications marked in blue and red]

============================ Start of Proposed Text ==================================

[Accept the following modifications from line 27, page 106 (16.2.3.7)]

#### 16.2.3.7 AAI_SON-ADV message

Table 685—Parameters for AAI_SON-ADV message

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action type | 2 | Used to indicate the purpose of this message 0b00: ABS Reconfiguration 0b01: ABS Restart 0b10: ABS Scanning 0b11: ABS Reliability | mandatory |
| O | Unavailable start time (UST) |  | Start of unavailable time | shall present if Action type = (0b00, 0b01, 0b10) may present if Action type =0b11 |
| O | Unavailable Time Interval (UTI) |  | Interval of unavailable time | shall present if Action type =(0b00, 0b01, 0b10) may present if Action type =0b11 |
| O | Reason | 23 | 0b000: Power down 0b001: Power reduction 0b010: FA change 0b011: Backhaul link down 0b100: Disable subframes | If Action type=0b11, it is mandatory, otherwise it will not exist. |
| O | Tx power reduction |  | dB value of Tx power reduction | If Reason = 0b001, it is mandatory. |
| O | FA index |  | FA index | If Reason = 0b010, it is mandatory. |
| O | Disabled subframes index | 6 | The status of DL subframes Bit 0: the status of the first DL subframe Bit 1: the status of the 2nd DL subframe Bit 2: the status of the 3rd DL subframe Bit 3: the status of the 4th DL subframe Bit 4: the status of the 5th DL subframe Bit 5: the status of the 6th DL subframe (0 means disabled or no such subframe; 1 means enabled) | If Reason = 0b100, it is mandatory. |
| O | Time of power down |  | Expected power down time or resource adjustment time or current FA downtime | If Action type = 0b11, it is mandatory, otherwise it will not exist. |
| O | Expected uptime or new FA uptime |  |  | If Action type = 0b11, it is optional, otherwise it will not exist. |
| O | New IDcell |  | New IDcell that the ABS will use after the reconfiguration process. | If Action type=0b00, it is optional, otherwise, it will not exist. |
| O | Recommended BSID list |  | Recommended BSID list | It is optional to help AMS to HO. |

[Insert the blue text into line 4, page 849 (16.4.11)]

#### 16.4.11 Interference Avoidance and Interference Mitigation

The interference between Femto and/or macro can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner and/or DL power control. While using the TDM manner, Femto ABS may disable some of its subframes and announce the disabled subframes via “Disabled subframes index” in the AAI_SON-ADV message. Femto ABS shall keep the midambles, preambles, and common pilots in the disabled subframes.

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D8. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, August 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.