| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-RNG-REQ (16.2.3.1) |  |
| Date Submitted | 2010-9-16 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Xiangying Yang Intel | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-RNG-REQ |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-RNG-REQ (16.2.3.1)

Kelvin Chou

MediaTek Inc.

Xiangying Yang

Intel

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 83, line 14, replace Table 679 with the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 679—AAI-RNG-REQ message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Purpose Indication | 4 | 0b0000 = Initial network entry 0b0001 = HO reentry 0b0010 = Network reentry from idle mode 0b0011 = Idle mode location update 0b0100 = DCR mode extension 0b0101 = Emergency call setup (e.g., E911) 0b0110 = Location update for updating service flow management encodings of E-MBS flows 0b0111 = Location update for transition to DCR mode from idle mode 0b1000 = Reentry from DCR mode, coverage loss or detection of different ABS restart count. 0b1001 = Network reentry from a Legacy BS 0b1010 = Zone switch to MZONE from LZONE 0b1011 = Location update due to power down. 0b1100 = Experiencing “femto interference” 0b1101 = NS/EP call setup 0b1110 - 0b1111 = reserved | - |
| CMAC indicator | 1 | Indicate whether this message is protected by CMAC tuple 0b0: not protected 0b1: protected | Shall always be present |
| If (Ranging Purpose Indication = 0b0000) { |  | // Initial network entry |  |
| If (advanced network mode and AMSID privacy is enabled) { |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to Table 16.2.5.3.1 |  |
| } else { |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } |  |  |  |
| MAC version | 8 | See 11.1.3 |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from -15dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful initial ranging process |  |
| } else if (Ranging Purpose Indication = 0b0001) { |  | // HO reentry //a: uncontrolled HO //b: HO |  |
| If (uncoordinated HO) { |  |  |  |
| Serving BSID | 48 | The BSID of the AMS's previous serving ABS (S-ABS) before incurring a coverage loss, or the BSID of the S-ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). |  |
| Previous STID | 12 | The STID which the AMS uses in the previous S-ABS. |  |
| } else{ |  |  |  |
| if (legacy network mode){ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address | Presented in legacy network mode only |
| } else{ |  |  |  |
| STID | 12 | The Station ID pre-assigned by the Target ABS |  |
| } |  |  |  |
| } |  |  |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs 1≦N_FIDs≦15 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b0010) { |  | // Network reentry from idle mode |  |
| if (legacy network mode){ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address | Presented in legacy network mode only. |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 12 | The ID which the AMS is assigned for idle mode and currently maintains. | Presented in advanced network mode only. |
| } |  |  |  |
| Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. |  |
| PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| Paging Cycle | 4 | PAGING_CYCLE applied to the AMS |  |
| Paging Offset | 12 | PAGING_OFFSET applied to the AMS |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs 1≦N_FIDs≦15 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b0011 ¦0b0110¦0b0111¦ 0b1011) { |  | // Idle mode location update (and with other additional purposes) |  |
| if (legacy network mode){ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address | Presented in legacy network mode only. |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 12 | The ID which the AMS is assigned for idle mode and currently maintains. | Presented in advanced network mode only. |
| } |  |  |  |
| Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. |  |
| PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| Paging Cycle | 4 | PAGING_CYCLE applied to the AMS |  |
| Paging Offset | 12 | PAGING_OFFSET applied to the AMS |  |
| Paging Cycle Change | 4 | PAGING_CYCLE requested by the AMS | Presented if AMS wants to change Paging Cycle |
| Paging Carrier update | 6 | Preferred Paging carrier index requested by the AMS | Presented if AMS wants to change Paging Carrier |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented for secure Location Update |
| } |  |  |  |
| AMS Mobility Information | 2 | 0b00 = Slow (0-10km/h) 0b01 = Medium (10-120km/h) 0b10 = Fast (above 120km/h) 0b11 = Reserved | Optional |
| SMS | var | Short Message content up to 140 bytes | May be presented when there is SMS content to be sent |
| } else if (Ranging Purpose Indication = 0b0100) { |  | // DCR mode extension |  |
| CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b0101) { |  | // Emergency call setup (e.g., E911) |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| MAC version | 8 | See 11.1.3 |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from -15dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful initial ranging process |  |
| } else if (Ranging Purpose Indication = 0b1000) { |  | // Reentry from DCR mode, coverage loss or detection of different ABS restart count. |  |
| CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs 1≦N_FIDs≦15 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b1001) { |  | // Network reentry from a Legacy BS |  |
| If (advanced network mode and AMSID privacy is enabled) { |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to Table 16.2.5.3.1 |  |
| } else{ |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } |  |  |  |
| Serving BSID | 48 | The BSID of the previous serving BS. |  |
| Previous Basic CID | 16 | The Basic CID which the AMS used in the previous serving BS |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b1010) { |  | // Zone switch to MZONE from LZONE |  |
| If (advanced network mode and AMSID privacy is enabled) { |  |  |  |
| AMSID* | 48 | The AMSID hash value. Refer to Table 16.2.5.3.1 |  |
| } |  |  |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication. |
| } |  |  |  |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs 1≦N_FIDs≦15 |  |
| FID | 4 | Flow identifier |  |
| FID change count | 4 | FID change count |  |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b1100) { |  | // Experiencing “femto interference” |  |
| If (CMAC indicator =0b1){ |  |  |  |
| AK_COUNT | 16 | The AMS's current value of the AK_COUNT, which is used to update the security keys in the T-ABS. | Shall be presented if the AMS has a CMAC Tuple necessary to expedite security authentication |
| } |  |  |  |
| } else if (Ranging Purpose Indication = 0b1101) { |  | // NS/EP call setup |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| MAC version | 8 | See 11.1.3 |  |
| Initial Offset for uplink power control (OffsetInitial) | 5 | The bit size represents power level ranging from -15dB (0x00) to 16dB (0x1F) with 1dB step The value is determined by AMS after successful initial ranging process |  |
| }//end of Ranging Purpose Indication |  |  |  |
| for (i=0; i<N_CSG_ID_Infos; i++) { |  | N_CSG_ID_Infos is the number of CSG ID Information blocks. 1≦N_CSG_ID_Infos≦15 | Optional for loop. May be included for quick CSG membership detection or ABS reselection assistance |
| Operator ID of the CSG Femtocell | 24 | The Operator ID of the CSG Femtocell | Present if the Operator ID is different from the one of the ABS |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs belongs to this Operator ID. |  |
| CSGID | var | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------