| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-RNG-RSP (16.2.3.2) |  |
| Date Submitted | 2010-9-16 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Xiangying Yang Intel | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-RNG-RSP (16.2.3.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-RNG-RSP (16.2.3.2)

Kelvin Chou

MediaTek Inc.

Xiangying Yang

Intel

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 88, line 27, replace Table 680 with the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 680—AAI-RNG-RSP message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Ranging Abort | 1 | Set to 1 when an ABS rejects the AMS | Presented when an ABS rejects an AMS |
| If (Ranging Abort = 1) { | - |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65535, In units of seconds |  |
| }else{ |  |  |  |
| Temporary STID | 12 | Used for AMS identification until STID is assigned to the AMS during registration procedure. | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected and when the AMS has not been assigned its STID/DID. |
| } |  |  |  |
| If (advanced network mode) { |  |  | Shall be included in response to the AAI-RNG-REQ message which is not CMAC protected when the AMS has not been assigned its STID. |
| AMSID* | 48 |  |  |
| } else { // legacy network mode |  |  |  |
| AMS MAC address | 48 |  |  |
| } |  |  |  |
| STID | 12 | AMS identification to be used in the T-ABS | Shall be included for uncontrolled HO, NW reentry or Zone switching in case this message is encrypted |
| CRID | 72 | The AMS identifier newly assigned by the network for coverage loss or DCR mode | Shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/zone-switch with assigning a new CRID in response to AAI-RNG- REQ or it shall be included when the network assigns a new CRID unsolicitedly to the AMS. |
| Emergency Service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | Shall be included when this message is sent in response to an AAI-RNG-REQ with Ranging Purpose Indication set to code 0b0101 |
| NS/EP service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | Shall be included when this message is sent in response to an AAI-RNG-REQ with Ranging Purpose Indication set to code 0b1101 |
| Location Update Response | 4 | 0x0= Success of Location Update 0x1= Failure of Location Update 0x2 = Reserved 0x3 = Success of location update and DL traffic pending 0x4 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x5 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x6~0xF: Reserved | Shall be included when this message is sent in response to an AAI- RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| If (Location Update Response = 0x0 ) { | - |  |  |
| New Paging Cycle | 4 | New Paging Cycle to be assigned to the AMS 0b0000-0b1111 | Shall be included when the Paging parameters are changed. |
| New Paging Group ID | 16 | New PGID of the AMS |  |
| New Paging Offset | 12 | New paging offset of AMS which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. |  |
| Deregistration Identifier (DID) | 12 | The new DID which the AMS shall maintain in idle mode. | Shall be included only in advanced network mode when DID is changed. |
| New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | Shall be included only if Paging Controller ID is changed. |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| For(i=0; i< N-E-MBS-Zone-IDs; i++) { | - |  |  |
| Service flow update indicator | 1 | Indicator whether “bitmap + new service flow” or “current service flow + new service flow” is included. 0b0: “Service Flow Update Bitmap+ New_E-MBS ID and FID” 0b1: “Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings” |  |
| E-MBS Zone ID | 7 | The E-MBS Zone ID to update |  |
| New E-MBS Zone ID | 7 | The new E-MBS Zone ID which the AMS shall maintain in any state on being served E-MBS. | Present only if the new E-MBS Zone is served in the T-ABS. |
| Physical Carrier Index | 6 | The physical carrier index of the T-ABS that supports the new EMBS Zone. | Present only if the new E-MBS Zone ID is served on a different carrier from the previous one at the T-ABS. |
| If(Service flow update indicator = 0b0) { | - |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by EMBS ID and FID, is updated. |  |
| For each 1-bit in the Service Flow Update Bitmap { |  |  |  |
| New E-MBS ID | 12 | New E-MBS identifier |  |
| FID | 4 | New Flow ID |  |
| } |  |  |  |
| }else if ( Service flow update indicator = 0b1) { |  |  |  |
| For (j=0; j<N_EMBS_IDs; j++) { | - | N_EMBS_IDs is the number of EMBS IDs listed here |  |
| Current E-MBS ID | 12 | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current FID | 4 |  |  |
| New E-MBS ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| }//end of if | - |  |  |
| }//end of for (N-E-MBS-Zone-IDs) | - |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS | . Presented if it needs to be updated. |
| For (i=0; i< M; i++) { | - | Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated |  |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| SMS | Var | Short Message content up to the size of 140 bytes | May be included when SMS contents is sent in idle mode. |
| }//end of If (Location Update Response = 0x0 ) | - |  |  |
| Reentry Process Optimization | 5 | Reentry process optimization bitmap indicates which MAC control message transactions may be omitted during an attempted reentry (i.e. reentry during HO (including zone switching), and reentry from idle mode) A value of 1 in the bitmap indicates that the corresponding MAC control message transaction may be omitted, while a 0 indicates that the corresponding MAC control message transaction shall be completed. The AMS shall only commence normal operation with the T-ABS after completing all the required MAC control message transactions. Bit #0: Omit AAI-SBC-REQ and AAI-SBC-RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI-REG-REQ and AAI-REG-RSP message during reentry processing. Bit #3: Omit higher layer protocol triggering for IP address refresh during reentry processing Bit #4: For the case of reentry during HO including zone switching, a 1 indicates to the AMS that the T-ABS has received the full service and operational states for static and dynamic context (including ARQ window parameters and state machines). For the case of reentry from Idle mode, a 1 indicates to the AMS that the T-ABS has received the static context of the AMS. The static context includesSFIDs and related description (QoS descriptors and CS classifier information) for all service flows that the AMS has currently established as well as any SAs with their related keying information. | It shall be included when the AMS is attempting to perform network reentry (i.e. reentry during HO (including zone switching), and reentry from idle mode), and the T-ABS wishes to identify which MAC control message transactions may be omitted. |
| Activation Deadline | 6 | LSB bits of Superframe number after the AAI-RNG-RSP message to confirm the activation of pre-assigned secondary carrier by sending the AAI-CM-IND message. | Shall be included when the T-ABS has pre-assigned the secondary carriers through AAI-HO-CMD. |
| Neighbor station measurement report indicator | 1 | 1: Perform Neighbor station measurement report. | Optional. Identifies Neighbor station measurement report is required during current network entry for ARS. |
| Resource Retain Time | 8 | The time when the S-ABS discards AMS's context | May be included if Reentry Process Optimization Bit #3 is set to 1. |
| For (i=0; i<N_FIDs; i++) { |  | N_FIDs is the number of FIDs to be included here | Shall be included if the ABS needs to update the AMS's existing flows. FIDs that are not presented here shall be regarded as guaranteed by the ABS. |
| SFID | 32 | SFID of the existing flow to be updated |  |
| Update or delete | 1 | 0: delete the flow 1: updated QoS Info exists |  |
| Updated QoS Info | Var | All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this field. |  |
| } |  |  |  |
| Unsolicited bandwidth grant indicator | 1 | 1: indicates an unsolicited BW grant will be available for transmission of BR header without request from AMS during network entry | Shall be included when AMS is attempting network entry |
| CLC Response | Var | The information of co-located coexistence response (as defined in Table 698) | Shall be included when the AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before handover. |
| CSGID | Var | CSGID of the Femto ABS sending the AAI-RNG-RSP | Optional. If included, to help the AMS make the decision if the Femto ABS is accessible or not. |
| For (i=0; i<N_Redirect_Info; i++) { |  | N_Redirect_Info is the number of redirection information included here. ABSID, preamble index and center frequency for one or more neighbor ABSs of the S-ABS | Sent by S-ABS to aid cell reselection in case the S-ABS does not allow the AMS to perform entry (due to various reasons such as high S-ABS load, non existence of CSG membership etc) |
| ABSID for neighbor ABS | 48 |  |  |
| Preamble index for neighbor ABS | 10 |  |  |
| Center frequency for neighbor ABS | 32 |  |  |
| } |  |  |  |
| Ranging Request bit | 1 | 1 : AMS shall send the AAI-RNG-CFM message after a successful periodic ranging | Shall be included when S-ABS performs coverage loss detection procedure (16.2.26) |
| For (i=0; i<N_Invalid_FIDs; i++) { |  | N_Invalid_FIDs is the number of invalid FIDs listed here | Shall be included if there is any invalid FID. ABS and AMS may exchange the AAI-DSx-messages for invalid FIDs |
| FID |  |  |  |
| } |  |  |  |
| SAID update bitmap | 16 | Bitmap for indicating the specific FID(s) that are being updated to SAID of AES-CTR | Shall be included if specific FID(s) are to be remapped to SAID of AES-CTR in case of Zone Switch from LZone to MZone |
| If(it is under HO reentry){ |  |  |  |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| For(i=0; i< N-E-MBS-Zone-IDs; i++) { | - |  |  |
| Service flow update indicator | 1 | Indicator whether “bitmap + new service flow” or “current service flow + new service flow” is included. 0b0: “Service Flow Update Bitmap+ New_E-MBS ID and FID” 0b1: “Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings” |  |
| E-MBS Zone ID | 7 | The E-MBS Zone ID to update |  |
| New E-MBS Zone ID | 7 | The new E-MBS Zone ID which the AMS shall maintain in any state on being served E-MBS. | Present only if the new E-MBS Zone is served in the T-ABS. |
| Physical Carrier Index | 6 | The physical carrier index of the T-ABS that supports the new EMBS Zone. | Present only if the new E-MBS Zone ID is served on a different carrier from the previous one at the T-ABS. |
| If(Service flow update indicator = 0b0) { | - |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by EMBS ID and FID, is updated. |  |
| For each 1-bit in the Service Flow Update Bitmap { |  |  |  |
| New E-MBS ID | 12 | New E-MBS identifier |  |
| FID | 4 | New Flow ID |  |
| } |  |  |  |
| }else if ( Service flow update indicator = 0b1) { |  |  |  |
| For (j=0; j<N_EMBS_IDs; j++) { | - | N_EMBS_IDs is the number of EMBS IDs listed here |  |
| Current E-MBS ID | 12 | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current FID | 4 |  |  |
| New E-MBS ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| }//end of if | - |  |  |
| }//end of for (N-E-MBS-Zone-IDs) | - |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS | Presented if it needs to be updated. |
| For (i=0; i< M; i++) { | - | Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated |  |
| Current Multicast Group ID | 12 |  |  |
| Current FID | 4 |  |  |
| New Multicast Group ID | 12 |  |  |
| New FID | 4 |  |  |
| } | - |  |  |
| }//end of If (it is under HO reentry ) | - |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------