| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-RNG-ACK and AAI-RNG-CFM (16.2.3.3) |  |
| Date Submitted | 2010-9-8 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-RNG-ACK and AAI-RNG-CFM |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-RNG-ACK and AAI-RNG-CFM (16.2.3.3)

Kelvin Chou

MediaTek Inc.

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 95, line 18, replace Table 681 with the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 681—AAI-RNG-ACK message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (transmitted in a DL resource allocated by Broadcast assignment A-MAP IE) { | - |  |  |
| N_Frame_Identifiers | 3 | The number of frame identifiers included in this RNG-ACK message. |  |
| For (i=0; i<N_Frame_Identifiers; i++) { | - |  |  |
| Frame identifier | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe (ranging from 0b00 to 0b11). |  |
| RNG-ACK Bitmap | 4 | Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. |  |
| For each 0b1-bit in the RNG-ACK Bitmap { | - |  |  |
| N_Received_Codes | 6 | The number of ranging preamble code indices detected in this corresponding ranging opportunity. |  |
| For (j=0; j<N_Received_Codes; j++) { | - |  |  |
| Ranging preamble Code Index | 6 | Ranging preamble code index received in this ranging opportunity. |  |
| Ranging Status | 2 | Indicate whether ranging preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue |  |
| If (Ranging Status = 0b01) { | - |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65535, In units of seconds |  |
| }else if (Ranging Status = 0b00¦0b10) { | - |  |  |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |  |
| If( API Bit#0=0b1 ) { | - |  |  |
| Timing offset adjustment | 15 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. The AMS shall advance its transmission time if the value is negative and delay its transmission time if the value is positive. |  |
| } | - |  |  |
| If( API Bit#1=0b1 ) { | - |  |  |
| Power level adjustment | 3 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the MS shall apply to its current transmission power for initial ranging. The bit size represents power level correction in the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. |  |
| } | - |  |  |
| If (API Bit#2=0b1) { | - |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (Δf) multiplied by the 9-bit number interpreted as a signed integer |  |
| } | - |  |  |
| }//end of (Ranging Status = 0b00¦0b10) | - |  |  |
| }//end of for | - |  |  |
| }//end of for each | - |  |  |
| }//end of for | - |  |  |
| } else if (transmitted in a DL resource allocated by DL basic assignment A-MAP IE) { | - |  |  |
| Ranging Status | 2 | Indicate whether ranging preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue |  |
| If (Ranging Status = 0b01) { | - |  |  |
| Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1-65535, In units of seconds |  |
| }else if (Ranging Status = 0b00¦0b10) { | - |  |  |
| Adjustment parameters indication (API) | 3 | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication |  |
| If( API Bit#0=0b1 ) { | - |  |  |
| Timing offset adjustment | 15 | Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. The AMS shall advance its transmission time if the value is negative and delay its transmission time if the value is positive. |  |
| } | - |  |  |
| If( API Bit#1=0b1 ) { | - |  |  |
| Power level adjustment | 3 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the MS shall apply to its current transmission power for initial ranging. The bit size represents power level correction in the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. |  |
| } | - |  |  |
| If (API Bit#2=0b1) { | - |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (Δf) multiplied by the 9-bit number interpreted as a signed integer |  |
| } | - |  |  |
| }//end of (Ranging Status = 0b00¦0b10) | - |  |  |
| } | - |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 99, line 42, replace Table 682 with the follow table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 680—AAI-RNG-CFM message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| STID | 12 | STID of the AMS | N/A |

--------------------------------------------------  Text End  -----------------------------------------------------------------