| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-SBC-REQ and AAI-SBC-RSP (16.2.3.5) |  |
| Date Submitted | 2010-9-15 |  |
| Source(s) | Kelvin Chou MediaTek Inc. Shaocheng Wang Intel | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-SBC-REQ and AAI-SBC-RSP (16.2.3.5) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-SBC-REQ and AAI-SBC-RSP (16.2.3.5)

Kelvin Chou

MediaTek Inc.

Shaocheng Wang

Intel

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

Note that the version of the contribution C802.16m-10/1114r1 attempts to resolve the following comments:

- Comment B195 / contribution C802.16m-10/1230  suggested to accept contribution 1230r4

- Comment B10024  accept the suggested remedy text

- Comment B204 suggested to accept contribution 1250

Proposed Text

Remedy 1:

In the P802.16m/D8, page 100, line 4, modify Table 683 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686—AAI-SBC-REQ message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
|  | If (AMS requests transmittal of NSP information) { |  |  |  |
|  | SIQ (Service Information Query) |  |  |  |
|  | }else{ |  |  |  |
| M | CAPABILITY_INDEX | 5 | It refers to the "Capability Class" that the AMS can sup-port. Value: 0~31 |  |
| M | DEVICE_CLASS | 5 | It refers to the "Device Class" that the AMS can support. Value: 0~31 |  |
| O | CLC Request | Variable | See Table 696 | Present if AMS requests to activate one Type I or II CLC class for fast CLC class activation during initial network entry |
| O | Long TTI for UL | 1 | If Bit#0 =1,it supports | Present as needed |
| O | Long TTI for DL | 1 | If Bit#0 =1,it supports | Present as needed |
| O | UL sounding | 1 | If Bit#0 =1,it supports | Present as needed |
| O | DL resource metric for FFR | 1 | If Bit#0 =1,it supports | Present as needed |
| O | Max. Number of streams for SU-MIMO in DL MIMO | 3 | The number in the range 1 through 8 that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1 | The number in the range 1 through 2 that is higher by 1 than this field | Present as needed |
| O | DL MIMO mode | 36 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports | Present as needed |
| O | feedback support for DL | 9 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports | Present as needed |
| O | Number of Tx antenna of MS | 2 | The number in the range {1, 2, 4} that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 2 | The number in the range 1 through 3 that is higher by 1 than this field | Present as needed |
| O | UL MIMO mode | 5 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports | Present as needed |
| O | Modulation scheme | 2 | If Bit#0=1, DL 64 QAM sup-ports If Bit#1=1, UL 64 QAM sup-ports | Present as needed |
| O | UL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. | Present as needed |
| O | DL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. | Present as needed |
| O | AMS DL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per subframe in units of 600 bytes that the AMS can process. | Present as needed |
| O | AMS UL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per subframe in units of 600 bytes that the AMS can process. | Present as needed |
| O | FFT size(2048/1024/512) | 3 | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports | Present as needed |
| O | Authorization policy support | 1 | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported | Present as needed |
| O | Inter-RAT Operation Mode | 2 | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved | Present as needed |
| O | Supported Inter-RAT type | 8 | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero | Present as needed |
| O | MIH Capability Sup-ported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services sup-ports. | Present as needed |
| O | MAX Tx Power | 24 | Bit 0-7: Maximum transmit-ted power for QPSK. Bit 8-15: Maximum transmit-ted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. | Present as needed |
|  | } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 103, line 10, modify Table 684 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 687—AAI-SBC-RSP message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
|  | If (SIQ bit 0 in AAI-SBC_REQ is set and AAI-SII-ADV Message Pointer is not included) { |  |  |  |
|  | for (i=0; i<number of NSP IDs; i++) { |  | List of the Network Service Provider identifiers |  |
|  | NSP identifier | 24 | NSP identifier which ABS supports |  |
|  | } |  |  |  |
|  | If (if SIQ bit 1 in AAI-SBC_REQ is set) { |  |  |  |
|  | for (i=0; i< number of NSP IDs; i++) { |  | List of the verbose names of the NSPs. The order of Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. |  |
|  | verboseNspName | Variable (1 ... 128 x 8) | Verbose NSP name string |  |
|  | } |  |  |  |
|  | } |  |  |  |
|  | }else if (SIQ bit 0 in AAI-SBC_REQ is set and if AAI-SII-ADV Message Pointer is included) { |  |  |  |
|  | AAI-SII-ADV Message Pointer | 14 | The 14 least significant bits of the frame number of the frame in which the AAI-SII-ADV message with requested NSP information is transmitted. |  |
|  | }else{ |  |  |  |
| M | CAPABILITY_INDEX | 5 | It refers to the "Capability Class" that the AMS can sup-port. Value: 0~31 |  |
| M | DEVICE_CLASS | 5 | It refers to the "Device Class" that the AMS can support. Value: 0~31 |  |
| O | CLC Response | variable | see Table6967 | Present if CLC Request is included in the corresponding AAI-SBC-REQ |
| O | Long TTI for UL | 1 | If Bit#0 =1,it supports | Present as needed |
| O | Long TTI for DL | 1 | If Bit#0 =1,it supports | Present as needed |
| O | UL sounding | 1 | If Bit#0 =1,it supports | Present as needed |
| O | DL resource metric for FFR | 1 | If Bit#0 =1,it supports | Present as needed |
| O | Max. Number of streams for SU-MIMO in DL MIMO | 3 | The number in the range 1 through 8 that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1 | The number in the range 1 through 2 that is higher by 1 than this field | Present as needed |
| O | DL MIMO mode | 36 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports | Present as needed |
| O | feedback support for DL | 9 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports | Present as needed |
| O | Number of Tx antenna of MS | 2 | The number in the range {1, 2, 4} that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | The number in the range 1 through 4 that is higher by 1 than this field | Present as needed |
| O | Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 2 | The number in the range 1 through 3 that is higher by 1 than this field | Present as needed |
| O | UL MIMO mode | 5 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports | Present as needed |
| O | Modulation scheme | 2 | If Bit#0=1, DL 64 QAM sup-ports If Bit#1=1, UL 64 QAM sup-ports | Present as needed |
| O | UL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. | Present as needed |
| O | DL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. | Present as needed |
| O | AMS DL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. | Present as needed |
| O | AMS UL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. | Present as needed |
| O | FFT size(2048/1024/512) | 3 | If Bit#0 =1, FFT 2048 sup-ports If Bit#1 =1, FFT 1024 sup-ports If Bit#2 =1, FFT 512 supports | Present as needed |
| O | Authorization policy support | 1 | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported | Present as needed |
| O | Inter-RAT Operation Mode | 2 | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved | Present as needed |
| O | Supported Inter-RAT type | 8 | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero | Present as needed |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services sup-ports. | Present as needed |
|  | } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------