| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-REG-REQ and AAI-REG-RSP (16.2.3.8) |  |
| Date Submitted | 2010-9-8 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on AAI-REG-REQ and AAI-REG-RSP (16.2.3.8) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-REG-REQ and AAI-REG-RSP (16.2.3.8)

Kelvin Chou

MediaTek Inc.

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 107, line 2, modify Table 686 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686—AAI-REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | AMS MAC address | 48 | AMS's real MAC address |  |
| O | AMS capability negotiation parameters | Variable |  | Present if needed |
| O | A) ARQ parameters | Variable |  | Present if needed |
| O | A.1) ARQ_SN_MODULUS | 10 | the number of unique SN values | Present if needed |
| O | A.2) ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | Present if needed |
| O | A.3) ARQ_SUB_BLOCK_SIZE | 10 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.4) ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.5) ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | Present if needed |
| O | A.6) MAXIMUM_ARQ_BUFFER_SIZE | 16 | The maximum ARQ buffer size allowed by the system. The actual maximum ARQ buffer size depends on AMS's memory, and shall be below 2048 bytes. | Present if needed |
| O | B) Non-ARQ parameters |  |  |  |
| O | B.1) MAXIMUM_NON_ARQ_BUFFER_SIZE (P) | 23 5 | Non-ARQ buffer size depending on AMS's memory. Buffer size is defined as 2P(0<=P<=23). Unit is byte. | Present if needed |
| O | C) Multicarrier capabilities | 3 | 0b000: No MC modes 0b001: Basic MC mode 0b010: Multicarrier Aggregation 0b011: Multicarrier Switching 0b100: Both Multicarrier Aggregation and Multicarrier Switching | Present if needed |
| O | D) Capabilities for Zone switching mode |  |  | Present if needed |
| O | D.1) Zone Switch Mode Support | 1 | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported | Present if needed |
| O | E) LBS Capabilities |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported | Present if needed |
|  | If (LBS Capabilities are required to setup) { |  |  |  |
| O | E.1) Capability for supporting A-GPS method | 1 | 0b0: no support 0b1: support | Present if needed |
|  | } |  |  |  |
| O | F) Capabilities for interference mitigation support If (Interference mitigation is supported) { |  |  | Present if needed |
| O | F.1) DL PMI coordination capability | 1 | 0: AMS is not DL PMI coordination capable 1: AMS is DL PMI coordination capable | Present if needed |
| O | F.2) DL collaborative multi-BS MIMO capability | 1 | 0: AMS is not DL collaborative multi-BS MIMO capable 1: AMS is DL collaborative multi-BS MIMO capable | Present if needed |
| O | F.3) DL closed-loop multi-BS macro diversity capability | 1 | 0: AMS is not DL closed-loop multi-BS macro diversity capable 1: AMS is DL closed-loop multi-BS macro diversity capable | Present if needed |
| O | F.4) UL PMI combination capability | 1 | 0: AMS is not UL PMI combination capable 1: AMS is UL PMI combination capable | Present if needed |
| O | F.5) Multi_BS sounding calibration capability | 1 | 0: AMS is not multi_BS sounding calibration capable 1: AMS is multi_BS sounding calibration capable | Present if needed |
|  | } |  |  |  |
| O | G) E-MBS capabilities | 3 | If Bit#0 is set to 1, it indicates E-MBS in S-ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present if needed |
| O | H) Channel BW and Cyclic prefix | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) | Present if needed |
| O | I) frame configuration to sup-port legacy(5MHz) | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| O | J) frame configuration to sup-port legacy(10MHz) | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| O | K) Persistent Allocation support | 1 | If Bit#0 =1,it supports | Present if needed |
| O | L) Group Resource Allocation support | 1 | If Bit#0 =1,it support | Present if needed |
| O | M) Co-located coexistence capability support | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class | Present if needed |
| O | N) HO trigger metric | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | Present if needed |
| O | O) Capabilities for Handover EBB mode |  |  | Present if needed |
| O | O.1) EBB Handover support | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported | Present if needed |
| O | P) Capabilities for Handover EBB mode If (EBB Handover support = 0b1) { |  |  | Present if needed |
| O | P.1) Minimal HO Reentry Interleaving Interval | 2 | The minimal HO_Reentry_Interleaving_Interval measured in frames. For AMS capable of Carrier Aggregation (Multicarrier Capability = 0b010 or 0b100), this value shall be 0. Shall be included only if EBB support is set to 1 | Present if needed |
|  | } |  |  |  |
| O | Q) Capability for sounding antenna switching |  |  |  |
| O | Q.1) Capability for sounding antenna switching support | 1 | 0b0: no support 0b1: support |  |
|  | If (Capability for sounding antenna switching support = 0b1) { |  |  |  |
| O | Q.2) Antenna configuration for sounding antenna switching | 1 | 0b0: Among DL Rx antennas 0b1: Among UL Tx antennas | Present if Capability for sounding antenna switching support is 0b1. |
|  | } |  |  |  |
| O | Convergence sublayer capabilities |  |  | Present if needed |
| O | A) CS type | 16 or 32 | A bit set to "1" Indicates which CS Type the AMS supports Bit #0: Reserved Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: Reserved Bit #4: Reserved Bit #5: Reserved Bit #6: Reserved Bit #7: Reserved Bit #8: Reserved Bit #9: Reserved Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, none Bit #15: Multiprotocol flow, IPv4 or IPv6 Traffic Bit #16 - Bit #31 Reserved | Present if needed |
| O | B) Maximum number of classification rules | 16 | Maximum number of simultaneous admitted classification rules that a service flow supports. | Present if needed |
| O | C) ROHC support | 1 | 0: not support 1: support | Present if needed |
| O | D) PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed |
| O | E) IP-Service-Type IE | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if needed |
| O | Host configuration capabilities and its parameters | Variable |  | Present if needed |
| O | A) Host-Configuration-Capability-Indicator IE | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI-REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. 0b0: not support 0b1: support | To be included only when Requested-Host-Configurations IE is not included |
| O | B) Requested-Host-Configurations IE | Variable | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI-REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. | To be included when additional host configurations are required. |
| O | Global carrier configuration change count | 3 | Indicates the AMS's last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. | Present if needed |
| O | AMS initiated aGP Service Adaptation Capability: | 1 | 0b0: no support 0b1: support | Present if needed |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 112, line 27, modify Table 687 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 687—AAI-REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | STID | 12 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. | Present if needed |
| O | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
| O | A) AMS capability negotiation parameters |  |  |  |
| O | A.1) ARQ parameters |  |  |  |
| O | A.1.1) ARQ_SN_MODULUS | 10 | the number of unique SN values | Present if needed |
| O | A.1.2) ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | Present if needed |
| O | A.1.3) ARQ_SUB_BLOCK_SIZE | 10 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.1.4) ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.1.5) ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | Present if needed |
|  | A.2) If (Femto ABS LDM parameters are required to setup) |  |  |  |
| O | A.2.1) Start_superframe_offset | 9 | The 9 LSB of the start superframe number |  |
| O | A.2.2) Available_interval_length | 4 | Length of available interval in units of four frames |  |
| O | A.2.3) Unavailable_interval_length | 8 | Length of unavailable intervals in units of four frames |  |
|  | } |  |  |  |
|  | A.3) If (LBS capabilities are required to setup) |  |  |  |
| O | A.3.1) Capability for support-ing A-GPS method | 1 | 0b0: no support 0b1: support |  |
|  | } |  |  |  |
|  | A.4) Capabilities for interference mitigation support If (Interference mitigation is supported) { |  |  |  |
| O | A.4.1) DL PMI coordination capability | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |  |
| O | A.4.2) DL collaborative multi-BS MIMO capability | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |  |
| O | A.4.3) DL closed-loop multi-BS macro diversity capability | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |  |
| O | A.4.4) UL PMI combination capability | 1 | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capa-ble |  |
| O | A.4.5) Multi_BS sounding calibration capability | 1 | 0: ABS is not multi_BS sounding calibration capable 1: ABS is multi_BS sounding calibration capable | Present if needed |
|  | } |  |  |  |
| O | A.5) Capability for sounding antenna switching |  |  |  |
| O | A.5.1) Antenna configuration for sounding antenna switching | 1 | 0b0: Among DL Rx antennas. This is only possible when Antenna configuration for sounding antenna switching in AAI-REG-REQ is 0b0. 0b1: Among UL Tx antennas | Present if needed |
| O | E-MBS capabilities | 3 | If Bit#0 is set to 1, it indicates E-MBS in S-ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O | Persistent Allocation support | 1 | If Bit#0 =1,it supports | Present if needed |
| O | Group Resource Allocation support | 1 | If Bit#0 =1,it supports | Present if needed |
| O | HO trigger metric | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | Present if needed |
| O | Convergence sublayer capabilities | Variable |  | Present if needed |
| O | A) CS type | 16 or 32 | A bit set to "1" Indicates which CS Type the AMS supports Bit #0: Reserved Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: Reserved Bit #4: Reserved Bit #5: Reserved Bit #6: Reserved Bit #7: Reserved Bit #8: Reserved Bit #9: Reserved Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, none Bit #15: Multiprotocol flow, IPv4 or IPv6 Traffic Bit #16 - Bit #31 Reserved | Present if neede |
| O | B) Maximum number of classification rules | 16 | Maximum number of simultaneous admitted classification rules that a service flow supports. | Present if neede |
| O | C) ROHC support | 1 | 0: not support 1: support | Present if neede |
| O | D) PHS support | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if neede |
| O | E) Resource_Retain_Time | 16 | 0-65535: In units of 100 milliseconds Duration that the S-ABS shall retain the AMS context | Present if neede |
| O | F) IP-Service-Type IE | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if neede |
| O | Host configuration capabilities and parameters | Variable |  |  |
| O | A) IPv4-Host-Address IE | 32 | The allocated IPv4 Host Address for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | B) IPv6-Home-Network-Pre-fix IE | 64 | The allocated IPv6 Home Network Prefix for the AMS. | May be included when AMS indicates its capability of configuring host parameters |
| O | C) Additional-Host-Configurations IE | Variable | Includes additional host configuration options in DHCP Options format | May be included when AMS sends Requested-Host-Configurations IE in AAI-REG-REQ or net-work decided to configure Additional-Host-Configuration parameters |
| O | Redirection Info array [1..8] For(i=0; i<N_Redirect_Info; i++) { | Variable | Sent by S-ABS to aid cell reselection in the event the S-ABS is not able to allow the AMS to perform entry N_Redirect_Info [1…8] is the number of redirection information to be included here. | This for-loop is present if needed |
| O | A) ABSID | 48 | ABS ID of the neighbor | Present if neede |
| O | B) Preamble index | 10 | Preamble index of the neighbor | Present if neede |
| O | C) Center frequency | 32 | Center frequency of the neighbor | Present if neede |
|  | } |  |  |  |
| O | CSGID Length | 5 | Length of CSGID | Since the CSGID can be of variable length (MAX upto up to 24 bits). |
| O | Global carrier configuration change count | 3 | Indicates the current value of Global carrier configuration change count of the network | Present if neede |
| O | Multicarrier capabilities | 3 | 0b000: No MC modes 0b001: Basic MC mode 0b010: Multicarrier Aggregation 0b011: Multicarrier Switching 0b100: Both Multicarrier Aggregation and Multicarrier Switching | Present if needed |
| O | CS specification type for default service flow | 8 | Indicates the CS type selected for default service flow (refers to CS specification in the table 783) | Shall be included if multiple CS types are enabled |
| O | CLC Limit if (ABS configures the CLC Limit parameters different from the default values in Table 795) { | array | see Table688 | present if ABS configures the CLC Limit parameters different from the default values in Table 770, and may include up to 2 CLC Limit parameter sets |
|  | For(i=0; i<N-CLC-Params; i++) { |  | N-CLC-Params [1..2] is the number of CLC Limit parameter sets to be included here |  |
|  | Type I Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type I CLC class |  |
|  | Type II Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type II CLC class |  |
|  | Active Class Limit | 3 | 0: the maximum number of active CLC classes is 8 x (x>0): the maximum number of active CLC classes is x |  |
|  | Active Ratio Limit | 6 | x: the maximum CLC active ratio is x/100 |  |
|  | Active Interval Limit | 5 | x: the maximum CLC active interval is x x m AAI subframes, where m is the total number of AAI subframes in a AAI frame. |  |
|  | } |  |  |  |
|  | } |  |  |  |
| O | AMS initiated aGP Service Adaptation Capability: | 1 | 0b0: no support 0b1: support | Present if needed |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D8, page 116, line 58, delete Table 688

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 688—CLC Limit parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Type I Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type I CLC class | N/A |
| M | Type II Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type II CLC class | N/A |
| M | Active Class Limit | 3 | 0: the maximum number of active CLC classes is 8 x (x>0): the maximum number of active CLC classes is x | N/A |
| M | Active Ratio Limit | 6 | x: the maximum CLC active ratio is x/100 | N/A |
| M | Active Interval Limit | 5 | x: the maximum CLC active interval is x x m AAI subframes, where m is the total number of AAI subframes in a AAI frame. | N/A |

--------------------------------------------------  Text End  -----------------------------------------------------------------