| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup on AAI-DSC-REQ/RSP/ACK and AAI-DSD-REQ/RSP (16.2.3.47.4) |  |
| Date Submitted | 2010-9-8 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Table Reformatting and Cleanup on DSC and DSD messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Table Reformatting and Cleanup on AAI-DSC-REQ/RSP/ACK and AAI-DSD-REQ/RSP (16.2.3.47.4)

Kelvin Chou

MediaTek Inc.

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed.

In addition to table reformatting, technical changes are highlighted in yellow for the ease of peer review.

Proposed Text

Remedy 1:

In the P802.16m/D8, page 221, line 12, modify Table 740 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 740—AAI-DSC-REQ Message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | N/A |
| O | SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| O | FID | 4 | Flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| M | Uplink/Downlink Indicator | 1 | 0: uplink; 1:downlink | N/A |
| O | Service Class Name | 16 to 1024 | Null-terminated string of ASCII characters. The length of the string, including null-terminator may not exceed 128 bytes | Present when a predefined BS service configuration is used for this service flow. |
| O | Global Service Class Name field | 84 | Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| M | QoS parameter set type | 3 | Bit 0: Provisioned Set Bit 1: Admitted Set Bit 2: Active Set |  |
| O | QoS parameters | variable |  |  |
| O | A) Traffic Priority parameter | 3 | 0 to 7-Higher numbers indicate higher priority Default 0 | Present when needed |
| O | B) Maximum Sustained Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| O | C) Maximum Traffic Burst parameter | 32 | Burst size (bytes) | Present when needed |
| O | D) Minimum Reserved Traffic Rate parameter | 32 | Rate (in bits per second) | Present when needed |
| O | E) Maximum Latency parameter | 32 | Milliseconds | Present when needed |
| O | F) Paging Preference parameter | 1 | 0: No paging generation 1: Paging generation |  |
| O | G) Tolerated Jitter parameter | 32 | Milliseconds | Present when needed |
|  | If (UL scheduling type = UGS ¦ ertPS) { |  |  |  |
| O | H) Unsolicited Grant Interval parameter | 16 | Milliseconds | Present when UL scheduling type is UGS or ertPS |
|  | }else if (UL scheduling type = rtPS) { |  |  |  |
| O | I) Unsolicited Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is rtPS |
|  | }else if (UL scheduling type = rtPS) { |  |  |  |
| O | J) Primary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service |
| O | K) Primary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service |
| O | L) Secondary Grant Polling Interval parameter | 16 | Milliseconds | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
| O | M) Secondary Grant Size parameter | 16 | Number of bytes. | Present when UL scheduling type is aGP Service and if secondary parameter set is needed |
|  | } |  |  |  |
|  | If (Uplink/Downlink Indicator = 0) { |  |  |  |
| O | O) Access Class | 2 | This parameter specifies the priority assigned to a service flow. This priority is used in prioritizing access requests as described in paragraph 16.2.11.1.1. | Present for UL |
| O | P) Differentiated BR Timer | 6 | Grant reception timeout before contention-based BR is attempted again for the service flow. Value of range 1 ~ 64 frame(s) | Present for UL |
| O | Q) Predefined BR Index Array [1..15] For(i=0; i<N-Predefined-BR-indices; i++) { | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions (see table 789) N-Predefined-BR-indices is the number of predefined BR indices [1..15] | Present for UL |
| O | Q.1) Predefined BR index | 4 | Predefined BR index |  |
| O | Q.2) BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved |  |
|  | If(BR action = 0b01) { |  |  |  |
| O | Q.3) BR size | 11 | Number of bytes requested by the pre-defined BR index | Present if BR action = 0b10 |
|  | } |  |  |  |
|  | } |  |  |  |
|  | Initial Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
|  | Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 |  |
|  | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Back-off |  |
|  | } |  |  |  |
| O | R) MAC in-order delivery indicator | 1 | Indicate whether or not the order of delivery in the connection is preserved by the MAC. 0 : not preserved 1 : preserved For ARQ connections, it shall be always set to '1'. |  |
| O | S) Vendor Specific QoS Parameter | variable |  |  |
| O | SDU Inter-Arrival Interval parameter | 16 | SDU inter-arrival interval in the resolution of 0.5 ms | Present when needed |
| O | S.1) Vendor ID | 24 | Vvendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. | Present if needed |
| O | Time Base parameter | 16 | Time base in milliseconds | Present when needed |
| O | SCID | 4 | Sleep cycle ID | Present if needed |
| O | CS parameter encoding rules | 4 | 99 Reserved 100 Packet, IPv4 101 Packet, IPv6 102 Reserved 103 Reserved 104 Reserved 105 Reserved 106 Reserved 107 Reserved 108 Reserved 109 Reserved 110 Reserved 111 Reserved 112 Reserved 113 Packet IP 114 Packet, Multiprotocol flow | Present if needed |
| O | Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier | Present for AAI-DSC-REQ |
| O | If (Packet Classification Rule parameter is needed) { | variable |  | Present if needed |
| O | A) Classification Rule Priority field | 8 | 0-255 |  |
| O | B) Protocol field | 8 | protocol |  |
| O | C) IP Masked Source Address parameter | 8 (IPv4) or 32 (IPv6) | src, smask |  |
| O | D) IP Masked Destination Address parameter | 8 (IPv4) or 32 (IPv6) | dst, dmask |  |
| O | E) Protocol Source Port Range field | 32 | sportlow, sporthigh |  |
| O | F) Protocol Destination Port Range field | 32 | dportlow, dporthigh |  |
| O | G) Associated PHSI field | 8 | Index value |  |
| O | H) Packet Classification Rule Index field | 16 | Packet Classification Rule Index |  |
| O | I) Vendor-specific classification parameters | variable |  |  |
| O | I.1) Vendor ID | 24 | Vvendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| O | J) IPv6 Flow Label field | 24 | Flow Label |  |
| O | K) Classification Action Rule | 8 | Bit 0: 0 = none. 1 = Discard packet Bit 1-7: Reserved. |  |
| O | L) IP Type of Service | 8 | Bit 0-Bit 1: reserved. Shall be set to 0b00 Bit 2-Bit 7: DSCP value |  |
| O | M) PHS DSC Action field | 2 | 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |  |
| O | N) if(PHS Rule field is needed) { | variable |  | Present if needed |
| O | N.1) PHSI field | 8 | Index value |  |
| O | N.2) PHSF field | n | String of bytes suppressed |  |
| O | N.3) PHSM field | n | Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field1 = Suppress (x + 1) byte of the suppression field |  |
| O | N.4) PHSS field | 8 | Number of bytes in the suppression string |  |
| O | N.5) PHSV field | 1 | 0: Verify 1: Don't verify |  |
| O | N.6) Vendor-specific PHS parameters | variable |  | Present if needed |
|  | } |  |  |  |
|  | } |  |  |  |
| O | If ( ROHC Parameter is needed) { | variable |  | Present if needed |
| O | A) ROHC Max Context ID | 16 | Non-negative integer |  |
| O | B) Large Context IDs | 8 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) 2-255: Reserved |  |
| O | C) ROHC MRRU | 16 | 0: no segmentation Otherwise: MRRU |  |
|  | } |  |  |  |
| O | Packet Error Rate (PER) | 8 | MSB (bit 7): 0 - PER measured by the application, post-ARQ and post-HARQ process 1 - PER measured on the airlink, before the application of ARQ and HARQ Bit 6: 0 - Interpret bits 0-5 as an integer %, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = N/100 (= N%) 1-Interpret bits 0-5 as 10 times a negative exponent of 10, i.e., if bits 0-5 are the binary representation of the integer N, then the PER = 10-N/10 LSB 6 bits (bits 0-5): PER value If bit 6 =0, [0 to 63%] PER If bit 6=1, [~5x10-7 to 1x10-0] PER | Present if needed |
| O | Emergency Indication parameter | 1 | Indicates the associated flow is used for emergency purposes. | Present if needed |
| O | for (i=0; i< Num of E-MBS Zone ID to update; i++) { | 3 | Num of E-MBS Zone ID [1…8] is the nNumber of E-MBS ID to update | Present when ABS initiates AAI-DSC-REQ Present only if Num of E-MBS Zone ID to update > 0 |
| O | fullNumE-MBSIdtoUpdateArr[1..8] | Variable |  | Present only if Num of E-MBS Zone ID to update > 0 |
| M | A) Service_flow_update_indicator | 1 | Indicator whether "bitmap + new ser-vice flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Map-pings" | N/A |
| M | B) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N/A |
| O | C) New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| O | D) Physical Carrier Index | 6 | The physical carrier index that is sup-port for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
|  | If (Service_flow_update_indicator = 0b0) { |  |  |  |
| O | E) Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS.A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | F) fullServiceFlowUpdateBitmapArr[1..15] For each 1-bit in the Service Flow Update Bitmap { | Variable | For each bit in the Service Flow Update Bitmap = 1, an enrey is present to indicate the corresponding service flow is updated | Presented only if Service_flow_update_indicator is set to 0b0.and Service Flow Update Bitmap > 1. |
| O | F.1) New_E-MBS ID | 12 | New E-MBS identifier. |  |
| O | F.2) FID | 4 | Flow ID |  |
|  | } |  |  |  |
|  | }else if (Service_flow_update_indicator = 0b1) { |  |  |  |
| O | G) fullEMBSIdFidMappin-gArr[0...16] For (i=0; i<N-E-MBS-IDs; i++) { | Variable | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. N-E-MBS-IDs is the number of E-MBS IDs | Presented only if Service_flow_update_indicator is set to 0b1 |
| O | G.1) Current E-MBS ID | 12) | current E-MBS identifier |  |
| O | G.2) Current FID | 4 | Current flow ID |  |
| O | G.3) New_E-MBS ID | 12 | New E-MBS identifier |  |
| O | G.4) New FID | 4 | New flow ID |  |
|  | } |  |  |  |
|  | }//end of for |  |  |  |
| O | If ( Group Parameter Create/Change is needed) { | variable |  | Present if needed |
| O | A) Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. |  |
| O | B) Qty SFID request | M8 | The number of service flows to change | Shall only be sent by an AMS |
| M | C) Coupled FID List For(i=0; i<N-FIDs-Coupled; i++) { | 4 x N | FIDs of service flows changed with this Group Parameter Create/Change. |  |
|  | FID | 4 |  |  |
|  | } |  |  |  |
| O | D) Group FID Parameters List For(i=0; i<N-FIDs-Noncommon; i++) { | variable | N-FIDs-Noncommon is the number of non-common service flow IDs |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
|  | } |  |  |  |
|  | } |  |  |  |
| O | If ( Coupled Group Create/Change is needed) { | variable |  |  |
| O | A) Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | B) Qty Coupled SFID request | M8 | The number of DL service flow and UL service flow to change |  |
| M | C) Coupled FID List For(i=0; i<N-FIDs-Coupled; i++) { | N x 8 | FIDs of DL and UL service flow4MSB for DL FID and 4LSB for UL FID N-FIDs-Coupled is the number of FIDs |  |
|  | FID | 8 |  |  |
|  | } |  | Flow identifier |  |
| O | D) Coupled FID Parameter List For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { | variable | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs |  |
| O | D.1) FID | 4 |  |  |
| O | D.2) Non-common for Coupled Group | Variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
|  | } |  |  |  |
|  | } |  |  |  |
| O | Initial Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Maximum Backoff Window Size | 4 | Window size expressed as a power of 2 | For Uplink only |
| O | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Back-off | For Uplink only |
| O | For(i=0; i< Num of Multicast Group ID to add; i++) { | 4 | Num of Multicast Group ID is the nNumber of Multicast Group IDs to add | Present when ABS initiates AAI-DSC-REQ Present only if Num of Multi-cast Group ID to be added > 0 |
| M | A)Multicast Group ID to be added | 12 | Multicast Group ID to be added | Present only if Num of Multi-cast Group ID to be added > 0 |
|  | } |  |  |  |
| O | For(i=0; i< Num of Multicast Group ID to be deleted; i++) { | 4 | Num of Multicast Group ID is the nNumber of Multicast Group IDs to be delete | Present when ABS initiates AAI-DSC-REQ Present only if Num of Multi-cast Group ID to be deleted > 0 |
| M | A)Multicast Group ID to be deleted | 12 | Multicast Group ID to be deleted | Present only if Num of Multi-cast Group ID to be deleted > 0 |
|  | } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D8, page 229, line 22, modify Table 741 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 741—AAI-DSC-RSP Message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-REQ message | N/A |
| O | SFID | 32 | Service flow identifier | Present when an ABS initiates AAI-DSC-REQ |
| O | Confirmation Code | 1 | 0 = request was successful. 1 = request was failure |  |
| O | Group Parameter Create/Change If(received an AAI-DSA-REQ including Group Parameter Create/Change parameter ) { | variable |  | Shall be included upon receiving an AAI-DSA-REQ including Group Parameter Create/Change parameter |
| O | A) FID List For(i=0; i<N-FIDs; i++) { | 4xN | This parameter indicates FIDs of service flows created successfully with this Group Parameter Create/Change. N-FIDs is the number of successfully created service flow. | Shall be included when N-FIDs > 0 |
|  | FID | 4 | FIDs of service flows created successfully with this Group Parameter Create/Change. |  |
|  | } |  |  |  |
|  | } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D8, page 230, line 8, modify Table 742 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 742—AAI-DSC-ACK Message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI-DSC-RSP message | N/A |
| O | Confirmation Code | 1 | 0 = request was successful. 1 = request was failure |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 4:

In the P802.16m/D8, page 230, line 40, modify Table 743 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 742—AAI-DSD-REQ Message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | The change count of this transaction assigned by the sender. If new transaction is started, FID Change Count is incremented by one (modulo 16) by the sender. | N/A |
|  | If(ABS initiated AAI-DSD-REQ) { |  |  |  |
| O | FID | 4 | Flow identifier to be deleted. | Present when an ABS initiates AAI-DSD-REQ |
| O | E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | Present when ABS initiates AAI-DSD-REQ |
| O | fullEMBSIdFidArr[1..15] For (i=0; i<N-E-MBS-IDs; i++) { | Variable | Mapping of E-MBS ID and FID | Present when ABS initiates AAI-DSD-REQ |
| M | A) E-MBS ID | 12 | E-MBS identifier. |  |
| M | B) FID | 4 | Flow identifier |  |
|  | } |  |  |  |
|  | } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 5:

In the P802.16m/D8, page 231, line 24, modify Table 744 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 742—AAI-DSD-RSP Message Field Descriptions

| M/O | Attributes / Array of attributes Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | FID Change Count | 4 | FID Change Count from corresponding the AAI-DSD-REQ message | N/A |
| O | FID | 4 | Flow identifier to be deleted. | Present when an ABS initiates AAI-DSD-REQ |
| O | Confirmation Code | 1 | 0 = request was successful. 1 = request was failure |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------