| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A-MAP Enhancement in 16m |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | George Cummings Erik Colban Ronald Mao Phillip Barber Jia Lin Bin Chen Huawei Technologies, Ltd | E-mail: gcummings@huawei.com ecolban@huawei.com rmao@huawei.com pbarber@huawei.com linjia@huawei.com binchen@huawei.com |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document shows suggested changes to P802.16m Draft D8 related to enhance A-MAP privacy process without additional overhead to it. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A-MAP Enhancement in 16m

George Cummings, Erik Colban, Phillip Barber, Ronald Mao, Jia Lin, Bin Chen

Huawei Technologies Co., Ltd

# Overview

The current 16m radio resource allocation mechanism includes permutation of physical resources to logical resource units, and A-MAP resource allocation. The permutation is defined in the standard text and known by all the terminals, while A-MAP is broadcast to all AMSs, using a pre-defined MCS. Every AMS in the cell can detect and decode the A-MAPs, no matter which AMS was the intended one. Upon reception of the A-MAP, the AMS just uses a STID to mask the CRC to check whether the A-MAPs are intended for it.

Now we have protected STID, MACID and other important IDs from unintended AMSs, but expose the A-MAP to all terminals, which will cause privacy concern and potential security crisis.

About the privacy concern, for example, information in DL/UL A-MAP (such as MCS, HARQ, MIMO, resource index, and so on) can be used by a malicious user to deduce further privacy information (such as location, moving rate, habit of using cell phone, and so on), or an unintended user makes a casual observation of the data. A hacker can gain valuable personal information even without decoding the user data (ie without getting past the security layer) by knowing the users location & speed or other information, thus making A-MAP protection important for 16m. Although the STID conceals the identity of the user, an attacker may still make malicious use of the information that is transmitted in the Assignment A-MAP.  The attacker may also crack the association between the user and the STID. A UL A-MAP exposed to malicious user will make it very easy to block the intended users’ UL radio signal.

A very serious security threat is posed by a hacker, who uses the A-MAP information, which is sent in the clear, to receive all MAC frames, extract the encrypted MAC layer PDUs and send them to a background high quality calculating system to save and decrypt the data., especially when a hacker has stolen the secure key from a way other than air interface (such as breaking into core network or operator’s database). The easier it becomes to extract the MAC PDUs, the more vulnerable the encryption of the MAC PDUs becomes to being cracked.

In this contribution, we would like to provide a simple method to make the A-MAP more private and obfuscate the allocations from simple malicious inspection and piracy exposure with no additional overhead to it. The main idea is to mask the A-A-MAP IEs prior to the CRC calculation. The mask is only applied when the CRC is masked by STID or T-STID, therefore the masking can be statically configured in hardware such that no significant additional processing burden is required.

This proposal is especially valuable when a operator’s database or core network is broken into and the secure keys are stolen. Even at this critical condition, this proposal will also protect the system, since the hacker can not read the A-MAP and of course can not get the data.

# Proposed changes

## Change 1

On page 88, line 42, subclause 16.2.3.2:

-------------------------------  Start of text proposal #1 --------------------------------------------

Table 680—AAI-RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bit) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … |  |  |
| O | Temporary STID | 12 | Used for AMS identification until STID is assigned to the AMS during registration procedure | It shall be included in the AAI-RNG-RSP message in response to the AAI-RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID/DID yet. |
| O | Temporary MAPMask Key | 10 | Used to generate a 40-bit mask which the ABS shall use to mask the 40-bit A-A-MAP IE before CRC calculation when the CRC is masked with the TSTID, and used at the AMS to re-mask the 40-bit A-A-MAP IE after the CRC calculation when the CRC is masked with the TSTID. | Present only when Temporary STID is present |
| O | AMSID*/MAC address | 48 | A required parameter when the AMS confirms if the AAI-RNGRSP is a response to the AAIRNG- REQ message which the AMS sent. In the legacy network mode, the actual MAC address of the AMS is used instead. | It shall be included in the AAI-RNG-RSP message in response to the AAI-RNG-REQ message, which is not CMAC protected, when the AMS is not assigned its STID yet. |
|  | …. |  |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

## Change 2

On page 112, line 37, subclause 16.2.3.9:

-------------------------------  Start of text proposal #2 --------------------------------------------

Table 687—AAI-REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bit) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | STID | 12 | AMS identifier which the ABS assigns to the AMS in place of the temporary STID which has been transferred by AAI-RNG-RSP message. | Present if needed |
| M | MAPMask Key | 10 | Used to generate a 40-bit mask, which the ABS shall use to mask the 40-bit A-A-MAP IE before CRC calculation when the CRC is masked with the STID, and used at the AMS to re-mask the 40-bit A-A-MAP IE after the CRC calculation when the CRC is masked with the STID. |  |
| O | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode |  |
|  | …. |  |  |  |
| O | AMS initiated aGP Service Adaptation Capability: | 1 | AMS initiated aGP Service Adaptation Capability: | Present if neede |

-------------------------------  End of text proposal #2 --------------------------------------------

## Change 3

Add below text in blue to the table 691 in the page 123 line 35

-------------------------------  Start of text proposal #3 -------------------------------------------

Table 691—AAI-HO-CMD message Format

| Attributes / Array of attributes | Size (bit) | Value / Note | Conditions |
| --- | --- | --- | --- |
| Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI-HO-CMD message shall not include any T-ABS. | N/A |
| … | … | … | … |
| Ranging initiation deadline | 8 | An AMS shall send the AAI-RNG-REQ message during HO until Ranging initiation deadline |  |
| Pre-assigned MAPMask Key | 10 | Used to generate a 40-bit mask, which the T-ABS shall use to mask the 40-bit A-A-MAP IE before CRC calculation when the CRC is masked with the STID, and used at the AMS to re-mask the 40-bit A-A-MAP IE after the CRC calculation when the CRC is masked with the STID. |  |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS |  |
| …. | … | … | … |
| } |  |  |  |

-------------------------------  End of text proposal #3 --------------------------------------------

## Change 4

Add below text in blue to the section 16.3.5.2.2.4 in the page 572 line 25

-------------------------------  Start of text proposal #4 -------------------------------------------

The 40-bit A-A-MAP IE shall be masked with a 40-bit mask at the ABS before the A-A-MAP’s CRC generation, and shall be re-masked with the same mask at the AMS after the A-A-MAP’s CRC check. The mask is generated by concatenating four instances of the 10-bit MAPMask Key or Temporary MAPMask Key. The mask generated from the MapMask Key shall be used if and only if the CRC of the A-A-MAP IE is masked with the STID.  The mask generated from the Temporary MAPMask Key shall be used if and only if the CRC of the A-MAP IE is masked with the TSTID. The mask of A-A-MAP IE is applied through XOR operation.

-------------------------------  End of text proposal #4 --------------------------------------------

## Change 5

On page 599, in Table 846, line 45, indicate that the masking code could be the TSTID instead of the STID if the STID has not been assigned and transmitted to the AMS yet or the ABS cannot determine if the STID has been successfully received by the AMS.

-------------------------------  Start of text proposal #5 -------------------------------------------

0b000 ¦ 12 bit STID or TSTID

-------------------------------  End of text proposal #5 --------------------------------------------