| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Listening window Enhancement for power saving |  |
| Date Submitted | 2010-09-15 |  |
| Source(s) | Guanghui Fan, Erik Colban, Junxian Mo, Jia Lin, Bin Chen Huawei Technologies Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam Intel Corporation Giwon Park, Kiseon Ryu LG Electronics Yeongmoon Son Samsung Electronics Jaesun Cha ETRI | E-mail: fanguanghui@huawei.com ecolban@huawei.com mojunxian@huawei.com linjia@huawei.com binchen@huawei.com |
| Re: | Call for SB on “ P802.16m/D8”: |  |
| Abstract | This contribution proposes about listening window control enhancement for power saving in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Scheduling in listening window of sleep mode

Guanghui Fan, Erik Colban, Junxian Mo, Jia Lin, Bin Chen

Huawei Technologies

Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam

Intel Corporation

Giwon Park, Kiseon Ryu

LG Electronics

Yeongmoon Son

Samsung Electronics

Jaesun Cha

ETRI

Introduction

In the listening window of sleep mode, if the TIMF is set to 0, the AMS will listen in the whole listening window. If the TIMF is set to 1, AAI_TRF-IND is required in the beginning of the listening window. If the traffic indication is positive for an AMS, the AMS will wait in the rest of the listening window for any unicast traffic and the ABS shall transmit at least one DL MAC PDU to the AMS in the listening window. Else if the traffic indication is negative, the AMS can end the listening window directly for power saving.

In most cases, the AMS may only be scheduled in a few frames in the listening window, considering the MAP IE control overhead and scheduling complexity, while the service flow QoS can still be satisfied. Therefore, the AMS need not listen in the whole listening window and the power consumption can be further saved in the listening window.

If the AMS will be only scheduled at the first frames of the listening window, the ABS can inform the AMS to turn to sleep by using sleep control header (SCH) using early termination of sleep window. Therefore, to support this scenario early termination of listening window can be used.

On the other hand, if the AMS will only be scheduled in the second half of the listening window, the ABS can use the method in this proposal to inform the AMS to return to sleep at the beginning of the listening window and wake up at the necessary position. The AMS will wake up after the indicated period of no DL traffic and then listen to any DL unicast traffic in the remaining frames of the listening window. Towards this, this contribution proposes to include a one bit Traffic Location Indicator to specify if the traffic for the AMS is scheduled in the second half of the listening window. The usage of Traffic Location Indicator is as follows:

Traffic Location Indicator: 0 -> Remain awake during entire listening window

Traffic Location Indicator: 1 -> May return to sleep in the first half of listening window as the traffic is coming only in the second half of the listening window.

Text proposal for inclusion in the P802.16m/D8

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : modify the text as following in page 395 line 38]

16.2.17.2.3.1 Traffic Indication

Traffic Indication is enabled when TIMF=1 in AAI_SLP-REQ/RSP and is sent for one or a group of AMS using the AAI_TRF-IND message.

If the traffic indication is enabled for an AMS by TIMF = 1 in AAI_SLP-REQ/RSP and SLPID is assigned, the ABS shall transmit a traffic indication message during the Listening Window. Upon receiving the traffic indication message, the AMS shall check whether there is positive traffic indication (e.g., by the SLPID-Group Indication bit-map and Traffic Indication bit-map or the SLPID assigned to it). The ABS shall include Traffic Location Indicator Bitmap field in AAI-TRF-IND message if there is at least one SLPID that has a positive traffic indication. In this case, the Traffic Location Indicator field indicates whether the positive traffic indicated AMS may sleep in the first half of the listening window. The first half of the listening window is defined as the floor(N/2) first frames of the listening window where N is the length of the listening window. If the AMS receives the positive traffic indication during Listening Window, the current Sleep Cycle (i.e, which contains the Listening Window) shall be reset to the value corresponding to the NSCF (i.e., Initial Sleep Cycle, New Initial Sleep cycle, or doubles of previous Sleep Cycle) to synchronize the Listening Window of AMSs in sleep mode.

If the AMS receives a negative traffic indication, then it shall end the Listening Window and proceed with Sleep Window operation for the remainder of the Sleep Cycle, unless the AMS has UL signaling or traffic pending for transmission. If the ABS transmits a negative indication to the AMS, the ABS shall not transmit any DL data traffic to the AMS during the remaining part of the Listening Window.

If the AMS receives a positive traffic indication and no Scheduling Offset Value has been signaled to the AMS, it shall wait in the rest of Listening Window for unicast data. If the ABS sends a positive indication to a specific AMS, the ABS shall transmit at least one DL MAC PDU to the AMS during the AMS's Listening Window.

[Editor’s Note 2 : Insert the following field description at the bottom of Table 710 Parameters for AAI_TRF-IND in page 153]

Table 710 Parameters for AAI_TRF-IND

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | M | Num_of_SLPIDs | 6 | It indicates the number of SLPID included in AAI_TRF-IND message 0~63 | When FRMT ==1 |
|  | M | SLPID | 10 | Each SLPID is used to indicate the positive traffic indication for an AMS 0~1023 | When FRMT ==1 |
| O | Traffic Location Indicator Bitmap |  | L | The size L equals to the number of AMSs for which positive traffic is indicated. The max value of L is 63. The bits (starting with MSB) correspond to the AMSs with a positive traffic indication in ascending order of their SLPID. Each bit in this bitmap is the Traffic Location Indicator of the corresponding AMS; see 16.2.17.2.3.1. 0b0: AMS shall remain awake during entire listening window 0b1: AMS may return to sleep in the first half of listening window as the traffic is coming only in the second half of the listening window. | Shall be included when there is at least one SLPID that has a positive traffic indication. Omission of this field signals that the Traffic Location Indicator is 0 for all SLPIDs that have a positive traffic indication. |

………………

………………

Num_of_SLPID

The number of SLPID with positive indication

SLPID

The SLPID for AMS which has DL pending traffic

Traffic Location Indicator Bitmap

Each bit corresponds to a SLPID that has a positive traffic indication and indicates whether the corresponding data is present only in the second half of the listening window.

========================== End of Proposed Text ==============================