| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Concurrent Transmission during HO |  |
| Date Submitted | 2010-09-11 |  |
| Source(s) | Phillip Barber George Cummings Huawei Technologies, Ltd | E-mail: pbarber@huawei.com gcummings@huawei.com |
| Re: | Sponsor Ballot – P802.16m IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems - Amendment: Advanced Air Interface |  |
| Abstract | This document suggests removing the feature requiring concurrent transmission to both Serving ABS and Target ABS for both single and multicarrier during handover. |  |
| Purpose | Review and approve as a remedy to related Sponsor Ballot comment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Concurrent Transmission

Phillip Barber, George Cummings

Huawei Technologies Co., Ltd

# Overview

The process of concurrent multicarrier communication during handover defined section 16.2.6.3.3 is too complex and should be removed. The single carrier iteration of this is too complex as well and should be removed.

The process requires the AMS to communicate concurrently with both the Serving ABS and Target ABS during the re-entry process interval at the Target ABS. For both single and multicarrier this requires a degree of Scheduler coordination between the Serving ABS and Target ABS that is complex, has negative performance and latency implications, and is unnecessary to achieve robust and timely handover.

It is not as if the network will really be able to efficiently transfer data to the AMS while it is undergoing such transition, at least in a single carrier model. The dual carrier model is slightly more plausible, but requires that all terminals essentially implement full FDD operation.

The objective should be to minimize the interruption time for data transfer. Our best way to accomplish this is to minimize the amount of time that it takes to transfer the point-of-attachment relationship from the Serving ABS to Target ABS. The complex methods proposed here don't appreciably reduce the data latency, but they certainly do add tremendous complexity and network burden.

# Proposed changes

## Change 1

-------------------------------  Start of text proposal #1 --------------------------------------------

16.2.6.3.3 HO Preparation

Upon reception of the AAI-HO-CMD message, the AMS should pre-update STID and AK to be used in the T-ABS. Any mismatched system information between AMS and the T-ABS, if detected, may be provided to the AMS by the S-ABS during HO preparation. For AMS initiated HO, the ABS may detect an S-SFH mismatch by referring to the AAI-NBR-ADV change count of AMS included in AAI-HO-REQ message. In such case, the ABS should include mismatching delta SFH information in AAI-HO-CMD, or it should reject the HO.

For ABS initiated HO, the AMS may detect an SFH mismatch by referring to the S-SFH change count included in the AAI-HO-CMD message. The AMS should not select a T-ABS with mismatched SFH information. If the AMS does not have the latest SFH for any of the T-ABSs included in AAI-HO-CMD, the AMS should cancel the HO by sending AAI-HO-IND with HO event code 0b11 and and SFH mismatch indication 0b1. When the S-ABS receives the AAI-HO-IND with HO event code 0b11 and SFH mismatch indication 0b1, it may send another AAI-HO-CMD message which includes all up-to-date delta SFH information of each recommended T-ABS.

When only one T-ABS is included in the AAI-HO-CMD message, the HO preparation phase completes when S-ABS informs the AMS of its handover decision via an AAI-HO-CMD message. When multiple T-ABSs are included in the AAI-HO-CMD message, the HO preparation phase completes when the AMS informs the ABS of its T-ABS selection via an AAI-HO-IND message with HO Event code 0b00. The AAI-HO-CMD message shall include Action Time of each T-ABS for the AMS to start network reentry. The AAI-HO-CMD message shall also include a Disconnect Time Offset for each AMS to calculate disconnect time for each candidate T-ABS. Disconnect time is the time when the S-ABS expects the AMS to switch to a T-ABS. At disconnect time the S-ABS will stop sending DL data and stop providing any regular UL allocations to the AMS. When HO_Reentry_Mode is set to 0, the Disconnect Time will be (Action time - Disconnect Time Offset). For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect Time Offset).

The S-ABS may reject an AMS initiated handover by transmitting the AAI-HO-CMD message with mode set to 0b10. In this case, the S-ABS shall not include any candidate T-ABS as described in 16.2.6.3.4. If the ABS chooses to accept the handover, it shall set Mode in the AAI-HO-CMD to 0b00. If the ABS sets Mode to 0b00, it may include zero, one or more T-ABS in the AAI-HO-CMD message. The ABS may include candidate T-ABSs requested by the AMS in the AAI-HO-REQ message and/or alternate candidate ABSs not requested by the AMS.

The AAI-HO-CMD message indicates if the static and/or dynamic context and its components of the AMS are available at the T-ABS.

All on-going DSx transaction during HO shall be cancelled, and shall be re-started after HO completion. After an ABS receives the AAI-HO-REQ message from an AMS, the ABS shall not send any DSx message to the AMS until HO completion. After an ABS sends the AAI-HO-CMD message to an AMS, the ABS shall not send any DSx message to the AMS until HO completion.

-------------------------------  End of text proposal #1 --------------------------------------------

## Change 2

-------------------------------  Start of text proposal #2 -------------------------------------------

Remove dashed arrow containing the text “Data communication with S-ABS during network reentry if HO Reentry mode =1” from figure 408 (Generic HO Procedure)

-------------------------------  End of text proposal #2 --------------------------------------------

## Change 3

-------------------------------  Start of text proposal #3 -------------------------------------------

16.2.6.3.2 HO decision and initiation

Either the AMS or the S-ABS may initiate handover. If the S-ABS defines trigger conditions for initiating handover, the AMS shall initiate a handover when one or more of the trigger conditions are met.

When HO is initiated by AMS, an AAI-HO-REQ message is sent by the AMS to start the HO procedure. In case of ABS initiated HO, HO preparation is performed before HO initiation, and an AAI-HO-CMD message is sent by the ABS to initiate the HO procedure. For ABS-initiated HO, an ABS shall set Polling bit to 1 in MCEH of the AAI-HO-CMD message. Upon receiving AAI-HO-CMD message the AMS shall acknowledge it according to the procedure defined in 16.2.22, so that the ABS confirms whether the AMS receives the AAI-HO-CMD message correctly or not.

The AMS shall not perform HO to a cell with Cell Bar bit=1 in its S-SFH. The “Cell Bar” bit, when set, indicates that an AMS shall not perform network entry or re-entry to this cell. The AAI-HO-CMD message shall not include an ABS with cell bar bit =1.

-------------------------------  End of text proposal #3 --------------------------------------------

## Change 4

-------------------------------  Start of text proposal #4 -------------------------------------------

16.2.6.3.4 HO Execution

HO execution starts with AAI-HO-CMD message and ends at AMS's beginning to perform network reentry at Action Time. The S-ABS stops sending DL data and providing UL allocations to the AMS after the disconnect time derived from Action Time and Disconnect Time Offset included in the AAI-HO-CMD message or upon receiving AAI-HO-IND with HO Event Code 0b10, whichever occurs first. The S-ABS stops sending DL data and providing UL allocations to the AMS after the disconnect time, upon receiving AAI-HO-IND with HO Event Code 0b10 or after receiving HO completion confirmation from T-ABS, whichever occurs first.

If the AAI-HO-CMD message includes only one T-ABS, the AMS shall execute the HO as directed by the ABS, unless, during HO execution or network reentry, the AMS finds that the T-ABS is unreachable. The S-ABS defines conditions based on which the AMS decides if it is unable to maintain communication with the S-ABS. If the AMS decides, based on these conditions, that it cannot maintain communication with the S-ABS until the expiration of Disconnect Time, the AMS may send an AAI-HO-IND message with HO Event Code 0b10 to the S-ABS. If the AAI-HO-CMD message includes more than one T-ABS, and at least one of the T-ABSs in the AAI-HO-CMD message is reachable, the AMS shall select one of these targets and inform the S-ABS of its selection by sending an AAI-HO-IND message with HO Event Code 0b00 to the S-ABS before the expiration of Disconnect Time. The S-ABS should prioritize the HO candidates by presenting the candidate ABSs in descending order of priority in the AAI-HO-CMD message.

-------------------------------  End of text proposal #4 --------------------------------------------

## Change 5

In Table 689—AAI-HO-IND message format, page 118 lines 36-42, delete HO_reentry_mode from condition column

-------------------------------  Start of text proposal #5 -------------------------------------------

May be included when AMS's multicarrier capability = 0b10 and the target carrier is different from the serving carrier

-------------------------------  End of text proposal #5 --------------------------------------------

## Change 6

In Table 691—AAI-HO-CMD message format, page 124 lines 48-56, delete HO_reentry_mode row from table

-------------------------------  Start of text proposal #6 -------------------------------------------

Table 691—AAI-HO-CMD message format

Field; Size (bits); Value/Description; Condition. Mode; 2; 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI-HO-CMD message shall not include any T-ABS; N/A. If (Mode = 0b00) {.

-------------------------------  End of text proposal #6 --------------------------------------------