| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS scanning in femto cell |  |
| Date Submitted | 2010-9-14 |  |
| Source(s) | Guanghui Fan, Li Li, Wang Xuehuan, Ronald Mao Huawei Technologies | E-mail: fanguanghui@huawei.com lily08@huawei.com wangxuehuan@huawei.com rmao@huawei.com |
| Re: | Call for SB on “ P802.16m/D8”: |  |
| Abstract | This contribution proposes about some clarification on AMS scanning in femto cell |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS scanning in femto cell

Guanghui Fan, Li Li, Wang Xuehuan, Ronald Mao

Huawei Technologies

Introduction

This contribution gives some clarification on the content definition of the CSG white list. The CSG white list may include the absolute/relative location information, which is not properly described in current text.

To clarify the usage of the proposed location information content of the CSG white list, this contribution also gives an example of how to use the location information for an AMS initiating the scanning procedure.

Text proposal for inclusion in the P802.16m/D8

========================== Start of Proposed Text ==============================

[Remedy 1: modify the text in line 33 in page 845]

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

For neighbor Femto ABSs, an AMS performs the scanning procedure as per 16.2.6.1.2 with exceptions described in this subsection. An AMS may scan femto ABSs according to the FAs included in the broadcast AAI-NBR-ADV message. AMS may scan femto ABSs that are not included in AAI-NBR-ADV based on SA-preamble partitioning information (see 16.4.4). In addition, an AMS may scan allowed femto ABSs based on the CSG White List .,which may include the absolute or relative location information of the CSG Femto ABS, such as the GPS information or the overlay macro ABS BSID, respectively. Based on location information and/or speed, AMS may initiate the scanning procedure (See 16.2.6.1.2). The location information may include absolute/relative location information of CSG Femto ABS, such as GPS info, overlay Macro ABS BSID. For example, AMS may use the absolute or relative location information of the CSG Femto ABS or overlay Macro ABSs to initiate scanning when the distance between the AMS and the CSG Femto ABS is less than a pre-configured threshold or the AMS detects the overlay macro ABS. Details of the threshold configuration are vendor specific and outside the scope of this specification. The AMS may request additional scanning opportunity by sending AAI-SCN-REQ including the detected SA-preamble index and FA information. Upon reception of the AAI-SCN-REQ, the ABS shall respond with an AAI-SCN-RSP which may include neighbor accessible Femto ABS list based on the SA-preamble index.

========================== End of Proposed Text ==============================