| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_DSX-REQ/RSP/ACK ASN.1 code update |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_DSX-REQ/RSP/ACK attribute issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AAI_DSX-REQ/RSP/ACK ASN.1 code update

- Joey Chou

- Intel

- Introduction

This contribution proposes updates to the AAI-DSX ASN.1 code in Annex R.2 in order to align with AAI-DSX message table in the D8 draft.

- Proposed text

II.1 Proposed text 1 - AAI_DSA-REQ

------------------------------------------------- Start of proposed text I--------------------------------------------------

[Editor note: at page 940, line 55, change as the following]

QosPrameter ::= SEQUENCE {

trafficPriority                INTEGER (0..7) OPTIONAL,

maxSustainedRate               DataRate OPTIONAL, -- bps

maxTrafficBurst                DataRate OPTIONAL, -- bytes

minReservedTrafficRate         DataRate OPTIONAL, -- bps

maxLaytency                    DataRate OPTIONAL, -- ms

sduLengthIndicator             SduLengthIndicator OPTIONAL,

pagingPreference               PagingPreference OPTIONAL,

reqTransmissionPolicy          ReqTransmissionPolicy OPTIONAL,

ulGrantScheduling              UlGrantScheduling OPTIONAL,

toleratedJitter                DataRate OPTIONAL, -- ms

unsolicitedGrantInterval       Interval OPTIONAL, -- ms

unsolicitedPollingInterval     Interval OPTIONAL, -- ms

priGrantPollingInterval        Interval OPTIONAL, -- ms

secGrantPollingInterval        Interval OPTIONAL, -- ms

priGrantSize                   Interval GrantSize OPTIONAL, -- bytes

secGrantSize                   Interval GrantSize OPTIONAL, -- bytes

adaptationMethod               ENUMERATED {

implicit absInitiated,

explicitSustained,

explicitOneTime amsInitiated}OPTIONAL,

accessClass                    INTEGER (0..3) OPTIONAL,

differentiatedBrTimer          INTEGER (1..634) OPTIONAL, -- frames

predefinedBrIndexList          SEQUENCE (SIZE(0..15)) OF

PredefinedBrIndex OPTIONAL,

macInOrderDelivery             ENUMERATED {

notPreserved,

preserved} OPTIONAL,

vendorSpecificQoSParameter     SEQUENCE {

vendorId                     OCTET STRING (SIZE(3))} OPTIONAL

}

GrantSize ::= INTEGER (0..65535)

DataDeliveryServices ::=        ENUMERATED INTEGER {

ugs(0),

rtVrs(1),

nrtVrs(2),

be(3),

ertVrs(4)} (0..255)

[Editor note: at page 941, line 47, delete SduLengthIndicator]

SduLengthIndicator ::=          ENUMERATED {

variableLength,

fixedLength}

ArqAttribute ::= SEQUENCE {

arqEnable                      ENUMERATED {

arqNotRequested,

arqRequested} OPTIONAL,

arqWindowSize                  INTEGER (1..65535) OPTIONAL, --

arqBlockLifeTime               INTEGER (0..65535) OPTIONAL, -- 100us

arqSyncLossTimeout             INTEGER (0..65535) OPTIONAL, -- 100us

arqPurgeTimeout                INTEGER (0..65535) OPTIONAL, -- 100us

-- sub-block size = 2^(P+3), 0<=P<=7

arqSubBlockSize                INTEGER (0..65535 7) OPTIONAL, -- Bytes

arqErrorDetectionTimeout       INTEGER (0..65535) OPTIONAL, -- 100s

arqFeedbackPollRetryTimeout    INTEGER (0..65535) OPTIONAL  -- 100s

}

CsSpecification ::=             INTEGER {

packetIpv4 (1),

packetIpv6 (2),

packetIpv4OrIpv6 (14),

multiProtocol (15)} (0..15 255)

PhsRule ::= SEQUENCE {

phsiField                      INTEGER (0..255) OPTIONAL,

phsfField                      OCTET STRING (SIZE(0..2565))OPTIONAL,

-- phsmField[x] = 0 don't surpress x's byte,

-- phsmField[x] = 1 subpress x's byte

phsmField                      BIT STRING (SIZE (0..2565)) OPTIONAL,

phssField                      INTEGER (0..255) OPTIONAL,

phsvField                      ENUMERATED {

verify,

doNotVerify} OPTIONAL,

vendorSpecificPHSParameters    OCTET STRING (SIZE(0..999)) OPTIONAL

}

RohcAttributes ::= SEQUENCE {

maxContextId                   INTEGER (0..65535) OPTIONAL,

largeContextId                 ENUMERATED INTEGER{

smallCid (0),

largeCid (1)} (0..255) OPTIONAL,

-- 0: no segmentation,

-- 1..65535 Maximum reconstructed reception unit in Byte

rohcMrru                       ENUMERATED INTEGER(0..65535) {

noSegmentation,

mrru} OPTIONAL}

AAI-DSA-REQ ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

sfid                           SFID OPTIONAL,

fid                            FID OPTIONAL,

directionIndicator             DirIndicator,

serviceClassName               ServiceClassName OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosPrarmeterSet                QosPrarmeterSetType OPTIONAL,

qoSParameters                  QosPrameter OPTIONAL,

dataDeliveryServices           DataDeliveryServices OPTIONAL,

sduInterArrival                Interval OPTIONAL, -- 0.5ms

timeBase                       Interval OPTIONAL, -- ms

sduSize                        INTEGER (0..255) DEFAULT 49,

targetSaid                     INTEGER (0..65535 255) OPTIONAL,

sleepCycleId                   INTEGER (0..15) OPTIONAL,

nonArqReorderTimeout           INTEGER (01..312) OPTIONAL, -- 5ms

arqAttributes                  ArqAttribute OPTIONAL,

csSpecificationType            CsSpecification OPTIONAL,

classificationRules            ClassificationRule OPTIONAL,

rohcAttributes                 RohcAttributes OPTIONAL,

packetErrorRate                PacketErrorRate OPTIONAL,

emergencyIndication            BOOLEAN OPTIONAL,

macHeaderType                  ENUMERATED {

agmh,

spmh} OPTIONAL,

embsService                    EMBSService OPTIONAL,

numberOfEMBSZoneID             INTEGER (0..7) OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

embsidFIDMappingList         SEQUENCE (SIZE(1..15)) OF SEQUENCE {

embsid                     EMBSID,

fid                        FID}

},

unicastAvailIntervalBitmap     CHOICE {

nmsi2                        BIT STRING (SIZE(4)),

nmsi4                        BIT STRING (SIZE(4)),

nmsi8                        BIT STRING (SIZE(8)),

nmsi16                       BIT STRING (SIZE(16))} OPTIONAL,

initialBackoffWindowSize       INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize           INTEGER (0..15) OPTIONAL,

backoffScalingFactor           INTEGER (0..15) OPTIONAL,

groupParameterCreateChange     SEQUENCE {

-- ??? this part of the table is obscure

},

...

}

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2 - AAI_DSA-RSP

------------------------------------------------- Start of proposed text II--------------------------------------------------

[Editor note: at page 945, line 46, change as the following]

AAI-DSA-RSP ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

confirmationCode               ConfirmationCode,

fid                            FID OPTIONAL,

groupParameterCreateChange     SEQUENCE {

fidList                      SEQUENCE (SIZE(1..8)) OF SEQUENCE {

fid                        INTEGER (0..15) OPTIONAL}

} OPTIONAL,

embsService                    EMBSService OPTIONAL,

numberOfEMBSZoneID             INTEGER (0..7) OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

embsidFIDMappingArray        SEQUENCE (SIZE(1..15)) OF SEQUENCE {

embsid                     EMBSID,

fid                        FID}

},

unicastAvailIntervalBitmap     CHOICE {

nmsi2                        BIT STRING (SIZE(4)),

nmsi4                        BIT STRING (SIZE(4)),

nmsi8                        BIT STRING (SIZE(8)),

nmsi16                       BIT STRING (SIZE(16))} OPTIONAL,

...

}

------------------------------------------------- End of proposed text II--------------------------------------------------

II.3 Proposed text 3 - AAI_DSC-REQ

------------------------------------------------- Start of proposed text III--------------------------------------------------

[Editor note: at page 946, line 15, change as the following]

AAI-DSC-REQ ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

sfid                           SFID OPTIONAL,

fid                            FID OPTIONAL,

directionIndicator             DirIndicator,

serviceClassName               ServiceClassName OPTIONAL,

globalServiceClass             GlobalServiceClassName OPTIONAL,

qosPrarmeterSet                QosPrarmeterSetType OPTIONAL,

qoSParameters                  QosPrameter OPTIONAL,

dataDeliveryServices           DataDeliveryServices OPTIONAL,

sduInterArrival                Interval OPTIONAL, -- 0.5ms

timeBase                       Interval OPTIONAL, -- ms

sduSize                        INTEGER (0..255) DEFAULT 49,

targetSaid                     INTEGER (0..65535) OPTIONAL,

sleepCycleId                   INTEGER (0..15) OPTIONAL,

classifierDSCAction            ClassifierDSCAction OPTIONAL,

classificationRules            ClassificationRule OPTIONAL,

rohcAttributes                 RohcAttributes OPTIONAL,

packetErrorRate                PacketErrorRate OPTIONAL,

emergencyIndication            BOOLEAN OPTIONAL,

macHeaderType                  ENUMERATED {

agmh,

spmh} OPTIONAL,

embsService                    EMBSService OPTIONAL,

numberOfEMBSZoneID             INTEGER (0..7) OPTIONAL,

fullEMBSIDArray                SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

newEmbsZoneID                EMBSZoneID,

carrierIndex                 PhyCarrierIndex,

serviceFlowUpdateType        CHOICE {

bitmapAndNew               SEQUENCE {

serviceFlowUpdateBitmap  BIT STRING (SIZE(16)),

embsidFIDMappingArray    SEQUENCE (SIZE(1..16)) OF SEQUENCE {

newEMBSID              EMBSID,

newFID                 FID},

currentAndNew              SEQUENCE (SIZE(1..16)) OF SEQUENCE {

currentEMBSID            EMBSID,

currentFID               FID,

newEMBSID                EMBSID,

newFID                   FID}

}

}

},

-- ???? this part of the table is obscure

groupParameterCreateChange     SEQUENCE {  },

initialBackoffWindowSize       INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize           INTEGER (0..15) OPTIONAL,

backoffScalingFactor           INTEGER (0..15) OPTIONAL,

...

}

------------------------------------------------- End of proposed text III--------------------------------------------------

II.4 Proposed text 4 - AAI_DSC-RSP

------------------------------------------------- Start of proposed text IV--------------------------------------------------

[Editor note: at page 947, line 19, change as the following]

AAI-DSC-RSP ::= SEQUENCE {

fidChangeCount                 FidChangeCount,

sfid                           SFID OPTIONAL,

confirmationCode               ConfirmationCode,

groupParameterCreateChange     SEQUENCE {

fidList                      SEQUENCE (SIZE(1..8)) OF SEQUENCE {

fid                        INTEGER (0..15) OPTIONAL}

} OPTIONAL,

...

}

------------------------------------------------- End of proposed text IV--------------------------------------------------