| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of AAI-RNG-RSP/AAI-HO-CMD message (16.2.3.2) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Youngsoo Yuk, Jinsam Kwak LG Electronics | Jin1.lee@lge.com |
| Re: | Sponsor ballot recirc #2 on P802.16m/D8 |  |
| Abstract | This contribution is to clarify ‘re-entry/HO process optimization’ field. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of AAI-RNG-RSP/AAI-HO-CMD message

Jin Lee, Kiseon Ryu, Youngsoo Yuk, Jinsam Kwak

LG Electronics

- Introduction

Reentry process optimization and HO process optimization attribute need to be clarified.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1] Modify texts in page 79, line 11 (table 681) as suggested below:

| Attributes / Array of attributes | Size (bit) | Value/Note | Conditions |
| --- | --- | --- | --- |
| Reentry Process Optimization | 4 5 | For each bit, 0b0 indicates the associated reentry MAC control messages are required, while 0b1 indicates the associated reentry MAC control messages may be omitted. Bit #0: Omit AAI-SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI-REG-REQ/ RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: Omit higher layer protocol triggering for IP address refresh during reentry processing. Bit #3 4: If Bit #3 4 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines). Identifies reentry process MAC control messages that may be omitted during the current HO attempt due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with T-ABS until completing receiving all network reentry, MAC control message responses as indicated in HO process optimization | Shall be included when the AMS is attempting to perform network reentry, HO or zone switch and the T-ABS wishes to identify reentry process MAC control messages that may be omitted during the current HO attempt |

[Remedy2] Modify texts in page 110, line 36 (table 691) as suggested below:

| Attributes / Array of attributes | Size (bit) | Value/Description | Conditions |
| --- | --- | --- | --- |
| HO process optimization | 4 5 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI-SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI-REG-REQ/ RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: Omit higher layer protocol triggering for IP address refresh during reentry processing. Bit #3 4: If Bit #3 4 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | When Bit #2 is set to 0 and IP address refresh is needed, the AAI-REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |

-------------------------------  Text End  ---------------------------------------------------