| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of the SCH |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak LG Electronics | gwpark@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clean-up of the SCH in 16m/D8. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D9 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of the SCH

Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak

LG Electronics

Introduction

D8 describes AMS or ABS switches the SCID via Sleep Control Header (SCH), DSx MAC control message or AAI_SLP-REQ/RSP message when the AMS is in sleep mode. However, signaling header does not support integrity protection. Moreover, because the SCID switching impacts sleep cycle parameters in sleep cycle setting, if the SCID switching can be done by Sleep Control Header (SCH), the synchronization of the sleep cycle configuration may be broken between AMS and ABS. Thus, the SCID switching shall be done by MAC control message (i.e., DSx MAC control message or AAI_SLP_REQ/RSP message) when the AMS is in sleep mode.

Text proposal for inclusion in the P802.16m/D9

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify the parameters in “16.2.2.1.3.4 SCH” in page 62]

16.2.2.1.3.4 Sleep Control Header (SCH)

Table 660—Sleep Control header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Header Format () { |  |  |
| FID | 4 | Flow Identifier. Set to 0010. |
| Type | 5 | MAC Signaling Header Type = 0b00011 |
| SCH sub-type | 3 | 0b000 =Listening Window control 0b001= Resume Sleep Cycle Indication 0b010=Sleep cycle configuration switch 0b0110b010=Multi-Carrier Listening Window control 0b1000b011~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgment) to the request |
| Iif (SCH sub-type == Listening Window Control) { | - | - |
| Listening Window End or Extension | 1 | 0=Listening Window End Indication 1=Listening Window Extension Indication |
| if (Listening Window End or Extension ==1) { | - | - |
| Last frame of Extended Listening Window | 7 | LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } | - | - |
| } else if (SCH sub-type == Resume Sleep Cycle Indication) { | - | - |
| Scheduled Sleep Cycle Interruption included | 1 | 0=no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1=scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { | - | - |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 7 | Number of frames in the future from the frame containing this SCH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e., range is 1 to 128). |
| } | - | - |
| } | - | - |
| else if (SCH sub-type == Sleep Cycle configuration switch) { | - | - |
| SCID | 4 | SCID corresponds to the new sleep cycle setting to be switched |
| Start Frame Offset for new sleep cycle configuration | 3 | Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } | - | - |
| else if (SCH sub-type == Multi-Carrier Listening Window control) { |  |  |
| Num_Target Carrier | 4 |  |
| Ffor(i=0; i< Num_Target Carrier; i++) { |  |  |
| Target Carrier Index | 46 | Indicates that DL data transmission on the secondary carrier of physical carrier index ends. |
| } |  |  |
| } |  |  |
| Padding | variable | For byte alignment |
| } | - | - |

[Editor’s Note: Modify the text in page 428, line 37, in 16.2.17.2.4 Sleep Mode parameter update]

The Sleep Cycle setting update may be accomplished by the AMS sending an AAI-SLP-REQ message or SCH with request to re-activate a previously defined sleep cycle or change the sleep parameters of existing SCID. Changing the sleep parameters of existing SCID overrides the old parameters. On receipt of an AAI-SLP-REQ requesting Sleep Cycle setting change, the ABS shall respond with an AAI-SLP-RSP message to confirm the change along with the start frame number, or to propose alternate settings, or to deny the requested change. At the frame specified by Start_Frame_Number, the newly updated sleep cycle settings shall be applied. Alternatively, the ABS may initiate a Sleep Cycle parameter change by sending an unsolicited AAI-SLP-RSP message or SCH to the AMS.

[Editor’s Note: Modify the text in page 429, line 50, in 16.2.17.2.4 Sleep Mode parameter update]

In the event that an ABS-initiated request (i.e., Unsolicited AAI-SLP-RSP, SCH) and an AMS-initiated request for Sleep Cycle setting change or switch (i.e., AAI-SLP-REQ, SCH, Service Specific Scheduling Control Header) are being handled concurrently, the ABS-initiated request shall take precedence over the AMS-initiated Request.

========================== End of Proposed Text ==============================