| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Table Reformatting and Cleanup of Connection Management and System Information Messages |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_DSX-REQ/RSP/ACK attribute issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Table Reformatting and Cleanup of Connection Management and System Information Messages

- Joey Chou

- Intel

- Introduction

This contribution is intended to reformat connection management and system information messages.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.47.1 AAI_DSA-REQ

[Editor note: The following table only includes the attributes that have been changed.]

Table 737—AAI_DSA-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | S) Predefined BR Index Array [1..15] For (i=0; I < 15 ; i++) { | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O | S.1) Predefined BR index | 4 | Predefined BR index | Present when an ABS initiates AAI_DSA-REQ |
| O | S.2) BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| O | S.3) BR size | 11 | Number of bytes requested by the predefined BR index | Present if BR action = 0b10 |
|  | } |  |  |  |
| O | fullNumE-MBSIdArr[1..8] For (i=0; I < 8 ; i++) { | Variable |  | Present if needed |
| M | A) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N/A |
| O | B) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| M | C) fullEMBSIdFidMappingArr[1..15] For (i=0; I < 15 ; i++) { | variable | Mapping of E-MBS ID and FID. |  |
| M | C.1) E-MBS ID | 12 | E-MBS identifier | N.A. |
| M | C.2) FID | 4 | Flow ID |  |
|  | } |  |  |  |
|  | } |  |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.47.2 AAI_DSA-RSP

[Editor note: The following table only includes the attributes that have been changed.]

Table 738—AAI_DSA-RSP

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | fullNumE-MBSIdArr[1..8] For (i=0; I < 8 ; i++) { | Variable |  | Present if needed |
| M | A) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N/A |
| O | B) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| M | DC) fullEMBSIdFidMappingArr[1..15] For (i=0; I < 15 ; i++) { | variable | Mapping of E-MBS ID and FID. |  |
| M | DC.1) E-MBS ID | 12 | E-MBS identifier | N.A. |
| M | DC.2) FID | 4 | Flow ID |  |
|  | } |  |  |  |
|  | } |  |  |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

II.3 Proposed text 3

------------------------------------------------- Start of proposed text III--------------------------------------------------

16.2.3.47.4 AAI_DSC-REQ

[Editor note: The following table only includes the attributes that have been changed.]

Table 740— AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Q) Predefined BR Index Array [1..15] For (i=0; I < 15 ; i++) { | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions | Present for UL |
| O | Q.1) Predefined BR index | 4 | Predefined BR index | Present when an ABS initiates AAI_DSA-REQ |
| O | Q.2) BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| O | Q.3) BR size | 11 | Number of bytes requested by the predefined BR index | Present if BR action = 0b10 |
|  | } |  |  |  |
| O | fullNumE-MBSIdArr[1..8] For (i=0; I < 8 ; i++) { | Variable |  | Present if needed |
| O | A) E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid | N/A |
| O | B) Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| O | C) New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| O | D) Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
| O | E) Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. | A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |
| O | F) fullEMBSIdFidMappingArr[1..15] For (i=0; I < 15 ; i++) { | variable | Mapping of E-MBS ID and FID. |  |
| O | F.1) New_E-MBS ID | 12 | E-MBS identifier | N.A. |
| O | F.2) FID | 4 | Flow ID |  |
|  | } |  |  |  |
|  | } |  |  |  |

------------------------------------------------ End of proposed text III --------------------------------------------------

II.4 Proposed text 4

------------------------------------------------- Start of proposed text IV--------------------------------------------------

16.2.3.47.7 AAI_DSD-REQ

[Editor note: The following table only includes the attributes that have been changed.]

Table 743—AAI_DSD-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | fullEMBSIdFidMappingArr[1..15] For (i=0; I < 15 ; i++) { | variable | Mapping of E-MBS ID and FID. |  |
| O | A) New_E-MBS ID | 12 | E-MBS identifier | N.A. |
| O | B) FID | 4 | Flow ID |  |
|  | } |  |  |  |

------------------------------------------------- End of proposed text IV --------------------------------------------------

II.5 Proposed text 5

------------------------------------------------- Start of proposed text V--------------------------------------------------

16.2.3.50 AAI_SII-ADV

[Editor note: The following table only includes the attributes that have been changed.]

Table 747—AAI_SII-ADV

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | nspArray [1..16] For (i=0; I < 16 ; i++) { | Variable | List of one or more Network Service Provider 24 bit identifier |  |
| M | A) nspIdentifier | 24 | NSP identifier which ABS supports |  |
| M | verboseNspNameArray [1..16] For (i=0; I < 16 ; i++) { | Variable | List of the verbose names of the NSPs. The value of Verbose NSP Name List is a compound list of verbose NSP name lengths and verbose NSP names. The order of the Verbose NSP Name Lengths and Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. |  |
| M | A) verboseNspNames | Variable (1 ... 128 x 8) | Verbose NSP name string |  |
|  | } |  |  |  |
|  | } |  |  |  |

------------------------------------------------- End of proposed text V --------------------------------------------------

II.6 Proposed text 6

------------------------------------------------ Start of proposed text VI--------------------------------------------------

16.2.3.31 AAI_SCD

[Editor note: The following table only includes the attributes that have been changed.]

Table 711—AAI_SCD

| M/O |  | Attributes/Array of attributes |  | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | M | uLFeedbackInfo [1..8] | physical PrimaryCarrierIndex | 6 | the physical carrier index for primary carrier | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers and all the AMSs do not support a single identical primary carrier |
|  | M |  | startDLRUIndex | 7 | the start DLRUs index for feedback channel | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers |
|  | M |  | dLRUNum | 4 | the number of DLRUs for feedback channel per UL AAI sub-frame (Refer to 16.3.8.3.3.2) | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers |
|  | M |  | hARQChannelNum | 2 | the number of HARQ feedback channel per HARQ feedback region | Describes LHFB in 16.3.8.3.3.2. Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. |
| O |  | For (i=0; I < 8 ; i++) { |  |  |  |  |
| M |  | physical PrimaryCarrierIndex |  | 6 | the physical carrier index for primary carrier | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers and all the AMSs do not support a single identical primary carrier |
| M |  | startDLRUIndex |  | 7 | the start DLRUs index for feedback channel | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers |
| M |  | dLRUNum |  | 4 | the number of DLRUs for feedback channel per UL AAI sub-frame (Refer to 16.3.8.3.3.2) | Present when multicarrier is configured in multicarrier aggregation with DL-only secondary partially configured carriers |
| M |  | hARQChannelNum |  | 2 | the number of HARQ feedback channel per HARQ feedback region | Describes LHFB in 16.3.8.3.3.2. Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. |
|  |  | } |  |  |  |  |

------------------------------------------------- End of proposed text VI --------------------------------------------------