| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | On S-SFH SP1 IE Femto Indicator (16.3.5.5.1.2, Table 836) |  |
| Date Submitted | 2010-09-07 |  |
| Source(s) | Ying Li, Sudhir Ramakrishna, Kaushik Josiam , Zhouyue Pi, Anshuman Nigam, Jung Je Son Samsung Electronics | Phone: +1-972-761-7903 E-mail: yli2@sta.samung.com anshun@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m D8 change requests. |  |
| Abstract | This contribution is to propose changes in Table 840 to save the overhead of S-SFH SP1. |  |
| Purpose | To be discussed and adopted by WG SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# On S-SFH SP1 IE Femto Indicator (16.3.5.5.1.2, Table 836)

Ying Li, Sudhir Ramakrishna, Kaushik Josiam,

Zhouyue Pi, Anshuman Nigam, Jung Je Son

Samsung Electronics

# Introduction

- In the current draft D8, it uses a bit ‘femto indicator’ in S-SFH SP1 IE is changed to “ranging configuration  type” , for the purpose of indicating different ranging configuration of femto and other types of the cells but the change has not been reflected to other portions of the table 836.

# Proposed Text

Please revise Table 836 in D8 as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.3.5.5.1.2 S-SFH IE

[Modify texts as suggested below: Page 585, Line 47]

Table 837—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| …… |  |  |
| ABS EIRP | 6 | Unsigned integer from 1 to 64 in units of 1 dBm, where 000000=1 dBm and 111111=64 dBm. |
| …… |  |  |
| Ranging configuration type | 1 | <del>Indicates whether the ABS is a Femto ABS. 0b0: ABS not a Femto ABS. 0b1: ABS is a femto ABS.</del> <ins>Indicates the ranging configuration type. 0b0: ranging configuration type being the one for Femtocell 0b1: ranging configuration type being the one for Macrocell</ins> |
| …… |  |  |
| if(<del>Femtocell</del> <ins>Ranging configuration type ==0b0</ins>) { |  | for AAI <del>Femtocell </del><ins>ranging configuration type being the one for Femtocell</ins> |
| Allocation periodicity of the S-RCH | 2 | Indicates the periodicity of the S-RCH allocation according to the Table 926. |
| …… |  |  |
| } else { |  |  |
| Allocation periodicity of the NS-RCH | 2 | Indicates the periodicity of the NS-RCH allocation according to the Table 923. |
| …… |  |  |
| NS-RCH formats | 1 | Indicates the NS-RCH formats number of Table 916 |
| } |  |  |
| ……. |  |  |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------