| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the CLC-RSP Message Specification (16.2.3.18) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Lei Wang InterDigital Communications | leiw@billeigean.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D8) |  |
| Abstract | Clarification on the tables for CLC-RSP MAC message specification (16.2.3.18). |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the CLC-RSP Message Specification (16.2.3.18)

Lei Wang

InterDigital Communications

Introduction

Current MAC control message table in 16m/D8 is not clear from a readability point of view. Therefore alternative table format is suggested in this contribution, and the text changes are proposed for the CLC-RSP MAC control message.

Note that the version of the contribution C802.16m-10/1145r1 attempts to resolve the following comments:

- Comment B167 / contribution C802.16m-10/1145  suggested to accept contribution 1145r1

- comment B100  agree, and resolved by comment B167 with contribution 1145r1

- comment B130  agree, and resolved by comment B167 with contribution 1145r1

- comment B131  disagree, not specific text

- comment B207 / contribution C802.16m-10/1253  accepted-modified (with a table  number change), and resolved by comment B167 with contribution 1145r1

- comment B10039  suggest to discuss it in the TGm-B meeting, as it is about CLC-REQ message, not CLC-RSP.

Proposed Changes

The following are the proposed changes in the 802.16m/D8.

Suggested change #1: line 20, page 148

Delete the entire Table 698; and re-number the tables after Table 698 accordingly

Suggested change #2: throughout the 16m draft standard

Replace the references to “Table 698” by “Table 697”;

Suggested change 3:

page 148, line 3, replace Table 697 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 697— AAI_CLC-RSP message

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI_CLC-RSP Message () { |  |  |  |
| Confirmed Action | 8 | Each bit of the parameter set to 0 or 1 indicates whether the corresponding CLC class is inactive or active; |  |
| For (i=0; i<8; i++) { |  |  |  |
| If ((Confirmed Action [i] == 1) and “CLC type is II or III” and “ABS configures different starting time” ) { |  |  |  |
| CLC ID | 3 | 0~7 |  |
| Start Superframe Number | 3 | the 3 LSB of the superframe number of CLC start time |  |
| Start Frame Index | 2 | the frame index of CLC start time |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Suggested change 4:

page 148, Replace line 10 to 16 by the following text:

Parameters shall be as follows:

In addition to presence in the AAI-CLC-RSP message, the CLC Response Parameters are listed in Table 697 698 and may be present in AAI-SBC-RSP, AAI-CLC-RSP, and AAI-RNG-RSP.

Suggested change 5:

page 148, change line 51 to 56 as follows:

CLC Start Time parameters, CLC ID, Start Superframe Number, and Start Frame Index, are This field is included for each Type II or III CLC class that ABS configures its starting time differently from what is recommended by AMS, and its parameters is listed in Table 698. If the number of configured CLC classes is n, the size of this field is calculated as n x 8.

Suggested change 6: page 103, line 22

change Table 696 to Table 697

Suggested change #7: page 117, line 10

[comment: the sentence of “x (x>0): the maximum number of active CLC classes is x “ should start as a new line]

suggested remedy: move “x (x>0): the maximum number of active CLC classes is x” to a new line

Suggested change #8: page 146, line 1

[comment: “CLC Request”, “CLC Report”, and “CLC Start Time parameters” fields do not exist]

suggested remedy: remove texts between line 1, page 146 and line 17, page 146

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”