| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup on the Assignment A-MAP (16.3.5) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Email: lei.huang@sg.panasonic.com |
| Re: | IEEE 802.16m Sponsor Ballot: P802.16m/D8 |  |
| Abstract | The contribution proposes clean-up modifications on the assignment A-MAP |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up on the Assignment A-MAP (16.3.5)

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

According to [1], there are some restrictions on the structure of an assignment A-MAP. For example,

- If two assignment A-MAP groups using two channel coding rates are present in an A-MAP region, assignment A-MAP group using lower channel coding rate is allocated first, followed by assignment A-MAP group using higher channel coding rate.

- If a broadcast assignment A-MAP IE exists in a DL AAI subframe, it shall be present at the beginning of the assignment A-MAP group using the lower code rate in the primary frequency partition.

- All the multicast assignment A-MAP IEs present in any assignment A-MAP group shall occupy a contiguous set of MLRUs starting from the beginning of the assignment A-MAP group. If the broadcast assignment A-MAP IE is present in the assignment A-MAP group, the multicast assignment A-MAP IEs are located right after the broadcast assignment A-MAP IE.

For easy of understanding, it would be better to introduce a figure to illustrate how the assignment A-MAP is organized. In this contribution, we suggest to add a figure into the draft standard to illustrate the structure of an assignment A-MAP for the non-FFR configuration.

- Reference

- [1]	IEEE P802.16m/D8, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, August 2010

- Proposed Modifications

- -------------------------------------------------------------------------------------------------------------------

- To insert the following text and figure after line 53, page 572

- As an example, the structure of an assignment A-MAP for non-FFR configuration is illustrated in Figure xxx.

- Figure xxx - Structure of an assignment A-MAP for non-FFR configuration

- -------------------------------------------------------------------------------------------------------------------